3GPP TSG-SA WG2 Meeting #170	S2-2507474

Stor-Göteborg, Sweden, Aug 25 – 29, 2025 		                              (revision of S2-2506470)

Source:	ZTE, vivo, CMCC

Title:	IMS Enhancement Solution for GEO NB-IoT NTN Access with Rate and Call Setup Time Constraints

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a solution to optimize the IMS call setup procedure for voice over GEO satellite NTN, focusing on minimizing signaling and simplifying service interactions to reduce call setup time under low bitrate conditions.

1. Discussion

To support IMS voice communication over GEO satellite access connected to the EPC, the system shall enable IMS service capabilities in such scenarios and support lower codec bitrates to accommodate the low transmission rates of GEO NB-IoT NTN access. Due to the limited uplink bitrate (1–3 kbps) and long propagation delay introduced by GEO satellite, the conventional IMS call setup procedure may take up to 88 seconds under current conditions. Therefore, optimization mechanisms shall be introduced—aligned with SA1 requirements—to reduce IMS call setup time to within 30 seconds, ensuring service availability under low bitrate and high latency conditions. ‌

Given the limited bitrate and high latency of GEO satellite link, it is essential to implement the following optimizations:

- Minimize SIP signalling

Any SIP signaling exchanged between the UE and the P-CSCF should be minimized to the greatest extent possible.

For uplink message, UE should include only the essential SIP headers, with all supplementary headers added by the P-CSCF.

For downlink message, P-CSCF should transmit only the headers necessary for the UE, omitting any non-essential information.

- Simplify IMS service procedure

Pre-allocated bearer should be established for voice in advance, and the P-CSCF should be notified that the corresponding resources have already been reserved, avoiding the need for pre-condition.

For IMS registration and call setup processes, only the essential signalling messages should be exchanged to reduce overhead.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

6.X	Solution #X: IMS Enhancement Solution for GEO NB-IoT NTN Access with Rate and Call Setup Time Constraints

### 6.X.1	High level principles

This solution is to address Key Issue #2: IMS enhancement for GEO NB-IoT NTN access.

The following principles apply to IMS enhancement for this scenario:

- Fixed fields in SIP messages from the UE to the P-CSCF may be omitted, and completed by the P-CSCF.

- SIP messages from the P-CSCF to the UE include only necessary SIP headers; other headers are removed.

- The UE does not support pre-condition for resource reservation due to the bearer used for IMS signaling and voice are established in advance before SIP Registration procedure.

- The MO and MT call setup procedures involve only the exchange of essential information.

### 6.X.2	Description

The IMS enhancements include SIP signalling size reduction and reducing the number of required SIP messages.

6.X.2.1. SIP signalling size reduction

For SIP signalling size reduction, the UE transmits only essential headers. IEs marked “M” in RFC 3261[18] may be omitted and shall be supplied by the P-CSCF.

Editor’s note: Whether and how to handle certain headers that are not fully aligned with IETF specifications is FFS.

6.X.2.1.1. Reduction of SIP Registration Messages-Uplink

For SIP REGISTER message, the headers that require simplification or omission, which will be completed by the P-CSCF, are shown in the table below:

| header | Presence | Completed by P-CSCF | Note |
| --- | --- | --- | --- |
| Call-ID | M | / | Required to be carried by UE, can be simplified |
| From-tag | M | / | Required to be carried by UE, can be simplified |
| to-tag | M | / | Required to be carried by UE, can be simplified |
| Contact | C | Yes (complete Feature-Cap) | Retain hostport, P-CSCF adds feature-cap based on configuration or implementation |
| Supported | O | Yes (if not included) | / |
| Allow | O | Yes (if not included) | / |
| Max-forwards | O | Yes (if not included) | / |
| Via | M | / | Required to be carried by UE, branch can be simplified |
| Expires | O | Yes (if not included) | / |
| Accept | O | Yes (if not included) | / |

6.X.2.1.2. Reduction of SIP Registration Messages-Downlink

For SIP REGISTER Response message, the headers that require simplification and the corresponding simplification methods are provided in the table below:

| header | Presence | Simplification method |
| --- | --- | --- |
| Via | M | In the SIP request sent by the P-CSCF to the UE, the Via header is not simplified (only one Via is included if topology hiding is enabled). In the SIP response, the received parameter is omitted if the source IP matches the sent-by. |
| Contact | M | Remove all header parameters of the Contact header |
| Content-Length | O | Can be omitted or simplified |
| Supported | O | Can be omitted or simplified |
| Allow | O | Can be omitted or simplified |

The following SIP header parameters can be delivered to the UE without any simplification: Call-ID, From-tag, To-tag, CSeq, Accept-Resource-Priority, Service-Route, P-Associated-URI, Path, Reason, and WWW-Authenticate.

6.X.2.1.3. Reduction of Call Setup Messages - Uplink

For uplink Call Setup message toward P-CSCF, the headers that require simplification or omission, which will be completed by the P-CSCF, are shown in the table below:

| header | Presence | Completed by P-CSCF | Note |
| --- | --- | --- | --- |
| Call-ID | M | / | Required to be carried by UE, can be simplified |
| From-tag | M | / | Required to be carried by UE, can be simplified |
| To-tag | M | / | Required to be carried by UE, can be simplified |
| Contact | C | Yes (complete Feature-Cap) | Retain hostport, P-CSCF adds feature-cap based on configuration or implementation |
| Supported | O | Yes (if not included) | / |
| Allow | O | Yes (if not included) | / |
| Max-forwards | O | Yes (if not included) | / |
| Via | M | / | Required to be carried by UE, branch can be simplified |
| P-Preferred-Identity | C | / | Not included in initial INVITE(P-CSCF retrieves PUI from From); Included if anonymous |
| Route | O | Yes (derived from Record-Route in in-dialog requests, no simplification) | Not included in the initial INVITE; |
| Record-Route | O | Yes (if not included) | Not included in the initial INVITE |
| Min-SE | O | Yes (if not included) | / |
| Session-Expires | O | Yes (if not included) | / |
| P-Preferred-Service | O | Yes (if not included) | / |
| Accept-Contact | O | Yes (if not included) | / |
| Accept | O | Yes (if not included) | / |
| P-Access-Network-Info | O | Yes (if not included) | / |

6.X.2.1.4. Reduction of Call Setup Messages - Downlink

For downlink Call Setup messages, the headers that require simplification and the corresponding simplification methods are provided in the table below:

| header | Presence | Simplification method |
| --- | --- | --- |
| Via | M | In the SIP request sent by the SBC to the UE, the Via header is not simplified (only one Via is included if topology hiding is enabled). In the SIP response, the received parameter is omitted if the source IP matches the sent-by. |
| Contact | M | Remove all header parameters of the Contact header |
| Content-Type | O | Can be omitted or simplified |
| Content-Length | O | Can be omitted or simplified |
| Supported | O | Can be omitted or simplified |
| P-Preferred-Service | M | If a tel URI formatted PAI exists, retain only this PAI. If only a SIP URI formatted PAI exists, no simplification is applied. |
| Session-Expires | O | Can be omitted or simplified |
| Allow | O | Can be omitted or simplified |
| P-Asserted-Service | O | Can be omitted or simplified |
| P-Preferred-Service | O | Can be omitted or simplified |
| Accept-Contact | O | Can be omitted or simplified |

The following SIP header parameters can be delivered to the UE without any simplification: Call-ID, From-tag, To-tag, CSeq, Route, Record-Route, Min-SE, RSeq, Rack, Privacy, Reason.

6.X.2.1.5. Reduction of SDP Negotiation-UPLINK

For SDP carried in uplink SIP call setup message, the headers that require simplification and the corresponding simplification methods are provided in the table below:

| header | Simplification method |
| --- | --- |
| o-line IP Address | set to 1.1.1.1 (IPv4) or ::1 (IPv6) |
| s-line | set to s=- |
| c-line | include only one c-line |
| b-line | v-line does not carry b-line; only the b-line under m-line is retained |
| a=sendrecv | not included |
| precondition parameters | not included |

6.X.2.1.6. Reduction of SDP Negotiation -DOWNLINK

For SDP carried in downlink SIP message, the headers that require simplification and the corresponding simplification methods are provided in the table below:

| header | Specific method |
| --- | --- |
| o-line IP Address | set to 1.1.1.1 (IPv4) or ::1 (IPv6) |
| s-line | set to s=- |
| c-line | duplicate c-lines (with the same IP address) in the m-line are removed |
| b-line | delete the b-line under the v-line, retaining only the b-line under the m-line. |
| a=sendrecv | not included |
| precondition parameters | delete the precondition parameters |

6.X.2.2. Reducing the number of required SIP messages

The default bearer (for IMS signalling) and dedicated bearer (for IMS voice) are pre-established during PDN connection establishment procedure, and the IMS call setup procedure is conducted without the pre-condition. The detailed procedure is described in clause 6.X.3. The SIP signaling exchange between the UE and the P-CSCF comprises the following messages: SIP INVITE, 180 ringing,  200 OK, and ACK.

6.X.2.3	 SIP Timer Optimization

In GEO satellite access scenarios, if the default SIP timers (e.g., T1/T2 retransmission timers) designed for terrestrial networks are retained, the propagation delay (~270 ms one-way, ≥540 ms round-trip) will cause timer expiration. This results in unnecessary signaling retransmissions, increasing network congestion and call setup failure rates. To avoid this issue, the timer thresholds should be extended to align with GEO latency characteristics, effectively prevent retransmissions and ensuring reliable SIP protocol operation and session establishment in high-delay environments.

Table 6.X.2.3-1: SIP Timer Optimization Requirements

| Timer | Standard Value | Definition | Suggest Value | Optimization Information |
| --- | --- | --- | --- | --- |
| T1 | 0.5s | Round-trip message delay | 12s | Based on the formula of TR 23.700-19 A.1.2. The time required to send a SIP message: 8 × 1.3 KB (maximum round-trip message size) / 1 kbps (satellite link rate) + 0.6 (RTT delay) s = 11 s, recommended to adjust to 12 s. |
| T2 | 4s | Maximum retransmission interval for non-INVITE request messages and INVITE response messages | 24s | It is recommended to set T2 ≥ 2 × SIP response transmission time (24 s), ensuring at least 1 retransmission. |
| T4 | 5s | Maximum lifetime of a SIP message in the network | 26s | The time to stop retransmissions must be greater than the T2 timer duration. |
| Timer A | T1 | Retransmission interval for INVITE messages when using UDP transport | 12s | Same as the T1 timer setting. |
| Timer B | 64T1 | INVITE transaction timeout duration | 96s | For the maximum total retransmission duration timer, the intervals are 12 s, 24 s, 48 s, 96 s, set to 96 s for a total of 4 retransmissions. Previously, 0.5 s, 1 s, 2 s, 4 s, 8 s, 16 s, 32 s allowed 7 retransmissions. |
| Timer C | 180s | Proxy server timeout duration for processing INVITE transactions | - | The maximum duration of an INVITE transaction, after process simplification, is less than 180 s. No adjustment is needed. |
| Timer D | 64T1 | Time to wait for retransmitted response messages after receiving a response | 96s | Same as the Timer B setting. |
| Timer E | T1 | Retransmission interval for non-INVITE request messages when using UDP transport | 12s | Same as the T1 timer setting. |
| Timer F | 64T1 | Non-INVITE transaction timeout duration | 96s | Same as the Timer B setting. |
| Timer G | T1 | Retransmission interval for INVITE response messages | 12s | Same as the T1 timer setting. |
| Timer H | 64T1 | Time to wait for an ACK after sending a final response | 96s | Same as the Timer B setting. |
| Timer I | T4 | Time to continue waiting for retransmitted ACKs after receiving an ACK | 26s | Same as the T4 timer setting. |
| Timer J | 64T1 | Time to continue waiting for retransmitted non-INVITE requests after sending a final response to a non-INVITE request | 96s | Same as the Timer B setting. |
| Timer K | T4 | Time to continue waiting for retransmitted response messages after receiving a response | 26s | Same as the T4 timer setting. |

Editor’s note: Whether UE need to configure multiple sets of SIP timers, and the mechanism for how the UE enables different types of SIP timers based on the access type is FFS.

6.X.2.4	 IMS Transcoding Optimization

The terminal accessing high orbit satellite uses specific low bit rate codec of 1-3kbps. The existing ground network codec bit rate is greater than or equal to 4.25kbps. For the interconnection between the satellite network and the ground network, it is necessary to perform transcoding procedure within the IMS. The specific low bit rate codec bit flow that passes through IMS needs to be transcoded to the ground network codec to achieve successful media negotiation. After the call is setup, the IMS completes the satellite-ground transcoding of the media.

6.X.2.5. Evalution

6.X.2.5.1. MO call evaluation

Mobile-Originated (MO) Call Signaling Optimization Analysis

|  | Direction | Original Size (byte) | Simplified Size (byte) |
| --- | --- | --- | --- |
| INVITE | UE - P-CSCF | 2003 | 715 |
| 180 ringing | P-CSCF - UE | 924 | 644 |
| 200 OK | P-CSCF - UE | 1488 | 1117 |
| ACK | UE - P-CSCF | 576 | 472 |
| Total | / | 4991 | 2948 |
| Total Before Ringtone | / | 2927 | 1359 |

6.X.2.5.2. MT call evaluation

Mobile-Terminated (MT) Call Signaling Optimization Analysis

|  | Direction | Original Size (byte) | Simplified Size (byte) |
| --- | --- | --- | --- |
| INVITE | P-CSCF - UE | 1938 | 1106 |
| 180 ringing | UE - P-CSCF | 751 | 441 |
| 200 OK | UE - P-CSCF | 1226 | 781 |
| ACK | P-CSCF - UE | 404 | 326 |
| Total | / | 4319 | 2654 |
| Total Before Ringtone | / | 2689 | 1547 |

Editor’s note: Details of the SIP messages and the corresponding sizes are FFS.

6.X.2.5.3. Estimation of call setup time before ringing

The experienced data rate ranges from 1 to 3 kbps.

For the propagation delay the estimated number is based on PDB value for QCI 10 as specified in TS 23.003 cluase 6.1.7.2: 1100 ms.

Editor’s note: which QCI are applicable for the dedicated EPS bearer is FFS and need coordination with RAN WGs.

The signalling before ringtone include SIP INVITE, 180 ringing.

Based on Annex B.1 in TR23.700-19, the call setup time for MO UE is: 1359*8/(1k~3k)+.0.28s*(3+1)=4.744s~11.992s

Based on Annex B.1 in TR23.700-19, the call setup time for MT UE is: 1547*8/(1k~3k)+.0.28s*(3+1)=5.245s~13.496s

### 6.X.3	Procedures

6.X.3.1	SIP Registration Procedure

The following figure depicts the SIP registration procedure, with the IMS voice bearer pre-established and the TFT configured in advance as specified in clause 6.2.2.1 of TR 23.700-19.

Step 0. The dedicated bearer for IMS voice is pre-established, and the TFT has been configured. The address of the P-CSCF supporting voice over GEO is provided to the UE during the PDN session establishment procedure.

Step 1. UE sends a simplified SIP REGISTER request as described in clause 6.X.2.1.1, indicating to P-CSCF that the resource for IMS voice has already been reserved. P-Access-Network-Info header included in REGISTER request implies that the UE is accessed via a GEO.

Step 2. P-CSCF completes SIP REGISTER request by adding the omitted and simplified headers and forwards the completed request to IMS core.

Step 3,4. IMS core sends 401, P-CSCF simplifies it as described in clause 6.X.2.1.2 and sends to UE.

Step 5. UE performs authentication.

Step6,7. UE sends a simplified SIP REGISTER, which contains the same information as the initial request, with the addition of an authentication response. The P-CSCF handles it identically to Step 2.

Step 8,9. IMS core sends 200 OK, P-CSCF simplifies it as described in clause 6.X.2.1.2 and sends to UE.

6.X.3.2	Mobile Origination Procedure

The following figure depicts the IMS Mobile origination call setup Procedure, where the MO UE accesses the network via NB-IoT over GEO.

Step 1. Establish the default bearer for SIP signaling and dedicated bearer for voice in advance.

Step 2 UE has registered in IMS as clause 6.X.3.1.

Step3. The MO UE sends the SIP INVITE message including the satellite codec and basic headers as specified in clause 6.X.2.1. The omitted information is completed by the P-CSCF based on operator policy or implementation.

Step4 . P-CSCF complete the SIP INVITE and forwards to IMS core. The P-CSCF checks the SIP message and decides whether additional codec(s) need be inserted into SIP message based on the session information and interworking agreement.

Step5-10. The P-CSCF handles SIP 100 Trying, receives 183 Session Progress with SDP, sends PRACK, and processes 200 OK (PRACK). Based on the resource reservation indication received during SIP REGISTER, it skips resource reservation. It then directly forwards the SIP UPDATE from the IMS core and processes the 200 OK (UPDATE) response.

Step11.IMS CORE send completed 180 ringing to P-CSCF.

Step12. P-CSCF delete the header specified in clause 6.X.2 and send the simplified signaling to UE.

Step13. IMS CORE send completed 200 OK to P-CSCF.

Step14. P-CSCF delete the header specified in clause 6.X.2.1 and send the simplified signaling to UE.

Step15. UE sends ACK to P-CSCF, which will be completed and forwarded to IMS CORE.

6.X.3.3	Mobile Termination Procedure

The following figure depicts the IMS Mobile termination procedure, where the MT UE accesses the network via NB-IoT over GEO.

The principles and handling procedures for IMS signaling are consistent with those defined in clause 6.X.3.2, and are outlined as follows:

- The default and dedicated bearers are pre-established in step 1.

- The P-CSCF receives the complete SIP signaling from the IMS CORE, removes specific elements as specified in clause 6.X.2, and forwards the modified signaling to the UE in steps 3 and 12.

- Step 4 will be executed when the UE is in IDLE state.

- The UE transmits simplified SIP signaling as specified in clause 6.X.2 in steps 11,13, which will be completed by the P-CSCF as operator policy or implementation.

Editor's note:	FFS how the P-CSCF to be configured with appropriate context to complete the omitted information.

- The SDP answer is conveyed in the 200 (OK) response at step 13.

- The P-CSCF skips resource reservation based on the resource reservation indication received during SIP REGISTER.

### 6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	Further impact is FFS.

UE:

-	Support transmission and reception of simplified SIP signaling.

-	Support send SIP INVITE without pre-condition and early-media.

-	Support indication of the resource reservation state.

P-CSCF:

-	Support completion of simplified SIP signalling.

-	Support simplification of SIP signaling.

-	Support handling of SIP INVITE messages without precondition.

-	Support B2BUA.

-	Support bypassing the authorization process.

IMS-AGW:

- Support the transcoding between satellite codec(ULBC) and Terrestrial codec(e.g. AMR, EVS, etc.)

* * * End of Change * * * *