3GPP SA WG2 Meeting #170	S2-2507604

Gothenburg, Sweden, 25-29 August 2025

# Title:	Reply LS on LI requirements on IMS Data Channel

# Response to:	LS on LI requirements on IMS Data Channel (S3i250185/ S2-2506116)

# Release:	Release 19

# Work Item:	NG_RTC_Ph2

Source:	SA2

To:	SA3-LI, SA4

Cc:	CT1, SA3

Contact Person:

Name:	Kefeng Zhang

Tel. Number:

E-mail Address:	kefezhan AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks SA3-LI for the LS regarding LI requirements on IMS Data Channel.

SA2 would like to recommend SA4 and SA3-LI to find solutions to meet the LI requirement on IMS Data Channel in the following scenarios:

- Roaming cases with S8HR / N9HR model: SA2 would like to ask SA4 to study the possibility of removing encryption in DTLS or providing the information necessary to decrypt the Data Channel content (e.g. keys, algorithms, parameters) when any UE is in roaming state.

- Interoperability case between two CSPs, or in direct communications between two users P2P: When the MF acting as "HTTP Proxy" or "DC Application Proxy", the P2A, P2A2P Data Channel are always terminated in the MF which can provide a copy of the IMS Data Channel content after decryption for LI. SA2 kindly requests SA4 to confirm whether P2A Data Channel can be deployed with an HTTP Proxy or DC Application proxy mode. To support the LI requirements for P2P data channel, the serving IMS network can be configured to anchor the P2P Data Channel of the target UE or all UEs in the Media Function (MF) which acts as "HTTP Proxy" or "DC Application Proxy". The MF can provide a copy of the IMS Data Channel content after decryption.

- Interoperability between two CSPs, in which one is using IMS Data Channel and the other is using IMS without any IMS Data Channel feature: If the target UE is using the IMS without any Data Channel, and interworking between the DCMTSI UE and MTSI UE (i.e., the LI target) for specific DC application is required (as specified in clause AC.7.9 of TS 23.228), the IMS data channel content from the DCMTSI UE is terminated in the MF or DC AS. The MF or DC AS may support the interworking with the MTSI UE via IMS video flow or other existing mechanism (e.g. SMS, HTTP via Internet DN. It is assumed that LI for such interworking scenarios (IMS video, SMS, Internet) with the MTSI UE is supported by existing specifications. SA2 has not identified any gaps in supporting LI for these scenarios.

2. Actions:

To SA3-LI:

ACTION: 	SA3-LI is kindly requested to take the above information into account.

To SA4:

ACTION: 	SA4 is kindly requested to take the above information into account and provide any feedback.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, China

TSG-SA2 Meeting #172		17-21 November 2025	Dallas, TX, USA