3GPP TSG-WG SA2 Meeting #170

Goteberg, Sweden, August 25 - 29, 2025	S2-2507742

Source:	Ericsson

Title:	BDT invalidation and re-negotiation for network energy optimization when BDT is ongoing

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_ph2/Rel-20

Abstract of the contribution: PCFs applies normal policy for ongoing BDT by invalidating existing BDT policy and renegotiating with AF to select new BDT policy.

# 1. Introduction/Discussion

There are solutions on enhancing BDT request and BDT warning notification procedures considering energy related information in Release 19. The re-negotiation is made before BDT is started. However, the BDT can take a long time to finish if the data volume to be transferred is large. The energy related information of the network or the involved UPF/RAN may change when BDT is ongoing, which requires the core network to apply energy optimization related operations before the BDT is finished.

The pCR proposes to enhance existing BDT warning notification procedures to provide a solution for PCF to invalidate ongoing BDT policies considering network energy related information, to renegotiate of new BDT policies with AFs and to apply policy decision such as apply normal charging if the BDT is still ongoing after the re-negotiation.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-67

First change

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |  |
| X |  | X |  |  |

Second change (all new)

## 6.X	Solution #XX: BDT invalidation and re-negotiation for network energy optimization when BDT is ongoing

## 6.X.0	High-level solution principles

The solution is based on following principles:

- The PCF takes policy decision based on energy related information to re-negotiate with the AF to stop ongoing BDT and select a new BDT policy when BDT is ongoing. The trigger can be for example, the energy consumption or CO2 emission of a slice or the 5GC network go above a threshold, or renewable energy ratio of a slice or the whole 5GC network go below a threshold.

- A new reason causes is added in the BDT notification to the AF to inform the AF that the BDT policy for the ongoing BDT is no longer valid, and a policy decision such as a different (reduced) charging for BDT SDFs won’t be applied due to energy optimization reason if the AF continues.

- The PCF provides new candidate BDT policies, which considering both network performance and energy optimization, via NEF in the BDT warning notification to the AF.

- The PCF provides new invalidating graceful period in the notification to wait for the AF to take action to stop ongoing BDT activities and to select a new BDT policy.

- After the invalidating graceful period, the PCF applies policy decisions for affected BDT SDFs.

- It’s up to operator policy if subsequent BDT policies are set more aggressive or more relaxed.

### 6.X.1	Functional Description

This solution addresses KI#2.

Based on Energy Related information triggers, a PCF triggers BDT re-negotiation for network energy optimization and invalidates affected BDT policies after an invalidating grace period. The PCF marks the ongoing PDU-Sessions/BDT-SDFs pending for policy decisions. The trigger can be for example, the energy consumption or CO2 emission of a slice or the 5GC network go above a threshold, or the reviewable energy ratio of a slice or the whole 5GC network go below a threshold.

Upon receipt of the notification of BDT policy changes, the PCFs which are handling PDU-sessions/SDFs for BDT receive the notification of BDT policy changes from UDR. The PCFs mark the affected PDU-session/BDT-SDFs pending for actions after the invalidating grace period,

The PCF triggering the re-negotiation notifies AF for new candidate BDT policies with new timer window, reason causes for such a notification and invalidating grace period.

According to operator policy, all PCFs which are handling PDU sessions for BDT SDF

  - apply normal charging policy for affected BDT SDF if changeChargeableParty API (TS ) is used to activate the going BDT SDFs.

  - if changeChargeableParty API is not used

    - If EnhancedBDT is supported and used, the PCF may terminate the affected PDU sessions.

    - If EnhancedBDT is not used, PCF may terminate the SDF of BDT by interacting with SMF following Section 4.16.5.2 PCF initiated SM Policy Association Modification,TS 23.502.

### 6.X.3	Procedures

The picture below shows the procedures.

Triggering PCF: the PCF which checks the energy related information and initiates the BDT re-negotiation.

Participant PCFs: PCFs which handle ongoing PDU sessions related to the affected BDT policies. The triggering PCF can also be a participant PCF.

Prerequisite:

- The Background Data Transfer (BDT) was started.

- The triggering PCF subscribed to the energy related information of application and slice from the EIF and OAM.

- All PCFs subscribed to BdtData change notification according to section 5.2.2.6.2 NF service consumer subscribes to notifications to UDR in TS 29.522, and section 5.3.2 Policy Data Change in TS 29.519

- The triggering PCF subscribed to analytics on "Network Performance" described in TS 23.288 [50] for the area of interest and time window of a background data transfer policy.

Figure 6.X.3.1: BDT invalidation and re-negotiation when BDT is ongoing

- The Triggering PCF decides to take policy decisions for energy optimization when (the examples are as follows)

- the energy consumption, co2 emission for a network slice (from OAM) or the 5GC go above a threshold.

- the reviewable energy ratio of a slice or the 5GC network go below a threshold.

- the energy consumption for an application of the AF goes above a threshold.

- the reviewable energy ratio of AF or the 5GC goes above a threshold

- etc

- The Triggering PCF Request from the UDR the stored BDT policies using Nudr_DM_Query (Policy Data, Background Data Transfer) service operation.

- The UDR provides all Background Data Transfer Policies received from the AF (as defined in clause 6.1.2.4 of TS 23.503 [20]) to the Triggering PCF.

- The Triggering PCF identifies the BDT policies affected by the energy saving policy decision and which has energy indicator which means allowing network to perform service adjustment. For each of them, the Triggering PCF determines the AFs of by which the BDT will be influenced. The Triggering PCF performs the following steps for each of the determined AF. The steps onward can occur multiple times (i.e. one per AF).

NOTE: Assume the BDT policies of an AF which provides energy indicator should request to be notified for BDT policy changes due to energy saving reason.

- The Triggering PCF prepare a new list of candidate Background Data Transfer policies. If the Triggering PCF does not find any new candidate BDT policy, the previously negotiated BDT policy shall be kept, no interaction with that ASP shall occur and the procedure stops for that BDT policy.

- The Triggering PCF sets the invalid BDT policy in the UDR as invalid, add an invalidating graceful period invoking Nudr_DM_Update (Background Data Transfer Reference ID, invalidation flag) service. This IE indicates a timer value which the PCF will wait before applying policy decision for ongoing PDU-Sessions/BDT-SDFs.

- The UDR sends a response to the Triggering PCF as acknowledgement.

- The UDR sends a notification of the updated BDT policy with btdStatus as INVALID and an invalidating graceful period to all Participant PCFs who have subscribed to BdtData change notification.

- The Participant PCFs send a response to the UDR as acknowledgement.

- The Participant PCFs upon receipt of the UDR notification mark all affected PDU sessions/SDFs affected by the invalid BDT policy as pending for policy decisions after the invalidating graceful period.

- The Triggering PCF sends the notification to the NEF by invoking Npcf_PolicyControl_Notify (Background Data Transfer Reference ID, list of candidate Background Data Transfer policies, invalidating graceful period, causes) service operation.

- The NEF sends the BDT notification to the AF by invoking Nnef_BDTPNegotiation_Notify (Background Data Transfer Reference ID, list of candidate Background Data Transfer policies, invalidating graceful period, causes) service operation.

- The AF acknowledges the receiving of the request to the NEF.

- The NEF forwards the acknowledgement to the Triggering PCF.

One of the following Alternatives will be executed

Alt A.  If the AF selects any of the new Background Data Transfer policies, the flowing steps are applied:

A1. The AF invokes the Nnef_BDTPNegotiation_Update service to provide the NEF with Background Data Transfer Reference ID and the selected background data transfer policy.

A2.  The NEF invokes the Npcf_BDTPolicyControl_Update service to provide the Triggering PCF with the selected background data transfer policy and the associated Background Data Transfer Reference ID.

A3.  The Triggering PCF sends the acknowledge message to the NEF.

A4.  The NEF sends the acknowledge message to the AF.

A5.  The Triggering PCF stores the Background Data Transfer Reference ID together with the new Background Data T Transfer policy, the corresponding related information. At this stage, the BDT status is still  invalid.

A6.  The UDR sends a response to the Triggering PCF as its acknowledgement.

A7. The Triggering PCF waits for time specified in invalidating graceful period.

A8.  The Triggering PCF marks the BDT policy status shall still be VALID with invalidating graceful period.

A9.  The UDR sends a response to the Triggering PCF as its acknowledgement.

A10. In case EnhancedBDT is supported, PCF updates the URSP rule corresponding to the BDT policy with the new Validation Criteria in the Policy Set Entry of all affected UEs and send the updated rule to the UEs.

Editor’s Note: It’s FFS when the affected UEs will apply the new URSP Rule.

Alt B.  If the AF does not select a Background Data Transfer policy, the flowing steps are applied (This means the AF indicates that none of the candidate Background Data Transfer policies is acceptable).

Note: This alternative is the existing procedure.

B1. The AF invokes the Nnef_BDTPNegotiation_Update service to provide the NEF with Background Data Transfer Reference ID without a selected background data transfer policy.

B2.  The NEF invokes the Npcf_BDTPolicyControl_Update service to provide the Triggering PCF the Background Data Transfer Reference ID without a selected background data transfer policy.

B3.  The Triggering PCF sends the acknowledge message to the NEF.

B4.  The NEF sends the acknowledge message to the AF.

B5.  The Triggering PCF deletes the stored Background Data Transfer policy.

B6.  The UDR sends a response to the Triggering PCF as its acknowledgement.

B7. The Triggering PCF removes the USRP rule associated with the invalidated BDT policy in case EnhancedBDT is used.

Alt C.  If the AF does not send Nnef_BDTPNegotiation_Update, the flowing steps are applied:

C1.  The Triggering PCF waits for time specified in invalidating graceful period.

C1.  The Triggering PCF deletes the stored Background Data Transfer policy in UDR.

C3.  The UDR sends a response to the Triggering PCF as its acknowledgement.

C4. The Triggering PCF removes the USRP rule associated with the invalidated BDT policy in case EnhancedBDT is supported.

- after the expiry of invalidating graceful period for the invalid BDT policy:

For each associated PDU session/SDF for BDT:

Alt1: if changeChargeableAPI is used to initiate the BDT

16.  The PCF updates charge policy of the PCC rules for BDT SDFs to a normal charging group by interacting with SMF following Section 4.16.5.2 PCF initiated SM Policy Association Modification, TS 23.502.

Note: The AF is expected interact with UEs to postpone current BDT within invalidating grace period, the associated PDU sessions/SDFs only exist if the AF decides to continue the BDT.

Alt2: if changeChargeableAPI is not used to initiate the BDT, according to operator policy:

17. PCF may terminate the SDF of BDT by interacting with SMF following Section 4.16.5.2 PCF initiated SM Policy Association Modification, TS 23.502 or PCF may terminate the PDU session of BDT if enanhacedBDT is supported and used.

### 6.X.4	Impacts on existing services, entities and interfaces

PCF:

- checks the energy related information and invalidates ongoing BDT policy and initiates the BDT re-negotiation with AF.

- receives notification from UDR for BDT policy data change and prepares policy decisions for affected PDU-sessions/SDFs.

- waits for a graceful period to perform policy decisions for affected PDU-sessions/SDFs after re-negotiation.

UDR:

- supports new INVALID state of bdtpStatus of a BDT policy and new invalidating graceful period in BdtData

AF:

- stops current BDT if it wants BDT specific (reduced) charging and selects a new BDT policy from re-negotiation.

# * * * * End of changes * * * *