3GPP TSG-SA WG2 Meeting #170	S2-2507783

Goteborg, SE, Aug 25 - 29, 2025	(revison of S2-2507723)

Source:	OPPO

Title:	New Solution for KI#5: Sensing result delivery to the consumer

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution on Sensing result delivery to the consumer.

- Discussion

KI#5 have been defined in TR 23.700-14 as below:

This key issue addresses the exposure of sensing results to support sensing services. The aspects to be studied in this key issue include: - Study what types of sensing results with or without the sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the sensing service consumer. - How the Core Network exposes sensing results to the sensing service consumer (e.g., one time, periodic or event trigger).

For sensing result exposure, new sensing result may be derived from sensing data. Data collection often triggers sensing measurements. However, since multiple requests from diverse applications may seek identical results, reusing stored outcomes avoids redundant processing. This approach not only streamlines operations but also aligns with energy efficiency goals by minimizing unnecessary data generation and computation.

Sensing data storage preserves historical and real-time environmental context. Stored data ensures consistency for cross-service validation (e.g., AI training, network optimization) and supports regulatory requirements. For result exposure, centralized storage simplifies access via standardized APIs, allowing third parties or internal systems to retrieve processed insights (e.g., obstacle maps, interference patterns) without raw data duplication. It also facilitates on-demand querying, reducing redundant data collection. By decoupling storage from processing, stored datasets enable scalable, repeatable analytics, ensuring high-quality sensing results are dynamically exposed for applications like autonomous mobility or predictive network maintenance.

Sensing services, as a type of data service, can share the same data collection and capability exposure architecture with other data services (e.g., AI, positioning). However, due to the unique requirements of sensing services, there are also dedicated operations like sensing entity selection and sensing data processing. Thus, sensing service management is jointly implemented by the GSMF (Gateway Sensing Management Function) and SSMF (Sensing Service Management Function). The GSMF manages unified data collection and capability exposure. The SSMF handles sensing entity selection, sensing result generation, and storage.

This solution primarily focuses on a GSMF-based capability exposure framework, leveraging shared infrastructure for efficiency while ensuring specialized processing for sensing-specific tasks through the SSMF. By integrating sensing into the broader data service ecosystem, operators avoid redundant architectures while maintaining tailored support for sensing requirements like real-time entity coordination and environmental context handling.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X | X |  |  | X |  |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: Sensing result delivery to the consumer

### 6.X.0	High-level solution Principles

This solution is for Key Issue#1 and Key Issue#5 on Sensing result delivery to the consumer.

- GSMF (Gateway Sensing Management Function) is introduced to control sensing data and the associated information collection and sensing result exposure, which is centrually deployed and discovered based on configuration.

- SSMF (Sensing Service Management Function) is introduced to handle Sensing Entity selection, data collection, sensing result generation and storage, which is deployed in a distribution manner.

- The sensing result can be stored in a sensing result storage NF.

- The sensing service consumer(s) is the trusted AF.

- The sensing result can be transferred over Control Plane (CP) or Data connection for storage and exposure.

- If there is available sensing result matching the service request (i.e. the sensing result fufills service requirement indidated in the service request) in the sensing result storage NF, it can be provided to the consumer without triggering sensing procedure to the Sensing Entity.

Editor’s Note: Whether the stored result can be applicable for the new sensing service request is FFS.

### 6.X.1	Description

To enable sensing result exposure, the GSMF needs to:

- Determine the sensing result provider who transfers the sensing result to the consumer:

  - Based on operator configuration, GSMF may first check with the sensing result storage NF if there is available sensing result matching the service request

  - If there is available sensing result matching the service request, the sensing result storge NF is the sensing result provider;

  - Otherwise, SSMF is the sensing result provider

- Determine the sensing result transferring mode: Control Plane (CP) or Data connection.

- Control delivering the sensing results from the sensing result provider to the sensing service consumer(s).

To enable sensing result storage, the SSMF needs to:

- Determine the sensing result transferring mode: Control Plane (CP) or Data connection.

- Control delivering the sensing results from the SSMF to the sensing result storage NF.

If Data connection is required to deliver sensing results to the consumer and the sensing result storage NF, a Data connection needs to be established before transferring the sensing result.

GSMF/SSMF determines sensing result transferring mode based on the following info:

- Capability of the sensing result provider for transferring sensing result over data connection

- Capability of the sensing service consumer or the sensing result storage NF for transferring sensing result over data connection

- Sensing result format

- Sensing service type

- Operator configuration

### 6.X.2	Procedures

Figure 6.X.2-1: Service flow of sensing result exposure to AF and the storage

- GSMF receives a sensing service request from a trusted AF, and the service request is successfully authorized. The request includes:

- Consumer ID/Address

- Sensing service type

- Target sensing service area

- Expected sensing result，including sensing result format, time availability and QoS requirements

- Capability of AF for transferring sensing result over data connection

- GSMF checks with NF storing sensing result if there is available sensing result matching the service request by sending sensing service type, target sensing service area and expected sensing result received in step 1.

Editor's note:	More parameters to determine the stored sensing result matching the service request is FFS.

- If there is matching sensing result, the sensing result storage NF responds to GSMF including either Sensing result or Indication of transferring sensing result over data connection and the sensing result storage NF address

.

3. GSMF determines sensing result transferring mode (CP or data connection) based on the following information:

- Whether sensing result is received or indication of transferring sensing result over data connection received at step 2 from NF storing sensing data)

- Capability of the sensing service consumer for transferring sensing result over data connection, sensing result format and sensing service type received at step 1

- Operator configuration

4. If GSMF determines to transfer sensing result over CP, GSMF responds to sensing service consumer of the sensing result, and the rest steps are skipped.

5.

If GSMF determines transferring sensing result over data connection and there’s no available Data Connection, new Data Connection is established between sensing result storage NF and AF.

Editor's note:	Details of data connection establishment between sensing result storage NF and AF is FFS.

6. The sensing result is provided from sensing result storage NF to the AF over data connection, and the rest steps are skipped

7. If neither sensing result nor indication of transferring sensing result over data connection is provided in step 2, GSMF sends the sensing service request to the SSMF, including what is received in step 1.

8. SSMF triggers sensing data collection and result generation.

Editor's note:	Details of sensing data collection and result generation will be defined by solutions to KI#4.

Editor’s note:	Whether NG-RAN nodes can perform sensing data collection and result generation is RAN WGs decision and requires further coordination.

9.

SSMF determines sensing result transferring mode (CP or data connection) based on the following information:

- Capability of the sensing service consumer for transferring sensing result over data connection, sensing result format and sensing service type received at step 3

- Operator configuration

10. If GSMF determines to transfer sensing result over CP, GSMF responds to sensing service consumer of the sensing result, and the rest steps are skipped.

11. If GSMF determines to transfer sensing result over data connection and there’s no available data connection between SSMF and AF, new data connection is established.

Editor's note:	Details of data connection establishment between SSMF and AF is FFS.

12. The sensing result is provided from the SSMF to the AF over data connection.

13. If GSMF determines to transfer sensing result over CP, the SSMF notifies sensing result storage NF about the sensing result over CP.

14. If GSMF determines to transfer sensing result over data connection and there’s no available data connection between SSMF and sensing result storage NF, new data connection is established.

Editor's note:	Details of data connection establishment between SSMF and sensing result storage NF is FFS.

15. The SSMF notifies sensing result storage NF about the sensing result over the data connection.

16. Sensing result storage NF stores the sensing result.

### 6.X.3	Impacts on services, entities and interfaces

GSMF:

  - Determine the sensing result provider(s).

  - Determine the sensing result transferring mode.

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing service consumer(s).

  - Determine the sensing result storage entity(ies).

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing result storage NF.

SSMF:

  - Support sensing result generation

  - Support sensing result storage

Sensing Entity (i.g. gNB):

  - Support sensing data collection to SSMF

  - Support sensing measurement

Sensing result storage NF:

  - Support sensing result storage

* * * End of Change * * * *