3GPP TSG-SA WG2 Meeting #170	S2-2507906

Goteborg, Sweden, August 25 – 29, 2025        (revision of S2-2506982 and merged papers)

Source:	MediaTek Inc., OPPO

Title:	[WT#6, all topics] Computing service for 6G

Document for:	Approval

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution addresses scoping of WT#6 on Computing service for 6G covering Arch. Motivation, Gap analyses and expected Technical Impact and suggests some potential scope for relevant Key Issue(s).

R1:

Merged tdocs as below

S2-2506361 (China Mobile, AT&T, Rakuten Mobile, OPPO)

S2-2506387 (OPPO, China Unicom, China Telecom, China Mobile)

S2-2506441 (Samsung)

S2-2506680 (NEC, SoftBank)

S2-2506729 (ETRI)

S2-2506810 (NTT DOCOMO, T-Mobile USA, InterDigital Inc, Ofinno, KPN N.V.)

S2-25068899 (Apple)

S2-2506891 (Intel)

S2-2506924 (Lenovo)

S2-2506965 (China Telecom)

S2-2506982 (MediaTek)

S2-2506985 (vivo)

S2-2506995 (Huawei, Hisilicon, SK Telecom, China Unicom)

S2-2507020 (Rakuten Mobile, SK Telecom, SoftBank, NVIDIA, NEC, NIST, China Mobile)

S2-2507024 (LG Electronics)

S2-2507128 (ZTE)

S2-2507165 (Tencent, Tencent Cloud)

S2-2507184 (CATT)

S2-2507280 (Qualcomm)

S2-2507395 (Nokia, Ericsson)

S2-2507430 (Xiaomi)

Key issues mapping from WT scope

| General concepts description | Key issue item | Related proposals | Further Description |
| --- | --- | --- | --- |
| Overall architectural impact | X.1 and X.4 | S2-2506361, S2-2506441, S2-2506680, S2-2506729, S2-2506889, S2-2506891, S2-2506924, S2-2506982, S2-2506985, S2-2506995, S2-2507024, S2-2507128, S2-2507165, S2-2507184, S2-2507280, S2-2507395, S2-2507430 | new NF, re-use /enhance existing 5G MEC architecture or NFs, new or enhance exposure framework, etc. |
| How 6GC obtain the available computing resource information/compute nodes | X.3 | S2-2506361, S2-2506387, S2-2506441, S2-2506729, S2-2506891, S2-2506985, S2-2506995, S2-2507024, S2-2507128, S2-2507184, S2-2507430 | Computing resource could be hardware, software, application, in different type |
| How 6GC discover and select proper computing resource information | X.5 | S2-2506387, S2-2506387, S2-2506441, S2-2506680, S2-2506729, S2-2506889, S2-2506924, S2-2506924, S2-2506965, S2-2506985, S2-2507128, S2-2507184, S2-2507430 |  |
| How 6GC network allocate, monitor, release computing resource | X.6 | S2-2506361, S2-2506387, S2-2506441, S2-2506441, S2-2506680. S2-2506729, S2-2506995, S2-2507024, S2-2507184 |  |
| How6GC authorizes computing service consumer (e.g., UE, NFs, third party, etc. ) to use computing service | X.2 | S2-2506361, S2-2506387, S2-2506441, S2-2506729, S2-2506889, S2-2506891, S2-2506924, S2-2506965, S2-2506985, S2-2506995, S2-2507020, S2-2507184, S2-2507430 |  |
| How 6GC jointly consider communication requirements and computing requirements to guarantee an end-to-end delay required by a computing service | X.7 | S2-2506361, S2-2506387, S2-2506729, S2-2506810, S2-2506889, S2-2506924, S2-2506965, S2-2506982, S2-2506985, S2-2506995, S2-2507024, S2-2507128, S2-2507184 | Compute QoS/metrics/parameters |
| How 6GC re-allocate computing resource/computing nodes due to the mobility and provide service continuity for computing service | X.8 | S2-2506361, S2-2506729, S2-2506810, S2-2506889, S2-2506924, S2-2506985, S2-2506995, S2-2507020, S2-2507165, S2-2507184 | Compute node relocation/computing service handover, |

**** First Change ****

5.X. Key Issue #X: Support of Computing

In order to support computing, the following aspects need to be studied:

Key issue#6.1: Enablement/authorization of computing service to UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

Key issue #6.2: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource (e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

  - Whether and how to identify and/or collect the computing resource related information (e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics.

NOTE 1:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

  - Whether and how to characterize network metrics (e.g., by defining new metrics or enhancing the existing metrics for QoS) in order to improve the service experience/QoS.

  - Whether and how to support service continuity for computing service upon change of computing resource and/or user plane function (e.g., due to UE mobility, computing node load balancing, AF influence, etc.).

Key issue #6.3: Discovery and (re-)selection of compute site(s)/resource(s):

  - Whether and how to discover and (re-)select computing site(s)/resource(s) for UE/AF requested service.

  - Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548) to enable discovery or (re-)selection of Application Server providing services to UEs.

NOTE  2:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

**** End of Changes ****