3GPP TSG SA WG2 Meeting #170 	S2-2507992

25 - 29 August, 2025, Goteborg, Sweden 	was S2-2506688

# Source:	Nokia (Moderator)

# Title:	New SID: Study on IMS Architecture Enhancement

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on IMS Architecture Enhancement

Acronym: FS_ARCH_enIMS

Unique identifier:	TBC

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X |  | X |  |
| No |  |  | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_6G_REQ | SA WG1 | 1050110 | Study on 6G Use Cases and Service Requirements; Stage 1 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1060079 | Study on 6G Scenarios and Requirements | The architecture related requirements from RAN may need to be taken into account. |
|  | RAN WG studies (TBD) |  |

# 3	Justification

Empowered by the rapid development of advanced technologies such as Generative Artificial Intelligence (Gen AI), Large Language Models (LLMs) and AI Agent, and the emerging of new types of terminals such as AI glasses and embodied robots, operators will be able to provide various personalized and enriched communication services. 3GPP SA1 has started the FS_6G_REQ study item to identify use cases and service/operational requirements for new multi-media services. New use cases, e.g. intelligent communication assistant (TR 22.870 6.10), intelligent immersive calling service (S1-252487, approved), may bring new network requirements to both IP-CAN and IMS network. The approved S1-252891 also includes the requirement that IMS systems shall provide improved system capabilities for the Multimedia Telephony Service.

Interaction between AS(s) and CSCF

While looking into existing IMS architecture and mechanisms, several aspects could be enhanced to better support both existing and aforementioned new services, e.g.:

-	Service triggering efficiency: IMS relies on iFC (initial Filter Criteria) mechanism for service triggering, which requires any AS which may potentially be used during the call shall be triggered at initial INIVTE request. This may lead to waste of network resources since the invoked AS may not be used at all during the call.

-	Service triggering flexibility: Again, due to iFC mechanism, all the ASes are triggered in a predefined order. When a new service is deployed, operators have to figure out (manually) a proper place to insert the new service on the iFC chain, then the order of the services cannot be changed, which significantly reduces the flexibility for services interaction in real time. Also, after the initial INVITE exchange, there is no way to add new AS into the call, which reduces the service flexibility further.

-	Signalling cost and latency: In IMS network, when an AS send a SIP message executing service logic, the SIP message and relevant responses will be handled by every IMS entity stays on the signalling path. Therefore, the more SIP entities in the network, the more message handling required. This not only increases the signalling cost but also increases the latency of service execution.

Convergence of the media plane nodes

In 3GPP Rel-18 TS 23.228 [3], a new Media Function (MF) is introduced and supports all new media functionalities since then, e.g., data channel related functionalities, media rendering capabilities. On the other hand, MRF handles the legacy media capabilities, e.g., play tone, play announcement, DTMF collection etc. This fragmentation not only complicates the media establishment procedure but also creates conflicts. For examples, how to play announcement in an IMS data channel session is not defined. The capabilities of MRF are logically simple and have been stable for multiple 3GPP releases. Therefore, it is proposed to review the media capabilities in a holistic view and converge MRF and MF with SBI interfaces.

In addition, it may be further possible to further converge MF and IMS-AGW to reduce system resources overhead, promote system performance, and provide more flexible NF signalling with SBI interfaces. Almost all phone calls require the basic play tone functionality, which means almost all sessions need to anchor to MRF before they are established. Converging MF and IMS-AGW can also avoid the E2E media renegotiation procedure triggered by the called party answering the call.

Editor's note:	The justification text will be updated based the finally agreed objectives.

# 4	Objective

This study aims to enhance the IMS network architecture for both existing services and new service requirements identified in 6G.

All work tasks aim at supporting multi-vendor interoperable interfaces.

This study includes the following work tasks:

WT-1:	Study whether and how to simplify and enhance the IMS architecture including:

WT-1.1:	simplify/optimize the IMS control plane by converging control plane entities

Editor's Note:	WT#1.1 is not agreed yet as it has concerns raised from multiple companies. This needs further discussion.

WT-1.2:	simplify/optimize the IMS media plane by converging media plane entities.

WT-1.2a:	introduce service based interface for replacing H.248

Editor's Note:	WT#1.2a is not agreed yet as it has concerns raised from multiple companies. This needs further discussion.

WT-1.3:	optimise the interaction between AS(s) and CSCF

for triggering of AS and simplify AS chaining.

WT-2:	Whether and how to enhance the IMS architecture and procedures to support new services. This WT includes studying the following aspects:

WT-2.1:	whether and how introduce new capabilities to support new services defined by SA WG1.

WT-2.2:	whether and how to support offloading of media resource and capabilities from the UE(s) to IMS network besides rendering capability supported in Rel-19.

Editor's Note:	WT#2.2 is not agreed yet as it has concerns raised from multiple companies. This needs further discussion to clarify the scope.

NOTE 1:	WT#2 will be properly scoped when the related service requirements from SA WG1 will reach appropriate level of maturity.

WT-3:	whether and how to enhance the exposure of IMS capabilities for IMS media layer (e.g. monitoring and reporting of events from MGCF, MRF, MGW, IMS-AGW).

Editor's Note:	WT#3 is not agreed yet as it has concerns raised from multiple companies. This needs further discussion to clarify the scope.

NOTE 2:	This study assumes that the UNI signalling and inter PLMN IMS interfaces are supported for backward compatible consideration and IMS architecture remains access agnostic. An IMS network per the output of the study continues serving properly legacy terminals.

NOTE 3:	WT#1 will be more focusing on IMS optimization based on the existing IMS architecture.

NOTE 4:	The enhanced IMS will support regulatory services in backward-compatible way.

During the study, the progress and results of 3GPP TR 22.870 (SA1 study) shall be taken into account.

NOTE 5:	The content of the SID will be further refined on SA2 meeting.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT-1 | TBD | TBD | NO |  |
| WT-1.1 |  |  |  |  |
| WT-1.2 |  |  |  |  |
| WT-1.2a |  |  |  |  |
| WT-1.3 |  |  |  |  |
| WT-2 | TBD | TBD | NO |  |
| WT-2.1 |  |  |  |  |
| WT-2.2 |  |  |  |  |
| WT-3 | TBD | TBD | NO |  |

Total TU estimates for the study phase:     TBD

Total TU estimates for the normative phase:    TBD

Total TU estimate: TBD

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 23.xxx | Study on IMS Architecture Enhancement | TSG#xx (TBD) | TSG#xx (TBD) | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

NOTE: 	The timeline for the study will be decided at SA#110 (Dec 2025)

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential security impact to be covered by SA3/SA3-LI.

Potential multimedia and codecs aspects to be covered by SA4.

Potential charging and OAM impact to be covered by SA5.

# 9	Supporting Individual Members

Supporting IM name

AT&T?

Boost Mobile Network?

BT?

CATT?

China Broadnet?

China Mobile?

China Unicom?

Deutsche Telekom?

Honor?

Huawei?

KDDI?

KPN?

LG Uplus?

NEC?

OPPO?

Orange?

Rakuten Mobile?

SKT?

Telefonica?

T-Mobile USA?

Turkcell?

Vivo?

Xiaomi?

ZTE?

Telecom Italia?