SA WG2 Meeting #170	S2-2508039

25 – 29 Aug, 2025, Goteborg , SE	(was S2-2507984)

Source:	CATT, Thales, TNO, Novamint, vivo, Airbus

Title:	[WT#7]: Discussion and proposal on key issue for 6G satellite access

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the scope of WT#7 and corresponding Key Issue(s) for SA2 6G SID.

1. Justifications

According to the SA2 SID S2-2506096, 6G RAT for NTN has also already been a separate WT. The related items include:

-	WT#2: Study migration and interworking, including

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

-	WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

In addition, 3GPP RAN groups also agree to include NTN in the 6G Radio SI (see RP-251881) and will design harmonized 6G Radio for TN and NTN.

This paper would analyse the main works in SA2 to support satellite access in 6G system.

1.1 Architecture to support satellite access in 6G

1. Architecture for 6G network integrating satellite components

As both transparent and regenerative payload modes have already been supported in 5GS, it is clear that, the 6G network shall support both transparent and regenerative payload based satellite access.

However, the 5GS doesn’t support RAN node (in regenerative payload mode) mobility very well, as fast RAN nodes on-board NGSO satellite would lead frequent signalling exchange over N2/N3 interface and frequent AMF configuration update. In 6G, it is worth of having an architectural level designing for efficient network operation, e.g., Interworking Function (IWF) concept proposed in Rel-19 can be re-considered.

On the other hand, in EPS/5GS, to support some satellite specific features, like S&F, UE-satellite-UE communication, Core Network functions are required to be onboard satellites. Such features are important to satellite operators, and then are assumed to be supported in 6G as well, which requires supporting 6G CN network functions on board satellites.

In some mission critical scenarios, e.g., a disaster case, feeder link towards ground core network may be broken, the network would rely on the CN onboard satellites to provide mission critical service to satellite users.

Based on the above analysis, we have following proposals:

-	Proposal 1: study how does the 6G network support both transparent and regenerative payload based satellite access?

-	Proposal 2: study how does the 6G network support RAN node mobility efficiently?

-	Proposal 3: study whether and how to support selected core network functions onboard satellite access?

2. Interworking between 4G/5G NTN and 6G networks supporting satellite access

There will be network architectural requirements for the interworking between 4G/5G and 6G terrestrial networks, such requirements will also apply to the interworking between 4G/5G NTN and 6G networks supporting satellite access.

-	Proposal 4: No particular architectural requirement is required for the interworking between 4G/5G NTN and 6G networks supporting satellite access.

-	Proposal 5: No connected mode mobility procedure shall be defined for the interworking between 4G NTN and 6G satellite access.

1.2 Support of satellite specific features already implemented in EPS/5GS

Current EPS/5GS already support following satellite specific features:

-	Discontinuous coverage,

-	S&F,

-	UE-Satellite-UE communication,

-	and satellite backhaul

…

1. Discontinuous coverage

It is foreseen that sparse NSGO/GSO satellite constellations will also exist in 6G early phase, thus 6G has to support discontinuous coverage.

The main enhancements to support discontinuous coverage in 5GS/EPS include provision of coverage availability information to UEs, power saving optimization, and overload control procedures, which are important features to improve system efficiency, and should be supported in 6G day 1.

2. Store and Forward Satellite operation

This feature provides some level of service by storing and forwarding data when feeder link connectivity is intermittently unavailable, which is a key enabler for delay-tolerant services such as CIoT/MTC and SMS, and then is concerned by satellite operators.

Considering that sparse NSGO/GSO satellite constellations would exist in 6G early phase, it is proposed to support this feature for fast satellite service operation.

3. The UE-Satellite-UE communication

In 5GS, this feature enables UEs served by the same PLMN to communicate directly through satellite without routing user data back to ground network function, which can significantly reduce communication latency and save backhaul resources.

In addition, due to UE-Satellite-UE communication allowing exchange of user data without involving network function on the ground, it could be further enhanced to support mission critical services, e.g., IOPS, when ground gateway is broken due to disaster.

Therefore, it is proposed to study how to support UE-Satellite-UE communication in 6GS.

4. Dynamic satellite backhaul

Dynamic satellite backhaul naturally exists in a network supporting regenerative satellite access.

Dynamic satellite backhaul could also be used by terrestrial network where traditional terrestrial backhaul network is not available or subject to high cost, e.g., in remote area or under VMR scenario.

The feature of dynamic satellite backhaul in 5GS allows the network to dynamically change QoS/PCC policies to accommodate different backhaul characteristics, such as latency. To promise efficient resource utilization in a 6G system integrating NGSO satellite components, it is proposed to study how to support dynamic satellite backhaul in 6G from the first release.

Proposals:

-	Proposal 6: Gap analysis on how to support in 6G the NTN related capabilities specified in 5G advanced, including dynamic satellite backhaul, discontinuous network coverage, UE-Satellite-UE communication, and S&F.

1.3 Support of new requirements for satellite access

Besides above satellite-specific features, SA1 further defines following satellite related requirements:

1. Multi-orbit

As different satellite orbit types have different characteristics, e.g., LEO satellites can have lower communication latency over Uu interface, but their coverage are not stable; whereas, GEO can provide good coverage but have longer communication latency, it is proposed in the TS 22.261 that a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g., altitude, orbital characteristics, and satellite capabilities) shall support satellite selection and switching based on factors like, latency requirements, QoS requirements, access capabilities and availability, establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics.

Possible enhancements to 5GS due to multi-orbit include:

-	the RAN onboard selection for establishing or switching connections based on different criteria, e.g., QoS requirements, access capabilities and new criteria such as different orbit types for signalling and data;

-	when switching occurs, supporting service continuity between multi-orbit satellites

Figure 1.3-1 Multi-orbit example

According to 6G vision, there will be also multiple layers of satellite coverage, thus such requirements and potential enhancements shall also be applied to 6GS, i.e., the 6GS shall also study how to support multi-orbits.

Currently, these “multi-orbit” requirements are not implemented for 5GS yet. Several satellite operators would like to support this feature in 5GS, so we need to consider how to coordinate the study for 5GS and the study for 6GS. Due to 6G study in advance of 5G rel-21, it is proposed that, the solutions concluded for 6G should be considered for 5G (Rel-21), but do not prevent the study of alternative solutions for 5GS.

Based on the above analysis, we have following proposals:

-	Proposal 7: study how does the 6G network support multi-orbit satellites, including how to select the RAN node onboard to serve the UE, and how to support service continuity while switching connections between RAN nodes onboard different orbit satellites?

-	Proposal 8: the solutions concluded for 6G to satisfy multi-orbit requirements could be considered for 5G (Rel-21), but do not prevent the study of alternative solutions for 5GS

2. Mobility support considering Service continuity

SA1 defines 6G satellite service requirements regarding service continuity among satellite access networks, or between terrestrial-satellite access networks, for video calls in remote areas.

From our perspectives, the primary challenge to promise service continuity arises from significant changes of communication latency during mobility procedures, e.g., when a UE moves from a terrestrial network to GEO satellite access network. During such mobility procedures, the network QoS supporting capability may degrade, or even service interruptions may be caused for real-time services like voice calls.

Therefore, it is proposed to study how to promise service continuity among satellite access networks, or between terrestrial-satellite access networks in 6GS.

Proposals:

-	Proposal 9: study how to ensure service continuity across the different satellite access networks, or between terrestrial-satellite access networks in 6GS?

3. Multi-connectivity via TN and NTN simultaneously

Multi-connectivity via TN and NTN is a new approach of exploiting satellite access to improve user experience and network performance, where the UE simultaneously connecting to both TN and NTN access networks, with different services / applications traffic routed over the selected access based on service requirements and/or UE/network policy. Various use cases have been identified as described in TR 22.841, e.g. UAV operations for safe and efficient, reliable communication for connected vehicles IoT devices across different areas. But due to many reasons, TN-NTN Multi-connectivity is not supported in 5GS. In 6GS, this can be further considered and studied to meet the challenges of supporting more diversified user requirements and communication scenarios in future.

Proposals:

-	Proposal 10: study how does the 6G network support multi-connectivity via TN and NTN simultaneously?

1.4 Adaption of new 6G features to satellite access

1. AI for 6G NTN

In 6G, AI will provide new assistance for satellite communication. Although the 6G AI framework has not yet been defined, its design is intended to be universal across TN and NTN. Based on the consideration, the main particularity of AI for satellite access focuses on identifying what kind of AI capability should be onboard, as it may differ from that of ground-based systems.

For example, given the unique challenges of satellite operations, such as latency and bandwidth limitations, AI related data handling shall be adapted accordingly. Satellite based RAN/CN nodes with AI capabilities can be part of 6G AI framework, facilitating data collection onboard and data delivery between RAN/CN node onboard and CN on ground.

Furthermore, AI can drive TN-NTN joint resource management, such as real-time collection of satellite parameters (coverage range, orbit data, beam load) to achieve intelligent control and resource allocation of TN-NTN multi-layer networks. AI can assist in optimizing network performance and selecting the optimal path for data. Additionally, deploying/training AI models including satellite characteristics may significantly enhance satellite mobility management, ensuring that satellites evolve in alignment with changing network conditions and operational demands.

Proposals:

-	Proposal 11: study how AI can benefit to 6G NTN?

2. Computing service for 6G NTN

In satellite communication systems, the growing demand for data transmission requires enhanced efficiency and speed in data processing. To reduce data transmission latency and minimize backhaul resource consumption, leveraging onboard computing for tasks such as image processing and data compression is beneficial. However, the satellite environments present unique constraints that challenge the distribution and coordination of computing tasks.

For example, satellite computing resources are limited, which means that complex processing tasks cannot always be fulfilled in real time. Additionally, the computing capability of a satellite may vary throughout its operational lifespan, complicating the management and allocation of computational resources at different stages. Furthermore, the mobility of satellites introduces the issue of computational mobility, especially in scenarios with or without ISL, careful consideration is needed regarding how computing tasks are allocated.

Therefore, it is important to study how to distribute processing tasks among different suitable satellites, considering their computing capabilities, mobility, and connectivity status. Moreover, it is also necessary to study how to handle computing coordination and offloading for UEs to optimize user experience. If onboard computing cannot meet the demand due to resource limitations, it will be necessary to consider offloading tasks to the TN.

Proposals:

-	Proposal 12: study how to support on board computing service for 6G NTN?

2. WT Scope proposal

As analysed in the section 1, the potential sub-work tasks for WT-7 can be classified into:

Category 1: Architecture to support satellite access in 6G.

Category 2: Support of Satellite-specific features already implemented in EPS/5GS.

Category 3: Support of new requirements for satellite access

Category 4: Adaption of new 6G features to satellite access

Considering limitation of TU, we propose to prioritize the study on these work tasks as followed:

-	Category 1 shall take the highest priority during the study, and must be done in 6G day 1.

-	Category 3 takes the second level priority, and should be studied in 6G day 1 to show the characteristics of 6G NTN.

-	Category 2 also takes the second level priority, but may not require too much study as such work tasks have already been studied for 4GS/5GS. So the gap analysis can be performed after the conclusion for 6G network architecture settles down. Such features can be implemented during 6G normative phase.

-	Category 4 takes lower priority, and then can be studied later.

Based on above justifications, the following sub-WTs are proposed in clause Annex A.X WT#7 Scope.

3. References

[1]	SP-250806_SP-250488 SA2 6G SID

[2]	RP-251881_6G_WG_SID

[3]	S2-2504685 Summary on NWM discussion on 6G study

[4]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.870: "Study on 6G Use Cases and Service Requirements; Stage 1".

[7]	3GPP TR 22.841: "Study on Upper layer traffic steer, switch and split over dual 3GPP access".

4. Updates

The following is proposed as the starting point for drafting the WT and KI(s) for NTN based on the list of Tdocs:

| Focus on basic NTN access and service continuity (e.g. TN-NTN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20.6.0 | S2-2506226 | DISCUSSION | Discussion | 6G system architecture to support of NTN | THALES | Rel-20 | FS_6G_ARC | Moved from 20.6.0 |  |
| 20.6.7 | S2-2506437 | P-CR | Approval | 23.801-01: [WT#7] Scope and New Key Issues to Support 6G RAT for NTN | Nokia, T-Mobile USA, China Telecom, TNO, Thales, Vivo, ESA, IRDM, EchoStar, Boost Mobile Network, Viasat | Rel-20 | FS_6G_ARC |  |  |
| 20.6.7 | S2-2506591 | P-CR | Approval | 23.801-01: [WT#7, NTN] Scope and New Key Issue to Support 6G RAT for NTN | LG Electronics | Rel-20 | FS_6G_ARC |  |  |
| 20.6.7 | S2-2506740 | P-CR | Approval | 23.801-01: [WT#7, all topics] Study proposal for NTN | MediaTek Inc. | Rel-20 | FS_6G_ARC |  |  |
| 20.6.7 | S2-2506873 | P-CR | Approval | 23.801-01: [WT#7, Service Continuity] Scope and Key Issue for Service Continuity | Futurewei | Rel-20 | FS_6G_ARC |  |  |
| 20.6.7 | S2-2506940 | P-CR | Approval | 23.801-01: [WT#7] WT and KI to support NTN | Ericsson | Rel-20 | FS_6G_ARC |  |  |
| 20.6.7 | S2-2507231 | P-CR | Approval | 23.801-01: [WT#7, NTN] Key Issues for 6G NTN | Xiaomi | Rel-20 | FS_6G_ARC |  |  |
| Focus on basic NTN access and service continuity and in addition other aspects, e.g. CN NFs on -board. |  |  |  |  |  |  |  |  |  |
| 20.6.7 | S2-2506520 | P-CR | Approval | 23.801-01: [WT#7, NTNs] Non-Terrestrial Networks | InterDigital Inc. | Rel-20 | FS_6G_ARC |  |  |
| 20.6.7 | S2-2506978 | P-CR | Approval | 23.801-01: [WT#7, NTN] Clarification on 6G NTN work task scope | CSCN | Rel-20 | FS_6G_ARC |  |  |
| 20.6.7 | S2-2507080 | P-CR | Approval | 23.801-01: Discussion and proposal on key issue for 6G satellite access | CATT, Thales, TNO, Novamint, vivo | Rel-20 | FS_6G_ARC | Used as a baseline (Only WT scope) |  |
| 20.6.1.1 | S2-2507179 | DISCUSSION | Approval | Key Issue on NTN in 6G - Study proposal for Service Continuity aspects | NEC Corporation (ARIB) | Rel-20 |  | Moved from 20.6.1.1 Used as a baseline (Only KI scope) |  |
| Focus on Multi-orbit |  |  |  |  |  |  |  |  |  |
| 20.6.7 | S2-2507144 | P-CR | Approval | 23.801-01: [WT#07, all topics] Scope and key issue on NTN | NTT DOCOMO | Rel-20 | FS_6G_ARC |  |  |
| Impacts on UP due to satellite backhaul / feeder link |  |  |  |  |  |  |  |  |  |
| 20.6.7 | S2-2507308 | P-CR | Approval | 23.801-01: [WT#7, NTN] 6G support of NTN | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |  |  |

4.1 WTs for: Architecture to support satellite access in 6G

-	Architecture to support 6G satellite access. (CATT, Thales, TNO, Novamint, vivo - S2-2507080)

NOTE 1:	In clause 5.X 'Key Issue #X: Support for 6G satellite access' of S2-2507080, it is mentioned that how to support both transparent and regenerative payload based satellite access. (CATT)

NOTE 2:	In clause 5.X 'Key Issue #X: Support for 6G satellite access' of S2-2507080, it is mentioned that how to support RAN node (in regenerative payload mode) mobility efficiently. (CATT)

-	Study architecture enhancements needed for NTN, if any, based on the 6G RAT specified by 3GPP RAN. (Nokia, T-Mobile USA, China Telecom, TNO, Thales, Vivo, ESA, IRDM, EchoStar, Boost Mobile Network, Viasat - S2-2506437)

-	How 6G satellite access is supported in 6G system, considering the progress on WT#1. (Ericsson S2-2506940)

-	Investigate whether and how to re-use the 5G satellite specific functions (e.g. earth fixed tracking areas, differentiation of TN and NTN) in 6G, considering progress on the overall 6G architecture. (Ericsson - S2-2506940)

-	Study how 6G system supporting a new 6G RAT for NTN if it is designed by RAN WGs. (Xiaomi - S2-2507231)

-	Study how to support 6G RAT for NTN in 6G system. (LG Electronics - S2-2506591)

-	Study whether and how to enable deployment and selection of NFs on-board satellites. (InterDigital Inc. - S2-2506520)

-	Study which core network NFs are necessary to be deployed on satellites, with interface and procedure enhancements. (CSCN - S2-2506978)

NOTE 3:	In clause 5.X 'Key Issue #X: Support for 6G satellite access' of S2-2507080, it is mentioned that whether and how to support selected Core Network functions onboard satellites. (CATT)

NOTE 4:	In clause 5.X 'Key Issue #X: Support for 6G satellite access' of S2-2506940, it is mentioned that features requiring CN functionality onboard satellites are not included in the scope. (Ericsson)

Update Proposals:

Sub-WT 1: Study how to support 6G satellite access in 6G system, including whether and how to re-use the 5G satellite specific functions (e.g. earth fixed tracking areas, differentiation of TN and NTN).

Sub-WT 2: Study how to support efficient RAN node mobility for regenerative payload based satellite access.

Sub-WT 3: Study whether and how to support selected core network NFs onboard satellites.

4.2 WTs for: Support of NTN related capabilities specified in EPS/5GS

-	Gap analysis on support of NTN related capabilities specified in EPS/5GS, such as dynamic satellite backhaul, discontinuous network coverage, UE-Satellite-UE communication, and S&F. (CATT, Thales, TNO, Novamint, vivo - S2-2507080)

-	Mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance. (Huawei/HiSilicon S2-2507308)

Update Proposals:

Sub-WT 1: Investigate whether and how to support 4G/5G satellite specific capabilities, including:

-	The use of dynamic satellite backhaul on both control plan and user plane;

-	UE-Satellite-UE communication;

-	Discontinuous network coverage;

-	S&F.

4.3 WTs for: Support of new requirements for satellite access

-	Study on how 6G networks support service continuity when a UE moves to(/from) an NTN network from (/to) a TN network, as well as among satellites of NTN that are deployed at different orbits (e.g., LEO, MEO, GEO). (Nokia, T-Mobile USA, China Telecom, TNO, Thales, Vivo, ESA, IRDM, EchoStar, Boost Mobile Network, Viasat - S2-2506437)

-	Study how 6G system supporting the service continuity when UE moves between TN and NTN. (Xiaomi - S2-2507231)

-	Study how 6G system supporting the service continuity when UE moves among satellites that are deployed in differ orbits. (Xiaomi - S2-2507231)

-	How to support service continuity between (N)TN and NTN. (LG Electronics - S2-2506591)

-	Whether and how to enhance mobility and service continuity when a UE moves from a TN to an NTN and vice versa for inter-PLMN. (MediaTek Inc. - S2-2506740)

-	Study on how 6G networks shall support service continuity when a UE moves among satellites in a “constellation”. (NEC Corporation - S2-2507179)

-	Study whether and how to enable Service Continuity for various cases: TN-NTN, NTN-TN, NTN-NTN. (across various orbits). (InterDigital Inc. - S2-2506520)

-	Study whether and how to enhance network/orbit (re)selection and cell (re)selection in multi-orbit deployments. (InterDigital Inc. - S2-2506520)

-	Study whether and how to set up and maintain ISLs for multi-orbit deployments. (InterDigital Inc. - S2-2506520)

-	Support of new requirements for satellite access, including multi-orbit satellite access, service continuity, and Multi-connectivity via TN and NTN simultaneously (CATT, Thales, TNO, Novamint, vivo - S2-2507080)

-	Study how the 6G system can support service continuity and communication path switching across satellites in different orbits (e.g., LEO, MEO, GEO) based on service requirements and orbit characteristics. Investigate architectural enhancements to enable orbit-aware mobility and session management, considering the dynamic nature of non-terrestrial connectivity and varying latency profiles. (NTT DOCOMO - S2-2507144)

-	Study potential impacts on core network functions to support orbit-informed policy and QoS control and identify whether enhancements or new functions are needed. (NTT DOCOMO - S2-2507144)

-	Study aspects related to UE mobility between orbit types, where the 6G system needs to maintain service continuity while meeting QoS expectations in NTN-only or other access scenarios including 3GPP accesses in terrestrial network and non-3GPP satellite accesses. (NTT DOCOMO - S2-2507144)

-	Study the QoS framework for 6G NTN, considering dynamic characteristics of the satellite link. (CSCN - S2-2506978)

-	Study whether and how to enhance session continuity mechanisms and procedures to provide seamless service continuity with support for high bandwidth and reliability for a service as connections/sessions of TN and NTN change due to user mobility or other network conditions. (Futurewei - S2-2506873)

Update Proposals:

Sub-WT 1: Study on how to support service/session continuity in 6G system when a UE moves between a NTN network and a TN network, or across satellite access networks deployed at different orbits (e.g., LEO, MEO, GEO)

Sub-WT 2: Study on how to support multi-orbit satellite access, including:

-	Study whether and how to enhance 6G mobility management and session management other than service/session continuity aspect, e.g., network/cell (re-)selection, considering the dynamic characteristics of satellite access networks (e.g., varying latency profiles)

-	Study whether and how to enhance 6G policy and QoS control to support different orbits of satellite access.

-	Study whether and how to set up and maintain ISLs for multi-orbit deployments.

Sub-WT 3: Study whether and how to support multi-connectivity via TN and NTN or via 2 different NTNs (e.g. GEO and LEO based) simultaneously.

NOTE1:	The multi-connectivity here does not mean dual connectivity in RAN, it should be seen as a multi-access PDU session defined in 5GS.

NOTE 2: The solutions concluded for 6G to satisfy such new requirements may also be considered for 5G advanced.

4.4 WTs for: Adaption of new 6G features to satellite access

-	Study on adapting the AI framework to 6G NTN (e.g. delivery of AI services over satellite networks), considering constraints such as limited feeder link bandwidth, satellite mobility, and long propagation delays (CSCN - S2-2506978)

-	How AI can benefit to 6G satellite access (CATT, Thales, TNO, Novamint, vivo - S2-2507080)

-	Study on supporting computing services for 6G NTN, considering the dynamic characteristic of both network and computing resources (CSCN - S2-2506978)

-	Support of on board computing service for 6G satellite access (CATT, Thales, TNO, Novamint, vivo - S2-2507080)

Update Proposals:

Sub-WT 1: Study how AI can benefit to 6G satellite access, considering constraints such as limited feeder link bandwidth, satellite mobility, and long propagation delays.

Sub-WT 2: Study how to support onboard computing services for 6G satellite access, considering the dynamic characteristics of both network and computing resources.

**** First Change (all text new) ****

Annex A.X 	WT#7 Scope

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN

NOTE 2:	The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G.

Editor's Note:	Whether the improvement of backhaul via 6G satellite access is to be considered is FFS.

Editor's Note:	Whether any QoS/policy improvement is needed for satellite backhaul is FFS.

**** End of Changes ****