SA WG2 Meeting #171	S2-2508276

Wuhan, China, Oct 13 - 17, 2025

Source:	OPPO, ZTE

Title:	[WT#3.2, Network for AI] 6G architecture for AI

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the updates to WT and new KI on the 6G network for AI.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

1.1 AI token-based data transmission

There is a significant trend for the fast growth of token consumption [1]. AI traffic has some special characteristics and token is the most popularly used “data” for application AI traffic. As reported recently, in some countries, daily Token consumption has exceeded 30 trillion and continues to grow rapidly [2]. In the 6G era, with the proliferation of mobile AI, Token-based data transmission is destined to become a typical requirement and a new service modality that 6G needs to support.

Figure 1. An example for Token-based data transmission

Figure 1 illustrates a typical Token-based data transmission flow:

- UE Tokenizes the source information (e.g., an image) and then transmits the Token sequence to the NW side.

- The receiver de-Tokenizes the Tokens to recover the information, which is then fed into an AI service (e.g., a classification task).

For AI services supporting content recovery, the essential requirement is deterministic user experience/service performance, rather than always performing 100% reliable data transmission. Thus, the AI token is a typical scenario of error tolerant communication.

Observation-1: Token-based data transmission is widely applied to AI service. As a kind of error tolerant communication. By using token, the AI application can recover the original data with a high error rate during the data transmission.

The following description shows some simulations on Token-based data transmission, in which the ImageNet dataset with 3000 images belonging to 1000 categories (https://www.image-net.org) and a classification task [3] is used. Each image is converted into 256 Tokens (each Token is 12 bits), and the impact of token based transmission errors rate on the classification accuracy (Top-1/Top-5) is evaluated.

1.2 The generated token has different importance

Some advanced tokenizer can generate Token representations differentiated by importance. For example, the 256 Tokens can be numbered from 0 to 255 in descending order of importance; a lower number indicates higher criticality for information expression, while a higher number corresponds to detail information. In this case, as shown in figure 2a and 2b, there is a content recovery in two situations:

a) only transmit a number of most important tokens without error while abandoning the rest of tokens, and

b) transmit a number of most important tokens without error and transmit rest of tokens with error.

Original

Only Transmit the most important 64 tokens without error

Transmit the most important 64 tokens without error, and the other 192 tokens with 20% error rate

Figure-2a

Original

Only Transmit the most important 128 tokens without error

Transmit the most important 128 tokens without error, and the other 128 tokens with 20% error rate

Figure-2b

The two figures below show the roles played by token importance. Compared to transmitting all 256 tokens with randomly distributed token error rate in Figure-3a, the performance can be obviously better in Figure-3b, i.e., selecting and guaranteeing lossless transmission for certain number of important Tokens, while applying lossy transmission (allowing a certain error rate) to the rest of Tokens. This approach both ensures basic performance through the critical Tokens and allows flexibly enhancing accuracy through supplementary transmission, adapting to constraints like channel conditions and limited transmission resources.

,

Figure-3a, 256 tokens are transmitted with randomly distributed token error rate

Figure-3b, 256 tokens are transmitted with the 32 most important tokens transmitted

without errors and the rest of 224 tokens transmitted with errors

Observation-2: by leveraging tokenizer, one original data (e.g. a picture) can be converted to multiple tokens (e.g., 256 tokens) in which the tokens have different importance. For different scenario, the token size varies from several to tens of bytes.

1.3 Adaptive QoS for token-based data transmission

In addition to Image mentioned above, some KPIs of token has been evaluated and illustrated in the table-1 below with considering relevant references.

Table-1 example token size

| Token type | Payload Size | Error rate | Scenario |
| --- | --- | --- | --- |
| Video | 400 bits per token, 500 tokens per video frame [5] | 0.01% for important tokens. [20%] for other tokens | Realtime video processing |
| Image | 12 bit per token, 256 tokens per image [3] |  | The picture in ImageNet data set |
| Text | 8-24 bits per token [4] | 0.01% for all tokens | Human-chatbot communication |
| NOTE: The token size in the table is for example. the token size may vary from specific scenarios. |  |  |  |

Based on the simulation in figure-2 and 3, for some scenarios, guaranteeing the most important Tokens (e.g. top 25%) can achieve a high-quality recovery. This active selection based on Token importance, which involves discarding or downgrading QoS for non-important Tokens as needed, also helps build efficient transmission schemes, reducing the amount of data transmitted and improving efficiency.

Observation-3: A proper QoS control is needed to fulfill token characteristics (e.g., granularity), to deal with bandwidth limitation or network congestion.

Observation-4: tokenizer is applied differently based on data type, network and application conditions. The network needs to know when and which service data is transmitted via token and apply the corresponding adaptive QoS (e.g., a high transmission error rate) on demand.

Reference

[1]	Explaining Tokens — the Language and Currency of AI (https://blogs.nvidia.com/blog/ai-tokens-explained/)

[2]	China Reports Surge in Token Consumption as AI Applications Expand (https://technode.com/2025/08/19/china-reports-surge-in-token-consumption-as-ai-applications-expand/)[3] 	K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.

[3] 	K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.

[4] 	Z. Wu, et al, DeepSeek-VL2: Mixture-of-Experts Vision-Language Models for Advanced Multimodal Understanding, arXiv, Dec. 2024.

[5] 	Cheng, Yihua, et al. "{GRACE}:{Loss-Resilient}{Real-Time} video through neural codecs." 21st USENIX Symposium on Networked Systems Design and Implementation (NSDI 24). 2024.

1.4 summary and proposal

Observation-1: Token-based data transmission is widely applied to AI service. As a kind of error tolerant communication. By using token, the AI application can recover the original data with a high error rate during the data transmission.

Observation-2: by leveraging tokenizer, one original data (e.g. a picture) can be converted to multiple tokens (e.g., 256 tokens) in which the tokens have different importance. For different scenario, the token size varies from several to tens of bits.

Observation-3: A proper QoS control is needed to fulfill token characteristics (e.g., granularity), to deal with bandwidth limitation or network congestion.

Observation-4: tokenizer is applied differently based on data type, network and application conditions. The network needs to know when and which service data is transmitted via token and apply the corresponding adaptive QoS (e.g., a high transmission error rate) on demand.

Proposal: study error tolerant communication considering the AI traffic characteristics (such as token) in WT#3.2, with adding the following bullet

-   Study whether and how the 6G network apply adaptive QoS for AI application traffic considering the characteristics (e.g. AI token) via user plane, e.g., when traffic congestion happens.

**** First Change ****

Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g., AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.2 6G Network for AI:

-   Study whether and how to support the AI agent on UE to discover another AI agent on a different UE.

NOTE 1: Coordination with SA6 on the application layer aspects may be needed.

-   Study whether and how to enable communication for AI agents on different UEs via the 6G network.

NOTE 2: This AI agent is assumed in the UE but not part of the MT, but it can leverage MT functions. It is also assumed that any solution that suggested using UE to Core Network interaction is not replacing NAS protocol.

NOTE 3: The AI agent in WT#3.2 is outside of RAN and Core Network for 6G.

- Study whether and how the 6G network (except RAN) can provide AI inferencing and AI training as services to applications (AF or in UE).

NOTE 4: Coordination with SA3 and SA5 may be needed on AI inferencing and AI training.

NOTE 5: Coordination with WT#6 for computing may be needed.

NOTE 6: New service charging part may be coordinated with SA5 Charging group.

-   Study whether and how the 6G network apply adaptive QoS for AI application traffic considering the characteristics (e.g. AI token) via user plane, e.g., when traffic congestion happens.

NOTE X: Coordinated with SA1 on the AI traffic characteristics and coordinated with WT1.2 for QoS.

**** Second Change (all new text) ****

5.X Key Issue #X: 6G architecture for AI

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue is to study how 6G supports the AI agent communication, with considering the follow aspects:

-   	Whether and how to support the AI agent on UE to discover another AI agent on a different UE.

-   	Whether and how to support the identification of AI agent on UE in 6G.

-    Whether and how to support the authentication of AI agents on UE in 6G.

-	Whether and how to enable communication for AI agents on different UEs via the 6G network.

NOTE 1: Coordination with SA6 on the application layer aspects may be needed.

NOTE 2: This AI agent is assumed in the UE but not part of the MT, but it can leverage MT functions. It is also assumed that any solution that suggested using UE to Core Network interaction is not replacing NAS protocol.

NOTE 3: The AI agent in this KI is outside of RAN and Core Network for 6G.

-	Whether and how the 6G network (except RAN) can provide AI inferencing and AI training as services to applications (AF or in UE).

NOTE 4: Coordination with SA3 and SA5 may be needed on AI inferencing and AI training.

NOTE 5: Coordination with WT#6 for computing may be needed.

NOTE 6: New service charging part may be coordinated with SA5 Charging group.

-   	Whether and how the 6G network apply adaptive QoS for AI application traffic considering the characteristics (e.g. AI token) via user plane, e.g., when traffic congestion happens.

NOTE 7: Coordinated with SA1 on the AI traffic characteristics and coordinated with WT1.2 for QoS.

**** End of Changes ****