3GPP TSG-WG SA2 Meeting #171	S2- S2-2508523

Wuhan, CN, 13th – 17th Oct., 2025	(revision of S2-2507015)

Source:	Huawei, HiSilicon

Title:	[WT#5] WT and KI proposal for the data framework

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC / Rel-19

Abstract: This document discusses the related aspects of WT#5 data framework and proposes the update on working task and key issue proposal.

1. Discussion

As described in SP-251261, SA tasked SA2 and SA5 to progress and collaborate on 6G data related work tasks and taking the 5G working scope as the starting point, and set the checkpoint for regular coordination and collaboration. Based on this instruction, the study of data framework in SA2 can start now and coordinate with other WGs as planned in the future checkpoints.

In addition, S2-2508132 on WT scope reflect the discussion and guidance of SA2#170 meeting, in which there are three high level requirements of working task of data framework. This paper proposes the following updates:

-	Update the existing text. Including:

-	Modify the description regarding data sources. The examples are removed since this WT is to define the framework and all possible applications can be decided in e.g., WT#3 and #4 later on.

-	Remove the bullet on security and keep NOTE 4, since the user consent, privacy, security and data governance related topics are all SA3-lead topics and SA2 needs to coordinate with SA3' output in general.

-	Add a new bullet regarding the handling of multiple existing types of data, i.e., "Study how to support the data request that requires the handling of multiple existing types of data". The rationale has already been specified in Clause 1.2.4 of S2-2507015, to support the handling of a more complex task and deliver greater value via the data framework to the MNO. In summary, a vast amount of data will be collected and stored according to the scope of data framework, including but not limited with AI and sensing data. And, these collected data can be consumed by the data consumer for more valuable requirement.

-	Update the support of the quality of data. Since the fulfilment of the data requirement needs to consider the whole the life-cycle of the data (including data collection, data distribution, data processing etc.) it is proposed to remove the unnecessary limitation on "processing" part.

There were some comments raised in last SA2 meeting on sub-WT of study support on data quality, i.e. it needs to clarify the relationship with WT#1.2 on QoS. As explained in S2-2507015, data quality aims to indicate the requirement (e.g., freshness, accuracy) on different data handling aspects, including data collection, data distribution, data processing. However, the WT#1.2 QoS, of which the scope is for “traffic” handling instead of data handling. On the other hand, the scope of WT#1.2 is only for connectivity service but not consider Sensing or AI service. It is therefore clarified that study on data quality is different from study on QoS.

Regarding the KI proposal, S2-2507947 is a good summary of the companies' proposals, we think it can be further simplified based on the WT scope and this document include our proposals as in clause 5.X.1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change (All Text New) * * * *

A.X 	Work Task X: Data framework

The following is WT#5 in the SA2 6G SID:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

This Work Task focuses on developing a data framework for the handling of data and for supporting services in the 6G CN, including data and services such as AI, sensing and positioning.

The scope of the WT includes the following aspects:

a)	Design end to end 6G system architecture for data framework, e.g. defining new NF(s), new data plane/enhancing CP/UP and interfaces, to support efficient and scalable data handling functionalities, e.g.:

b)	data collection e.g., labelling data for origin, purpose, context and quality/validity;

c)	data transfer and distribution;

d)	data processing e.g. handling metadata such as type and structure of data parameters produced during data collection;

e)	data storage within 6G system;

f)	data access and

data exposure within 6G system and to outside of 6G system;

g)	data generation based on request;

NOTE 1:	Coordination with SA5 is required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 2:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 3:  Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

h)	Study from SA2 architectural and system level on user consent, privacy, and security to support the 6G data framework.

NOTE 4:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

i)	Study how to support quality criterion for the data based on the requirements from data requester (e.g., freshness, accuracy).

# * * * * Second change (All Text New) * * * *

5.X. Key Issue #X: Data framework

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

5.X.1	Description

This Key Issue focuses on developing a data framework for the handling of data and for supporting services in the 6G CN, including data and services such as AI, sensing and positioning.

How to support all aspects of data handling from the collection to storage to the distribution. Specifically the data framework functionality will include:

a)	Design end to end system architecture for data framework, e.g. defining new NF(s), new data plane/enhancing CP/UP and interfaces, to support efficient and scalable data handling functionalities, e.g.:

b)	Collection of data from one or multiple data providers, including the characteristics of the data (e.g., time, location) as per the requirement for the later handling. For example, labelling data for origin, purpose, context and quality/validity;

c)	Transfer and distribution of the data to one or multiple data consumers;

d)	Processing of the data on the transmission path; e.g. handling metadata such as type and structure of data parameters produced during data collection.

e)	Data discovery, storage, and retrieval of the required data of various types;

f)	Data requester authorization to access the data requested from the framework;

g)	Data exposure and the associating interactions with the entity(ies).

h)	Data generation based on request.

i)	Study how to support quality criterion for the data based on the requirements from data requester (e.g., freshness, accuracy).

NOTE 1:	Coordination with SA5 is required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 2:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 3:	Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

NOTE 4:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

NOTE 5:	WT that makes use of the data framework will address the way of using data framework by the respective WTs;

# * * * * End of change * * * *