3GPP TSG-SA2 Meeting #171	S2-2508540

Wuhan,China, October 13th -17th 2025

Source:	Intel

Title:	KI#4, Interim conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Interim conclusion for KI#4 – Sensing Data and the Associated Information Collection and Transport is proposed.

# 1. Introduction

This pCR proposes the interim conclusion for KI#4 – Sensing Data and the Associated Information Collection and Transport.

Based on the discussion presented in TDoc S2-2508539, it is proposed to include the following Agreement principles in TR 23.700-14.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all text new)* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

-  The configuration parameters sent from the SSMF to the selected SE(s) determine whether dedicated data tunnel will be established between the SE and the SDCF (Sensing Data Control Function) for sensing data transfer.

- If the configuration parameters include the SDCF endpoint information (e.g. SDCF ID/ FQDN), the SE will initiate the data tunnel with the SDCF to transfer the sensing data requested.

- If the configuration parameters does not include the SDCF end point information, the SE will send the sending data via the AMF to the SDCF.

NOTE: The details on the protocol for data tunnel between SE and the new network function in the core network is to be determined in coordination with the RAN3 WGs.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

# * * * * End of changes * * * *