3GPP TSG-WG SA2 Meeting #171	S2-2508811

Wuhan, China, October 13 - 17, 2025

Source: 	Interdigital

Title: 	Proposal for Interim Agreements on KI#2

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: proposal for interim agreements on KI#2.

# 1	Introduction

This contribution provides the interim agreements on KI#2 on Authorization and Revocation to Support Sensing Service.

# 2 	Proposal

It is proposed to adopt the following changes into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.2	Agreed Principles for KI#2

An AF as sensing service consumer is manged with an authorization information for sensing service based on the service level agreement on the sensing service with a mobile operator.

The authorization information for AF includes the list of sensing service type, and for each sensing service type, allowed sensing service quality level (e.g., granularity, confidence level, etc), allowed target sensing service area, and allowed time condition are included.

When the requested sensing service type is not available at the target sensing service area (e.g., because of no available sensing entities), the sensing service request is rejected.

After successful authorization of sensing service request, a sensing operation ID is assigned to identify any sensing operation and the sensing result associated to the authorized sensing service request.

For each sensing function, it is assumed that the list of gNBs as authorized sensing entities at the sensing service area of the sensing NF and the capability of each gNB as a sensing entity are preconfigured at the sensing function.

NOTE: How to configure the capability of a gNB as a sensing entity is up to RAN WG.

# * * * * End of change* * * *