SA WG2 Meeting #171	S2-2509194

Wuhan, China, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	Interim agreements for FS_Sensing_ARC KI#1

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This contribution proposed the interim agreements for KI#1 of ISAC study.

# 1.  Discussions

Based on the discussion in the SA2 and the solutions documented in TR 23.700-14, this contribution proposed the interim agreements for KI#1 System Architecture to support Sensing.

# 2.  Proposals

To apply the following change to the TR 23.700-14.

**** First Change ****

### 7.1.Y	Agreed Principles for KI#1

The following principles are agreed in this release to address the key issue on System Architecture to support Sensing:

- 	a core network NF, i.e. Sensing Function, is introduced to support sensing operation;

- 	a direct connection between Sensing Entity, i.e. gNB, and the Sensing Function is introduced to support sensing;

-	the direct connection supports both sensing service control signalling and sensing data transport;

NOTE 1: 	the protocols used for the direct connection need to be determined based on RAN WGs and CT WGs feedback.

-	the Sensing Function may be further separated into two logical sub-functions, i.e. Sensing Control Function, and Sensing Processing Function, and the interface between the two sub-functions are implementation-based and not specified by 3GPP;

-	the direct connection is established by the Sensing Entity to the Sensing Function based on OAM configurations;

-	in this release, Sensing Function is independent of LCS framework.

NOTE 2: 	for sensing service request that includes a UE ID, the Sensing Gateway Function needs to subscribe to LCS service using the UE ID, and initiates one or more sensing service sessions towards the Sensing Function(s).

**** Next Change ****

### 7.2.Z	Topics for further consideration for KI#1

The following topics requires further considerations and would not be pursued in normative phase of this release:

- 	a standalone NF with standardized interface for sensing data storage and exposure.

**** End of Changes ****