3GPP TSG-WG SA2 Meeting #171	S2-2509195

Wuhan, China, 13th Oct – 17th Oct, 2025

Source:	HONOR

Title:	KI#2 new solution: AF activates/deactivates the DO-A traffic to the AIoT device

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: This solution proposes the DO-A traffic configuration procedure, as well as the activation/deactivation procedure for DO-A traffic.

1. Introduction

This contribution proposed the solution related to the KI#2.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-30.

# * * * * First change * * * *

6.X	Solution #x: AF activates/deactivates the DO-A traffic to the AIoT device

6.x.0	High-level solution Principles

This solution addresses KI #2. It includes DO-A parameters configuration for DO-A traffic, the procedure of the AF initiates or stops the AIoT device reporting the DO-A data (e.g., sensing data) via the Activation/Deactivation Command.

6.x.1	Description

This solution is applicable to both the Direct Connectivity and the Indirect Connectivity architectures as defined in TS 23.369.

The principles of this solution are as follows:

- The AIoT device registration: For an Ambient IoT devices of type 2b/c supporting DO-A traffic, the registration-like procedure should be performed before the DO-A traffic configuration and sending DO-A data.

- The DO-A configuration of the DO-A traffic: The AF sends the configuration parameters of DO-A traffic to the AIoT device via Write Command.

- The DO-A data reporting method: DO-A data (e.g., sensing data) reporting method of the AIoT device includes periodical reporting (i.e. reporting with the specific period) and non-periodical reporting (i.e. event-triggered DO-A traffic, reporting when meets configured event. ).

- The DO-A traffic is initiated or stopped by the AF request: The AF initiates or stops the device reporting the DO-A data via the Activation/Deactivation Command separately.

6.x.2	Procedures

The procedure is shown in Figure 6.x.2-1. It includes the AIoT Service Request from AF to configure the parameters of the DO-A traffic, Activation/Deactivation Command request from AF to initiate or stop the DO-A traffic.

Figure 6.x.2-1: Procedure for DO-A traffic configuration and AF activates/deactivates DO-A traffic

0.	The AIoT devices should perform the registration-like procedure to indicate the support of DO-A communication. Optionally, the AIOTF could indicate the presence of the AIoT device to corresponding AF.

Editor’s note: The AIOTF could know the presence of the device via registration-like procedure initiated by the AIoT device or the AF. The details of the device registration-like procedure are FFS.

- Step 1 to Step 6 of Command Procedure in clause 6.2.3 of TS 23.369 with write command type and additional DO-A configuration parameters.

The AF sends Nnef_AIoT_Command Request message to the NEF to configure the AIoT device for DO-A traffic. Nnef_AIoT_Command Request ( Write Command type, the offset where to write DO-A Configuration Parameters, DO-A Configuration Parameters, [the offset where to store the DO-A data (e.g., sensing data) in the AIoT device], [the length of the DO-A data sent by the AIoT device] ). Excepting the above parameters for DO-A traffic, the Nnef_AIoT_Command Request also includes other necessary parameters specified in clause 6.2.3 of TS 23.369.

The DO-A Configuration Parameters includes DO-A Period for reporting DO-A traffic if it is periodical DO-A traffic, DO-A Event (e.g., temperature threshold for the temperature sensor device) for reporting DO-A traffic if it is event-triggered DO-A traffic.

NOTE 1: 	The offset where to store the DO-A data in the AIoT device and the length of the DO-A data sent by the AIoT device are optional IEs and could also be pre-configured in the AIoT device.

NOTE 2:	There could be multiple sets of DO-A Configuration Parameters (e.g., Set A: DO-A Period=60s, Set B: DO-A Period=3600s).

The NEF selects the AIOTF and sends the Naiotf_AIoT_Command Request message to the selected AIOTF. The AIOTF checks the AF request and device capability to perform configuring the AIoT device for DO-A traffic.

- This step is the procedure for Inventory in clause 6.2.2 of TS 23.369. It could be optionally performed if a NGAP connection for the AIoT device between the AIOTF and the NG-RAN exists (e.g., if the NGAP connection is established in AIoT device registration-like procedure.).

- The AIOTF sends AIoT Command request to the NG-RAN. The AIOTF will encapsulate the DO-A Configuration Parameters in an AIOT NAS PDU. Some DO-A configuration parameters may be visible to the NG-RAN to assist resource scheduling in RAN and the AIOTF will list them in NGAP message IE (e.g., the length of the DO-A data, DO-A Period).

Editor’s note: Whether visible parameters are needed and what visible parameters are needed depend on RAN WGs.

- The NG-RAN sends AIoT Command request to the AIoT reader over the RRC connection, if the AIoT reader is a UE reader.

- The AIoT reader sends AIoT Command request to the AIoT Device. When the AIoT device receives DO-A Configuration Parameters, it sets the parameters to the device.

- The AIoT device sends AIoT Command response to the AIoT reader to indicate DO-A Configuration Parameters are set successfully in the AIoT device.

- The AIoT reader sends AIoT Command response to the NG-RAN over RRC connection, if the AIoT reader is a UE reader.

- The NG-RAN sends AIoT Command response to the AIOTF to indicate the successful configuration in the AIoT Device. After this step, the NG-RAN could keep the connection as a long-term connection with the AIOTF until the terminate of the DO-A traffic.

- The AIOTF reports the result of the AIoT Command request to the NEF.

- The NEF informs the AF the result of the AIoT Command request. The DO-A traffic configuration procedure is complete.

Additionally, the AF can initiate or stop the AIoT device DO-A traffic via the Activation/Deactivation Command request in step 11 ~ step 23.

- The AF sends Nnef_AIoT_Command Request message (Activation/Deactivation Command type, DO-A configuration set info, target AIoT device info) to the AIoT device via the NEF to initiate or stop the configured DO-A traffic. DO-A configuration set selection info is the optional IE and this information is only presented if there are multiple sets of DO-A Configuration Parameters.

Excepting the above parameters, the Nnef_AIoT_Command Request also includes other necessary parameters specified in clause 6.2.3 of TS 23.369.

- The NEF selects the AIOTF and sends the Naiotf_AIoT_Command Request message (Activation/Deactivation Command type, target AIoT device info) to the selected AIOTF.

- The AIOTF checks the AF Authorization and device capability to perform DO-A traffic.

- The AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, cause) to the NEF.

- The NEF sends the Nnef_AIoT_Command Response message (accept or reject, cause) to the AF. If the response was a reject the procedure stops here.

- The AIOTF sends AIoT Command request, including correlation ID and AIOT NAS PDU (the activation/deactivation indication), to the NG-RAN.

NOTE 3: After step 8, the NG-RAN keeps the connection as a long-term connection with the AIOTF so that the inventory procedure is not presented.

Editor’s note: Whether the activation/deactivation indication is useful and visible to the NG-RAN is FFS.

- The NG-RAN sends AIoT Command request, including AIOT NAS PDU (the activation/deactivation indication), to the AIoT reader over the RRC connection, if the AIoT reader is a UE reader.

- The AIoT reader sends AIoT Command request, including AIOT NAS PDU (the activation/deactivation indication), to the AIoT Device.

- The AIoT Device sends AIoT Command response, including AIOT NAS PDU (the activation/deactivation result), to the AIoT reader to indicate DO-A traffic is initiated successfully or stopped successfully.

NOTE 4:	After sending the AIoT Command response, if the command type in step 13 is Activation, for the periodical DO-A traffic the AIoT device begins to calculate the sending timing according to the configured DO-A period.

Editor’s note: How the AIoT Device supports timer is FFS.

- The AIoT reader sends AIoT Command response, including AIOT NAS PDU (the activation/deactivation result), to the NG-RAN over the RRC connection, if the AIoT reader is a UE reader.

- The NG-RAN sends AIoT Command response, including AIOT NAS PDU (the activation/deactivation result), to the AIOTF.

- The AIOTF sends AIoT Command response to the NEF to indicate the result of activation/deactivation command.

- The NEF sends AIoT Command response to the AF to indicate the result of activation/deactivation command.

After the activation procedure, the AIoT device generates the DO-A Data and send DO-A data with the AIoT device ID (e.g., AIoT device temporary ID) according to DO-A Configuration Parameters.

Editor’s note: The details procedure of the routing of the AIoT device DO-A data is FFS.

Editor’s note: It is FFS for whether there is response from the CN to the AIoT device.

After the deactivation procedure, the AIoT device stops to send the DO-A data until the AF sends activation command request.

6.x.3	Impacts on services, entities and interfaces

Impacts on existing entities:

AIOTF:

-	Receives DO-A Configuration Parameters for DO-A data reporting from AF.

-	Performs Inventory procedure and then sends Write Command Request with DO-A Configuration Parameters.

-	Receives DO-A traffic Activation/Deactivation Command request from AF and sends activation/deactivation indication to the AIoT device via AIOT NAS message.

-	Receives DO-A data (e.g., sensing data) from AIoT device(s) through AIoT reader(s) and delivers it to the AF through the NEF or not.

ADM:

-	Supports the AIoT device profile, including DO-A Capability.

NG-RAN:

-	Supports DO-A procedure for AIoT Device(s).

-	Supports to keep the connection as long-term connection with the AIOTF after the DO-A traffic configuration procedure.

AIoT reader:

-	Supports DO-A procedure for AIoT Device(s).

AIoT Device:

-	Supports DO-A procedure for DO-A data report.

-	Supports to set DO-A Configuration Parameters and behave according to DO-A Configuration Parameters.

-	Supports timer.

# * * * * End of changes * * * *