3GPP TSG-SA WG2#171	S2-2509270

Wuhan, China, 13-17 October 2025	(was S2-2508786)

Source:	Qualcomm Incorporated

Title:	Update of Solution #2: NW initiated data channel application in early media session with subscription

Document for:	Discussion/Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: Update Solution #2 to address the open issues and optimize the procedures in supporting NW initiated IMS data channel application in early media session.

# 1.	Discussion

Proposes changes in the solutions to support NW initiated IMS data channel application in early media session.

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-56:

# >>>>BEGINNING OF CHANGES<<<<

## 6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription

### 6.2.1	High level principles

This solution has the following high level principles, in order to support IMS data channel application in early media session:

-	Data Channel Application in Alerting Phase (DCAP) service can be configured by the user and stored in the HSS as part of the subscription data.

-	IMS AS triggers the application data channel establishment based on UE subscription data when receives INVITE from  remote UE.

-	IMS AS performs as a B2BUA to update the SDP for data channel in early session and media flow for the main session.

-	IMS AS activates the application data channel upon receiving 180 Ringing from the serving UE.

-	The application data channel may coexistent with other early session services, e.g. CAT, CRS.

### 6.2.2	Description

This is a solution for Key issue #1 " DC establishment in alerting phase for early media".

The user of UE#2 is able to subscribe to the Data Channel Application in Alerting Phase (DCAP) service, including activate (or de-activate) the service, and update the settings, e.g. to change by configuration the active data channel application. The DCAP subscriber is able to refine the data channel application selection behaviour with configured rules, e.g. time, calling party's location, called party's location, the identity of the calling and called party. The DCAP service is able to select the appropriate data channel application according to the rules.

DCAP is a terminating network service, but can also have an originating network functional component. That is, DCAP can be selected on behalf of the called subscriber for presentation to the calling party, but the calling (IMS) subscriber can also subscribe to and activate the DCAP service.

### 6.2.3	Procedures

The overall procedures of data channel establishment in early media phase are illustrated in figure 6.2.3-1.

Figure 6.2.3-1: ADC establishment in early media phase

0.	The user of UE#2 subscribes Data Channel Application in Alerting Phase (DCAP) service to his/her serving IMS network. The data channel application to be initiated in alerting phase is selected by the user and configured in UE#2's subscription data.

1.	UE#1 sends a SIP INVITE targeting UE#2. The initial SDP contains offers for audio/video flow, bootstrap data channel. UE#1 indicates support for preconditions, reliable provisional responses, and indication of early-session supported.

2.	Base on UE#2’s subscription data for DCAP service, the IMS AS decides to trigger the pre-configured data channel application to UE#1 in early media session. The IMS AS sends Nimsas_SessionEventControl_Notify to the selected DCSF to notify the session establishment, including the application data channel (inactive) and bootstrap data channel. The IMS AS may add the remote bootstrap data channel to UE#1 (BDC from UE#1 to the DCSF in the terminating network) if not presented.

3.	The DCSF decides whether the requested data channels are allowed and determines the control policy and media information for the application data channel and bootstrap data channels.

4.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction instructing the IMS AS how to set up the application data channel and bootstrap data channels with MF both for originating and terminating side.

5.	The IMS AS instruct the MF to allocate required media resources for the application data channel and bootstrap data channels. The IMS AS indicates to the DC AS that the media stream of ADC is inactive.

6.	The IMS AS responds to the MediaInstruction request received in step 4.

7.	The DCSF indicate the DC AS reserving the media resource for the application data channel and MDC2 end points to the MF. The DCSF indicates to the DC AS that the media stream of ADC is inactive.

8.	The DCSF responds to the Notify Request received in step 2.

9.	The IMS AS sends the INVITE to UE#2.

10.	UE#2 determines the subset of the media flows and responds with an Offer Response message back to the IMS AS.

11. The IMS AS forwards the Offer Response message to UE#1.

12.	The IMS AS initiates an UPDATE message to UE#1, which includes the SDP for the and remote bootstrap data channel and inactive application data channel for early session. The IMS AS may add a remote bootstrap data channel with UE#1 if not request in the INVITE message.

13.	Based on UE implementation and local policy, UE#1 may alert the user that an application data channel in early media is triggered. The user can decide to accept or reject the application.

14.	 If accepted by the user, UE#1 reserves the media resource for the application data channel and bootstrap data channel. UE#1 sends the 200 OK, which acknowledges the UPDATE message to the IMS AS.

15. The BDC between UE#1 and the DCSF is established as part of the SDP Offer in the early session. If the requested data channel application is not locally available, UE#1 downloads the data channel application via the established BDC and update the early session with the IMS AS.

16.	The IMS AS forwards the PRACK message to UE#2.

17-18.	UE#2 reserves resource and sends a SIP 200 (OK) response for the SIP PRACK request to UE#1. The application data channel between the DC AS and UE#1 is established.

19.	UE alerts the user and sends Ringing indication to the IMS AS.

20.	The IMS AS decides to activate the pre-configured data channel application to UE#1.

21. The IMS AS instructs the MF to activate the media resource for the application data channel. The IMS AS may notify the DC AS to activate the application towards UE#1..

22.	The IMS AS updates the IMS session  with UE#1 to activate the application data channel in early session.

23.	 UE#1 confirms the activation of the application data channel.

243.	The interaction between UE#1 and the DC AS via application data channel in early session is started.

25. UE#2 accepts the session and sends 200 OK to the INVITE.

26-27.	The IMS AS updates UE#1 with SDP offer to initiate the media flow of the main session and terminate the application data channel in the early session.

28.	The IMS AS instructs the MF to update the media resource and terminate the application data channel.

29-30. The IMS AS forwards 200 OK to UE#1. UE#1 sends ACK to UE#2.

31. The media flow of the main session between UE#1 and UE#2 starts. The requested BDCs for both UE#1 and UE#2 in the main session are established.

### 6.2.4	Impacts to Services, Entities and Interfaces

UE:

-	Indicates supporting of early media session in SIP INVITE message, as specified in existing specification, e.g. TS 24.229 [6].

-	Supports application data channel in the early session.

-	Notifies user the requested data channel application in early session.

HSS:

-	Supports management of user subscription for Data Channel Application in Alerting Phase (DCAP) service.

-	Supports the user configuration of data channel application(s) in alerting phase.

IMS AS:

-	Triggers the application data channel establishment when receiving INVITE from the originating UE.

-	Activates the established application data channel when receiving 180 Ringing from the terminating UE.

-	Terminates the application data channel when the call accepted by the terminating UE.

-	Performs as B2BUA to update the media flow in the SDP exchanged with UE.

# >>>>END OF CHANGES<<<<