3GPP TSG-WG SA2 Meeting #171	S2-2509444

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Title:	[DRAFT] LS on UE Reader Capabilities

Response to:

Release:	Release 20

Work Item:	FS_AmbientIoT_ARC_Ph2, Ambient_IoT_Solutions_Ph2

Source:	[Huawei to be] SA2

To:	RAN2

Cc:	RAN3

Contact Person:

Name:	Runze Zhou

Tel. Number:

E-mail Address:	zhourunze AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	link of TR 23.700-30

1. Overall Description:

SA2 is starting to conclude R20 Ambient IoT study for key issue #1: support AIoT services under the RRC-based option for UE Reader connectivity and will document the agreements in TR 23.700-30.

One aspect of the overall conclusion is on UE reader authorization and SA2 has agreed the following principles:

- UE Reader subscription information is available to AMF

- If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

During the discussion, SA2 understands in addition to UE Reader subscription information, AMF also need to obtain certain UE reader capability, in order to authorize the UE to operate as a UE reader.

Two options are discussed as following, on what the UE reader capability is and how the AMF retrieves the UE reader capability.

Option 1:

SA2 will define a new UE MM Core Network Capability, to indicate the capability to act as UE reader.

UE sends the UE MM Core Network Capability (i.e. capability to act as UE reader) to AMF via NAS signalling.

When AMF receives the UE reader capability, together with the UE reader authorization information in the UDM, AMF decides whether to authorize the UE to be the UE reader or not. If authorized, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Option 2:

Assuming a new UE radio capability for AIoT service to indicate e.g., the supported frequency bands by the UE reader to be defined by RAN2.

UE sends the radio capability to NG-RAN. NG-RAN further provides an indication to AMF whether the UE can currently operate as a reader depending on supported bands.

If RAN hands the UE over to a band in which the UE does not support operating as a UE reader, RAN indicates to AMF that UE cannot operate as a UE reader.

If RAN hands the UE over to a band in which the UE does support operating as a UE reader, RAN indicates to AMF that UE can operate as a UE reader.

AMF authorizes the UE to operate as a UE reader based on the indication from NG-RAN and the subscription information in the UDM (i.e. whether UE can act as reader), and provides the authorization information to NG-RAN.

(Question to be asked, general)

SA2 kindly asks RAN2 to evaluate whether option 2 is feasible from RAN point of view.

(Question to be asked, specific)

SA2 would like to ask the following question to RAN2:

Q1: Whether RAN2 will define UE radio capability for AIoT service, indicating e.g. the supported frequency bands by the UE reader? Q2: If the answer to Q1 is yes, is the UE reader radio capability signalled from UE to NG-RAN?

SA2 will finalize the R20 study in Feb meeting 2026, therefore SA2 hopes RAN2 can provide feedback in November meeting, 2025.

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RAN2 to provide feedback on the above questions.

Guidance – Provide clear information on what the other groups must do, and by when

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #172		17th Nov – 21st Nov, 2025		Dallas, US

TSG-SA2 Meeting #173		09th Feb – 13th Feb, 2026		India, IN