SA WG2 Meeting #171	S2-2509622

13 – 17 October 2025, Wuhan, China                 (revision of S2-2509601)

Source:	Samsung

Title:	KI#2: Solution for capability exposure of IMS DC events supported by DCSF

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#2.

# 1. Discussion

This paper proposes solution for KI#2 in TR 23.700-56 for the SID NG_RTC_Ph3_ARC, about capability exposure of IMS DC events supported by DCSF

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #X |  | X |  |

## 6.X	Solution #X: Capability exposure of IMS DC events supported by DCSF

### 6.X.0	High-level solution Principles

This solution addresses Key Issue #2: Capability exposure of IMS DC events supported by DCSF.

The main principle of this solution for support capability exposure of IMS DC events supported by DCSF as follows

-	The trusted AF or untrusted AF via NEF shall discover and select the DCSF.

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of supported events with DCSF and get notified when DCSF detects the events.

Editor's note:	This solution involves DCSF during registration of IMPU while it was invoked during session earlier. The implications of this approach is FFS.

### 6.X.1	Description

This solution proposes how the trusted AF and untrusted AF via NEF discovers the DCSF and subscribes to get notification for the supported events by DCSF.

### Discovery of DCSF:

The trusted AF can directly discover the DCSF with the help of HSS and in case of untrusted AF, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. Once HSS comes to know the DCSF address, it notifies the NEF/AF.

### Subscription and Notification with DCSF:

Upon successful discovery, AF/NEF shall subscribe with DCSF for the supported events and get notified when DCSF detects the event.

Table 6.X.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Successful Application download to UE | Detected when an Application Data Channel is established. DCSF detects that UE has been successfully downloaded one specific application | DCSF |

### 6.X.2	Service definition

#### 6.X.2.1	General

The following table shows the required service operation.

#### 6.X.2.1.1	HSS Service

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
| Management (_ImsUECM) | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfoGet | Subscribe/Notify | NEF, AF |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, AF |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, AF |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

#### AA.2.1.2.1	Nhss_ImsUECM_Registration service operation

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id is the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

##### 6.X.2.1.1.1	Nhss_ImsUECM_DcsfInfoGet service operation

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.1.1.2	Nhss_ImsUECM_DcsfUnsubscribe

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.1.1.3	Nhss_ImsUECM_DcsfNotify

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.X.2.1.2	DCSF Service

#### 6.X.2.1.2	Ndcsf_SessionEvent

##### 6.X.2.1.2.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

##### 6.X.2.1.2.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event ID is currently defined: successful download of specific application by UE.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application ID

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.X.2.1.2.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.1.2.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, DC Application ID.

Inputs, Optional: Calling party and called party ID.

Outputs, Required: Result.

### 6.X.3	Procedure

#### 6.X.3.1	DCSF instance registration in HSS

Figure 6.X.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.X.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS performs third party Registration with the DCSF instance assigned to the registering UE. This is over http and not SIP.

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE 1:	Figure 6.X.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.X.3.2	DCSF address discovery by AF

Figure 6.X.3.2-1depicts a signalling flow diagram for discovering DCSF address by AF.

.

Figure 6.X.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as per Fig1 solution.

2.	AF discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5. 	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to AF.

Editor's note:	It is FFS how to address the scenario where the IMPU which is not registered or deregistered but queried by DC AS.

#### 6.X.3.3	Subscription for IMS DC events supported by DCSF

Figure 6.X.3.3-1depicts a signalling flow diagram for subscriptions for IMS DC events supported by DCSF.

Figure 6.X.3.2-1: DCSF address discovery by DC AS

1.	AF subscribes with NEF using Nnef_ImsEE_Subscribe request.

2.	NEF provides Nnef_ImsEE_Subscribe Response.

3.	Based on the requested events, NEF decides that the request need to be sent to DCSF. It is assumed that NEF has discovered DCSF based on 6.X.2. NEF then sends the Ndcsf_ImsEE_Subscribe request with IMPU and Application ID, event ID as input.

4.	DCSF provides Ndcsf_ImsEE_Subscribe Response.

5. 	DCSF detected that the application which was subscribed by NEF has been downloaded by the UE.

4.	DCSF sends Ndcsf_ImsEE_Notify to NEF.

7.	NEF sends Nnef_ImsEE_Notify to AF.

### 6.X.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

AF/NEF:

-	subscribing with DCSF for specific application download by UE event and get notified when the event is detected by DCSF.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for specific successful application download event from NEF or AF and notifying when UE successfully downloads the application.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

# * * * * End of changes * * *