SA WG2 Meeting #171	S2-2508221

13 – 17 Oct, 2025, Wuhan, China	  (revision of S2-2508121)

Source:	CATT

Title:	[WT#1.2, Enhanced User Plane] WT Scope and Key Issue for Enhanced User Plane

Document for:	Approved

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This update contribution discusses areas to study for enhanced user plane that enable high scalability for user plane transport and functional entities.

- Justifications

1.1 Motivation on user plane

In order to meet the requirements of multi types data, sudden traffic transmission, new services and new capabilities, 6G user plane needs to enhance to meet the requirements of user plane path selection, application service awareness, and efficient data transmission.

In order to transmit data more efficiently and flexibly, the user plane function should enhance the protocol and select the protocol as needed based on the data type, e.g., SRv6. Compared to GTP-U, SRv6 has more advantages, e.g., more flexible network strategies, path optimization, and programmability, making path selection simpler, more automated, and efficient. In the 6GS, UPF will not only have traditional data routing and transmission capabilities, but also new capabilities(e.g. AI and computing). In order to meet the requirements of different NF  introduced for new capabilities and the reuse of 5G NF to invoke various services of UPF, the N4 interface between UPF and SMF needs to be using SBA.

1.2 Gap Analysis:

The 3GPP define multiple PDU session types for UE, mainly including IPv4, IPv6, IPv4v6, Ethernet, and Unstructured. Different session types represent different protocols carried by the user plane, but the protocol applied to data transmission only is the GTP-U protocol. Different session types refer to the underlying protocols carried by the GTP-U protocol, such as IPv4, IPv6, or Ethernet, so the user plane transmission protocol is relatively fixed and single. 3GPP has discussed replacing the GTP-U protocol with SRv6, but due to certain reasons, the proposal to apply the SRv6 protocol to user plane data transmission has not been approved. The unstructured session type refers to a type of open user plane data transmission protocol that does not restrict specific user plane data transmission protocol types, but provides certain protocol customization or privatization. This is also the initial attempt of 3GPP to dynamically evolve and enhance user plane protocols.

The support of event exposure service in R18 UPF is an important beginning of SBA over N4 , but the interface of N4 in the current  only exposures subscription/notification services to AF/NEF, and the N4 interface needs to enhance on SBI.

1.3 Proposal on Technical Impact:

Enhancement on the user plane architecture to support more flexible, efficient user plane traffic transfer and handling for 6GS:

- Enhancement on N4 using SBI to transmitter information with CN NF(e.g. connectivity services, beyond connectivity services).

- Replace GTP-U with SRv6 to support transmission user traffic more efficient.

- Flexible user plane path selection for beyond connectivity capabilities or services(e.g. AI, computing, data).

- Enhancement on user plane new functionalities or capabilities.

- At the SA2#170 meeting in Gothenburg Sweden, the S2-2508121 as the baseline of supporting enhanced user plane architecture was postponed. This paper mainly update the WT scope base on the S2-2508121 and gives the KI(s) on   enhanced user plane architecture based on the SA2#170 progress.

- The main content of the update is that UPF will have new services or capabilities. 6G has proposed six major scenarios and higher requirements for various indicators of network performance. e.g., peak data rate, user experience data rate, regional traffic capacity and connection density, etc. From the above scenarios and KPI requirements, it can be seen that it is closely related to the ability of UPF. Compared with 5G UPF, 6G UPF will have new services or capabilities to satisfy the above requirements.

**** First Change ****

Annex A.X. WT#1.2 User Plane Architecture for 6G Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

[WT#1.2 UP Architecture] In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- 1) Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to provide the network with information that allows the network to adapt to application traffic.
Editor’s Note: whether this issue belongs to UP or QoS is FFS.

2) Whether and how to enhance CP-UP interaction for better interoperability.

3) Whether and how to extend the service-based architecture for the user plane control interface to UPF.

4) Whether and how to support flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.

5) Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

6) Whether and how to enhance the user plane handing for new capabilities or services(e.g., AI, Computing) in UPF.

**** Second Change ****

- X. Key Issue #X: Enhanced User Plane Architecture for 6G

Study the enhancement on the user plane architecture to support more flexible, efficient user plane traffic transfer and handling for 6GS, the following aspects are to be studied:

- Whether and how to enhance the Architecture, NFs and high-level procedures to support flexible and efficient user plane path selection.

- Whether and how to support  full SBI over N4 .

- Whether and how to support replacing GTP-U with SRv6.

- Whether and how to support  new functionalities or services in UPF.

**** End of Changes ****