3GPP TSG-WG SA2 Meeting #171	S2-2508236

Wuhan, CN, 13th – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion for KI#1

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#1 on System Architecture to Support Sensing.

# 1. Introduction

Based on the discussions and analyses in S2-2508235, this paper provides the conclusion on KI#1 on System Architecture to Support Sensing.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * * all new text

### 7.1.1	Agreed Principles for KI#1 System Architecture to Support Sensing

The reference architecture in figure 7.1.1-1 is proposed to be adopted for KI#1.

Sensing functionalities are supported by the following two newly introduced NFs:

-	Sensing Control Function (SCF);

-	Sensing Processing Function (SPF);

The Sensing Control Function supports the control functionalities for the sensing services defined in detailed in the KI conclusions in the clauses 7.1.2, 7.1.3, 7.1.4, 7.1.5 and 7.1.6.

The Sensing Processing Function supports the generation of Sensing Result from the received Sensing Data from gNB under the control of SCF as defined in detailed in the KI conclusions in the clauses 7.1.3, 7.1.4 and 7.1.5. The SPF provides the Sensing Results to the Sensing Control Function as described in agreed principles in clause 7.1.4 and 7.1.5.

Figure 7.1.1-1: 5G System Architecture for Sensing.

NOTE 1:	Protocol used over NS1 reference point will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

NOTE 2:	The protocol stack used between the Sensing Control Function and the gNB will be concluded by RAN WG3.

NOTE 3:	NS3 denotes the reference point between the SCF and the SPF. The interface details are not specified and leave to implementation in this release.

The 5G System Architecture for supporting Sensing service contains the following service-based interfaces:

Nscf:	Service-based interface exhibited by SCF.

The 5G System Architecture for supporting Sensing service contains the following reference points:

NS1:	Reference point between the gNB and the SCF.

NS2:	Reference point between the gNB and the SPF.

NS3:	Reference point between the SCF and the SPF.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.1 are agreed as the conclusion.

# * * * * End of changes * * * *