3GPP TSG-WG SA2 Meeting #171	S2-2508242

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

# Title:	[DRAFT] LS on status of Sensing study

# Response to:

# Release:	Rel-20

# Work Item:	FS_Sensing_ARC

Source:	[Huawei to be] SA2

To:	RAN3

Cc:	SA3, SA6

Contact Person:

Name:	Marco Spini

Tel. Number:

E-mail Address:	marco DOT spini AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	 TR 23.700-14 v0.4.0 (or attach only approved Tdoc with conclusions)

1. Overall Description:

SA2 has started the discussion on the conclusion of Sensing in TR 23.700-14 and the conclusions in attached Tdoc has been agreed.

SA2 would like to resume that the main decisions with a relationship with RAN3 work:

- In this release the Sensing Entity is the gNB

- The Sensing functionalty is split in a Sensing Control Function and a Sensing Processing function. The Sensing Control Function is responsible of performing the actions related to the control of how the Sensing services is performed and the exposure to AF. The Sensing Processing function is responsible of perfomring the elaboration of Sensing Data mesurement received from sensing entity to obtain the Sensing Result.

- The interface between the Sensing Control Function and the Sensing Processing function is left to implementation in this realease

- The Sensing Control Function has a direct interface with the gNB to transfer control information (e.g. Sensing Request) as describes in KI#3.

- The Sensing Processing Function has a direct interface with the gNB to transfer Sensing Data as describe in KI#4.

- The gNB registers its own Sensing Capabilites to the SCF in order to enable the seleciton of the suitable gNB to adress the specific sensing service request. SA2 assumes that information described in the following are provided by gNB. From Stage 2 point of view the definition of detailed information is under RAN3 responsibility and it will be specified during the normative phase in cordination with RAN3.

  - The description of the Sensing Service area supported by gNB

  - The description of the Sensing QoS parameters, such as minimum sensing angular separation, minumum distance separation, etc

- the Sensing Control Function determines the configuration parameters and provide them to the selected Sensing Entity, i.e. gNB(s) for generating 3GPP Sensing Data. The detailed information provided by the Sensing Control Function to the gNB(s) depends by the information requested by gNB in order to perform the sensing measurement according to the needed sensing result, therefore from Stage 2 point of view the definition of detailed information is under RAN3 responsibility and it will be specified during the normative phase in cordination with RAN3.

- The gNB provides the Sensing Data of detected object(s) in the format of Point Cloud Data to the SPF via a direct data connection. The Point Cloud Data are eleborated by SPF to generate the Sensing Results. From Stage 2 point of view SA2 will define the assumed information needed to the generate the Sensing Result to be exposue to AF to fulfill the use case requirement. However the definition of detailed information will be specified in collaboration with RAN3.

- The definition of the protocol of the interfaces between the SCF and the gNB and between the SPF and gNB are under RAN3 responsibility.

For more detailed description of conclusion please see the TR 23.700-14 v 0.4.0.

SA2 kindly asks to RAN3 to take into accont the above information:

2. Actions:

To RAN3 group.

ACTION: 	SA2 kindly asks RAN3 to take account of the above informaiton

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #172		17th Nov – 21st Nov, 2025		Dallas, US

TSG-SA2 Meeting #173		09th Feb – 13th Feb, 2026		India, IN