SA WG2 Meeting #171	S2-2508279

13 - 17 Oct, 2025, Wuhan, China

Source:	OPPO

Title:	[WT#4] Proposed WT and KIs

Document for:	Agreement

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper is to propose Work Tasks (WTs) and Key Issues (KIs), including the rationale, expected scope, and structuring of WT#4.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

## Q4 Planning on WT#4 way forward

Q4 Planning on WT#4 way forward was discussed on FS_6G_ARC SA2#171 pre-meeting CC#1, and the following proposal was generally accepted by many companies.

Observation 1: For RAN dependent features, alignment with RAN WGs for 6G sensing work scope is necessary; for non-RAN dependent objectives, SA2 can determine the work scope based on agreement.

Proposal 1: SA2 starts to discuss the WT and KIs in Q4’25. Set a checkpoint in Q1’26 to align SA2 work scope with RAN WGs for RAN dependent objectives.  The checkpoint will not be applied for non-RAN dependent objectives, and the solution discussions on non-RAN dependent objectives can start earlier than Q1’26.

## WT and KI proposals on SA2#170

S2-2508043, S2-2507900 and S2-2507902 captured the latest technical discussions on WT and KI definitions. Although not approved, they can be used as the starting point.

Some open issues are listed in S2-2507900 for further considerations:

Aspects requiring resolution listed: Aspect 1: When to start the KI discussions for WT#4. - Option 1 (S2-2506938 Ericsson et.al): The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC as well as determination of scope of this work in RAN for 6G. SA2 could evaluate the situation in Q1 2026, for possible scoping discussions during Q2 2026. - Option 2: Start the discussion from Q3 2025 (SA2#170). Aspect 2: What is interworking/Migration, and whether or not to take them in the scope. - Supporting companies: OPPO, S2-2506364; LGE, S2-2506626; Tejas Networks. Aspect 3: Whether or not to consider Roaming in the scope. - Supporting companies: OPPO, S2-2506364; InterDigital, S2-2506402; Aspect 4: Whether or not to consider the scenario for the coverage-limited scenario (i.e., when UE is out-of-coverage). - Supporting companies: OPPO, S2-2506364; InterDigital, S2-2506402; Aspect 5: Whether or not to address the QoS aspects. - Supporting companies: LGE, S2-2506626;

Observation 2: WT and KI discussions already started on SA2#170.

Proposal 2: S2-2508043, S2-2507900 and S2-2507902 are used as the starting point for the WT and KI discussion in Q4/25. Way forward for open issues listed in S2-2507900:

- Aspect 1 (When to start the KI discussions for WT#4): Q4’25

- Aspect 2 (What is interworking/Migration, and whether or not to take them in the scope): Interworking/Migration is considered for the cases of SF and RAN nodes with different 5GA and 6G capabilities to support service continuity. A new KI needs to be defined.

- Aspect 3 (Whether or not to consider Roaming in the scope): Yes, for all features with UE impact.

- Aspect 4 (Whether or not to consider the scenario for the coverage-limited scenario (i.e., when UE is out-of-coverage).): Yes, the related use cases and requirements are already defined in TS 22.137. A new KI needs to be defined.

- Aspect 5 (Whether or not to address the QoS aspects.): This can be developed during TS phase

# 2. Proposals

Proposal 1: SA2 starts to discuss the WT and KIs in Q4’25. Set a checkpoint in Q1’26 to align SA2 work scope with RAN WGs for RAN dependent objectives.  The checkpoint will not be applied for non-RAN dependent objectives, and the solution discussions on non-RAN dependent objectives can start earlier than Q1’26.

Proposal 2: S2-2508043, S2-2507900 and S2-2507902 are used as the starting point for the WT and KI discussion in Q4/25. Way forward for open issues listed in S2-2507900:

- Aspect 1 (When to start the KI discussions for WT#4): Q4’25

- Aspect 2 (What is interworking/Migration, and whether or not to take them in the scope): Interworking/Migration is considered for the cases of SF and RAN nodes with different 5GA and 6G capabilities. A new KI needs to be defined.

- Aspect 3 (Whether or not to consider Roaming in the scope): Yes, for all features with UE impact.

- Aspect 4 (Whether or not to consider the scenario for the coverage-limited scenario (i.e., when UE is out-of-coverage).): Yes, the related use cases and requirements are already defined in TS 22.137. A new KI needs to be defined.

- Aspect 5 (Whether or not to address the QoS aspects.): This can be developed during TS phase

It is proposed to include the following changes in TR 23.801-V0.1.0.

**** First Change (updates based on S2-2508043, all new, OPPO updates under revision mark) ****

# Annex A.X. WT#4 Scope

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

The following aspects will be studied:

a)	Architecture and function support for sensing, including:

- Service authorization and revocation.

- Discovery and (re-)selection of Sensing Entities.

- Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

- Collection, distribution of sensing related data and exposure of sensing result.

b)	Sensing continuity in consideration of mobility of sensing entities and/or target objects.

c)	Support for non-3GPP sensing data and data fusion.

d)	Support of ISAC services for out-of-coverage scenario.

e)	Support of roaming, when UE is involved, e.g. the roaming UE as Sensing Entity or result generator or consumer or target object.

e)	Support of 5GA-6G interworking

NOTE X1: WT#4 needs to consider the work and conclusions of R20 FS_Sensing_ARC on gNB-based sensing.

NOTE X2: The scope of WT#4 includes sensing modes and their selection, where sensing may involve RAN node and UE.

NOTE X3: Aspects a), b) and e) require coordination with WT#5. The data framework applicable for collection, distribution and exposure of sensing related data is in scope of WT#5.

NOTE X4: Coordination with RAN WG for RAN aspects, SA3 for privacy and consent aspects, and SA5 for OAM aspects will be needed.

NOTE X5: The scope of WT#4 will be aligned with RAN for 6G. SA2 could evaluate the situation in Q1 2026, for possible scoping discussions during Q2 2026.

**** Second Change (updates based on S2-2507900 and S2-2507902, all new, OPPO updates under revision mark) ****

# 5.A.	Key Issue #A: Support of Integrated Sensing And Communication

### 5.X.1	Description

This Key Issue focuses on the 6G architecture and procedures to support Integrated Sensing And Communication (ISAC). The following aspects will be studied:

-	Architecture and function definition to support ISAC for various sensing mode(s);

NOTE 1:	The scope of this key issue includes sensing modes and their selection where sensing may involve RAN node and UE.

-	Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requirements from the Sensing service consumer, and the authorization and revocation for a UE as Sensing Entity;

-	Discovery and (re-)selection of Sensing Entity(ies) and Sensing Function(s) based on sensing service requirements from the Sensing service consumer and the capabilities of the Sensing Entity(ies) and Sensing Function(s);

-	Parameter configuration and policy provisioning of the selected Sensing Entity(ies);

-	Collection and distribution of the Sensing measurement data and the associated information for Sensing Result calculation.

-	Exposure of the Sensing Result to the Sensing service consumer.

NOTE 2:	The above-mentioned bullet may require coordination with WT#5. Data framework applicable for ISAC (e.g., Sensing measurement data collection, distribution, Sensing Result exposure, and the storage) will be addressed as part of Key Issues related to WT#5 of 6G SI on Data Framework.

-	Sensing continuity for the moving of the Object(s) and the UE(s) as the Sensing entity(ies);

-	Support for fusion of data from multiple Sensing Entities.

-	Support of roaming, when UE is involved, e.g. the roaming UE as Sensing Entity or result generator or consumer or target object.

NOTE 3:	Functionalities related to RAN nodes will be defined together with the RAN working groups.

NOTE 4:	Privacy and security require coordination with SA3, OAM and charging aspects require coordination with SA5.

# 5.B.	Key Issue #B: Non-3GPP sensing

Study below aspects on non-3GPP sensing in 6G:

- Support for collection of non-3GPP sensing data (e.g., from WLAN access, Camera, etc.), including transport and storage of sensing related data and exposure of sensing result.

- Support for fusion of sensing data (e.g., 3GPP sensing data and non-3GPP sensing data) from multiple sensing entities.

NOTE: Security and privacy aspects will be addressed by SA3.

**** Third Change (all new) ****

# 5.C.	Key Issue #C: Support of 5GA-6G Interworking and Migration

This Key Issue focuses on the enhancement to 5GA architecture for migration, the 6G architecture and the procedures to enable interworking between 5GA system and 6G system to support ISAC service (e.g. ensure seamless sensing service continuity and maintain consistent sensing quality). The following aspects will be studied:

- How to enhance 5GA sensing architecture for the migration?

- How to support interworking with 5GS, considering the differences in sensing data format, sensing modes, service control, sensing data transport, sensing data processing, QoS mechanism, etc.?

NOTE: Details will be coordinated and aligned with RAN WGs.

# 5.D.	Key Issue #D: Support of ISAC services for out-of-coverage scenario

This Key Issue focuses on the 6G architecture and the procedures to enable UE to perform ISAC service operations under operator control when not served by RAN. The following aspects will be studied:

- Service authorization and revoking authorization for UE to perform ISAC service operations when not served by RAN

- Policy authorization and parameter configuration to UE

- Selection of sensing entity(ies) (i.e. UE)

- Sensing data collection mechanism, including the sensing data (3GPP sensing data or non-3GPP sensing data) collection by itself or from other UE(s)

- Sensing data exposure to the consumer UE

NOTE: Details will be coordinated and aligned with RAN WGs.

**** End of Changes ****