3GPP SA WG2 Meeting #171	S2-2508313

# Wuhan, China, 13 – 17 October 2025                                                 (revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0070 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of ENs on security aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several Editor’s notes on security aspects unresolved in TS 23.369 v19.1.0, and these ENs can be resolved based on security solutions of TS 33.369 v19.0.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of ENs on security aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs on security aspects unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8, 6.2.2, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8	Filtering Information

The Filtering Information is used to identify or filter one or multiple AIoT Device(s). The Filtering Information is constructed by one or multiple filtering elements and each filtering element corresponds to a component (i.e. ID Type, PLMN Identifier, NID, Third Party Identifier or Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7.

Each filtering element shall include:

-	Component Type Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

-	An Offset from the beginning of the indicated component of the AIoT Device Permanent Identifier, which indicates the start location in the indicated component to be used to compare with the corresponding bitstring.

When the Component Type Information of a filtering element indicates a PLMN ID, NID or Third Party Identifier component, then the Offset is not included in the filtering element, i.e. the whole component is compared with the bitstring.

When the Component Type Information of the filtering element indicates an Identification Information component, the Offset is always included, i.e. the component from the start location indicated by the corresponding Offset is compared with the bitstring.

There may be multiple filtering elements corresponding to the component Identification Information of the AIoT Device Permanent Identifier.

NOTE 1:	It is assumed that the AIOTF includes at most one filtering element within the Filtering Information for component ID Type, PLMN ID, NID or Third Party Identifier of the AIoT Device Permanent Identifier. It is assumed that the AIoT Device does not validate the number of filtering element(s) for any indicated component within the Filtering Information.

NOTE 2:	The format of the Filtering Information is specified in clause 31.3 of TS 23.003 [6].

Figure 5.8-1: The example of the comparing the bitstring to the component

To determine whether an AIoT Device Permanent Identifier matches the Filtering Information, the bitstring of every filtering element within the Filtering Information is compared  with the indicated component of the AIoT Device Permanent Identifier. If all the compared bitstrings match the corresponding component of the AIoT Device Permanent Identifier, then an AIoT Device Permanent Identifier matches the Filtering Information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the Filtering Information.

# * * * * Second change * * * *

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory Request (AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], [location information requested]) service operation request to the NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

The time interval, if provided, is described in clause 5.9.

The location information requested parameter indicates whether the AF expects the network to provide the device location information.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information from the External Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the Nnef_AIoT_Inventory request with an appropriate cause code and step 6 is performed before ending the procedure

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], [location information requested]) service operation towards each of the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Inventory request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the AIOTF request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or an AIoT Device Permanent Identifier as defined in clause 5.7.2. When privacy protection is used, the AIoT Identification Information includes the T-ID as defined in clause 5.4.3 of TS 33.369 [9].

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF may perform AMF selection as described in clause 5.3.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, Requested Service Area Information and assistance information to the selected NG-RAN as specified in TS 38.413 [10].

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the AIoT service operation accordingly as specified in TS 38.413 [10]. If the Inventory Request is not rejected, then an AIoT Session is created in the NG-RAN. After this step, the AIoT Session between the NG-RAN and AIOTF identified by correlation ID is established.

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation as specified in TS 38.300 [5] and TS 38.391 [11]. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the Filtering Information, as described in clause 5.8.

The AIoT Device determines whether it matches the T-ID as described in clause 5.4.3 of TS 33.369 [9].

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device Permanent Identifier. When privacy protection is used, the AIoT Device Permanent Identifier is not included in the AIOT NAS message as described in clauses 5.4.2 and 5.4.3 of TS 33.369 [9].

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s) as specified in TS 38.413 [10]. The NG-RAN may aggregate multiple Inventory Report messages based on the assistance information before reporting the response to the AIOTF as described in clause 5.9. The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. When privacy protection is used, the AIOTF retrieves the AIoT Device Permanent Identifier(s) from the ADM as described in clauses 5.4.2 and 5.4.3 of TS 33.369 [9]. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete and the last inventory result. After the procedure has completed NG-RAN will not send any further Inventory Reports for this requested Inventory.

Editor's note:	The details about completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the Naiotf_AIoT_Inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s) and optionally location of each AIoT Device. The AIOTF may send multiple reports.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN in step 10 to determine the AIoT Device Location.

The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. When the last report is sent, the AIOTF ends the AIoT Session. If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Notify's from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the Nnef_AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s) and optionally location of each AIoT Device. The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

# * * * * Third change * * * *

6.2.3	Command Procedure

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request (AF ID, Command Type, information about the target AIoT Device(s), [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

2.	Step 2 of the Inventory Procedure specified in clause 6.2.2 is performed for External Target Area information processing and AIOTF selection with the following clarifications:

-	If AIOTF selection fails, the NEF rejects the Nnef_AIoT_Command request and step 6 of this procedure is performed instead.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, information about the target AIoT Device(s), [Target area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Command Request and checks the parameters included in the request. The AIOTF performs Reader Selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF performs AF authorization for AIoT service operation request as described in clause 5.6.

The AIOTF may perform AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory.

-	In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10].

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10].

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

NOTE 1:	Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

NOTE 2:	Security protection of NAS Command Request is described in clause 5.3.2 of TS 33.369 [9].

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11].

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the AIoT data.

NOTE 3:	Security protection of NAS Command Response is described in clause 5.3.2 of TS 33.369 [9].

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received.

12.	The AIOTF reports the result of the Naiotf_AIoT_Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Command Notify messages from multiple AIOTFs.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN during inventory in step 7 to determine the AIoT Device Location.

When the last report is sent, the AIOTF ends the AIoT Session.

13.	The NEF informs the AF of the result of the Nnef_AIoT_Command request by sending the Nnef_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]).

# * * * * End of changes * * * *