3GPP TSG-WG SA2 Meeting #171	S2-2508392

# 13 – 17 October, 2025, Wuhan, China                         (revision of S2-2507048)

Source: 	vivo

Title: 	KI#1, sol#10 update: Energy consumption calculation enhancements

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update solution#10 for KI#1 in TR 23.700-67.

1. Introduction

This contribution proposes to solve the editor’s note in solution#10 for KI#1 of R20 FS_EnergySys_Ph2 study.

- Some clarifications are added to show how this solution can cancel out the impact of the idle state energy consumption of the node.

Basically, the energy consumption of the node include two parts:

(a) Enode, data transmission : the energy consumed for data processing/transmission related tasks; and

(b) Enode,  static part  : the static energy consumed consumption information for maintaining the basic operation and assisting the main tasks at the node (e.g., memory retention or background monitoring).

So if we consider the difference of energy consumption between two time intervals, the part (b) Enode,  static part  between two time intervals can be considered similar and can be canceled out. So the difference of the energy consumption between two time intervals equals to the difference of the energy consumption for data transmission between two time time intervals. In this case, the impact of part (b) Enode,  static part   on the calculation of energy consumption per bit can be canceled out.

Please also see the detailed explaination within the solution, especially the equations (1.1) to (1.5).

- For the following editor’s note:

Editor's note:	How to solve the case of the energy consumption fluctuation at the node needs further clarification.

This EN proposes a new scenario of the energy consumption fluctuation, however, it is not clearly proven whether the energy consumption fluction exist. If it truly exists, it will also impact the release 19 calcution result not only this solution.

If we assume the energy consumption fluctuation exist, in order to sovle the potential issue about the energy consumption fluctuation, it is proposed to select the time interval Tb as the following:

- For the selection of time interval Tb, the total energy consumption over Tb needs to be considered. Normally, the energy consumption information and data volume information over the two time intervals should have the following relationship:

If DVUPF, Tb >DVUPF, Ta, then EUPF,Tb > EUPF,Ta;

If DVUPF, Tb < DVUPF, Ta, then EUPF,Tb < EUPF,Ta;

If the energy consumption fluctuation happens as in the EN stated, which means the following special case:

If DVUPF, Tb >DVUPF, Ta, then EUPF,Tb >> EUPF,Ta;

In this case, it is proposed to solve the issue as following:

The EIF could select the time interval Tb over which the total energy consumption within Tb is not changed dramatically compared with that over Ta.

If the total energy consumption over Tb is not changed dramatically compared with total energy consumption over Ta, it could be selected for calculation; Otherwise, the EIF could select another time interval Tb' over which the total energy consumption is not changed dramatically compared with that over time interval Ta. A NOTE has also been added to explain that the criterial for how the total energy consumption is considered as the dramatic change is based on EIF implementation.

Normally, the time interval Tb can be the time intervals after Ta. If there is energy consumption fluctuation at time interval Tb, it is suggested to choose another time interval Tb' which may be the previous time intervals before Ta.

- For the special time interval Ta, if the energy consumption over Ta is dramatically different from all the other time intervals, i.e., there is no other time intervals could help the calculation, the calculation could be based on the formula in Release 19.

According to the above analysis, it is proposed to add the related descriptions and NOTE 3 in the solution to clarify the energy consumption fluctuation issue to solve this editor’s note.

- For the following editor’s note:

Editor's note:	How to select time interval Tb for calculation is FFS.

For the selection of time interval Tb, it is proposed that the selection of the time interval Tb could be flexible. However, the static energy consumption for maintaining basic operation over the two time intervals could be similar, if the two time intervals are close to each other. So, it is suggested that the time interval Tb should be close to time interval Ta, in order to minimize the impact of energy consumption for basic operation.

The related descriptions can be found in NOTE 1. Furthermore, the updated descriptions and newly added NOTE 3 for the first Editor’s note can also apply to this Editor’s note.

- For the following editor’s note:

Editor's note:	The calculation formula in this solution may cause some delay. The delay needs to be further considered during the evaluation.

This Editor’s note is related with the evaluation of the solution. However, considering that the energy consumption information calculation for the time window is the sum of energy consumption information over the time intervals T within the time window, so the delay of the solution compared with the current calculation model is mainly at the calculation of energy consumption information over last time interval. For the last time interval, the previous time interval before the last time interval can be selected. So it is proposed to add the following NOTE to clarify this.

NOTE 6:	The solution may cause some delay. However, considering that the energy consumption information calculation for the time window is the sum of energy consumption information over the time intervals T within the time window, so the delay of the solution compared with the current calculation model is mainly at the calculation of energy consumption information over last time interval. For the last time interval, the previous time interval can be selected.

- For the following editor’s note:

Editor's note:	Whether the solution improves accuracy is FFS.

Regards to this Editor’s note, it is proposed to add NOTE 5 to clarify that this solution aims to minimize the impact of static energy consumption for maintaining basic operation compared with the current calculation examples defined in release 19 in Annex T of TS 23.501. The accuracy and robustness issue caused by static energy consumption for maintaining basic operation can be alleviated or improved by this solution. However, this solution doesn’t solve the accuracy and uncertainty issue of energy consumption measurement at the node level due to the measurement instrument or measurement environment.

NOTE 5:	The solution aims to minimize the impact of static energy consumption for maintaining basic operation compared with the calculation examples defined in release 19 in Annex T of TS 23.501. The accuracy and robustness issue caused by static energy consumption for maintaining basic operation can be alleviated or improved by this solution. However, this solution doesn’t solve the accuracy and uncertainty issue of node level energy consumption measurement due to the measurement instrument or measurement environment.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

FIRST CHANGE

6.10	Solution #10: Energy consumption calculation enhancements

6.10.0	High-level solution principles

The aim of this solution is to enhance the mathematical approach of energy consumption information for the required granularity defined in Rel-19 to reduce the impact of the energy consumption for maintaining the basic operation and assisting the main tasks at the node (e.g., memory retention or background monitoring, the energy consumption when the node is in idle/inactive state). The principle of the solution is to calculate energy consumption of the required granularity as following:

-	For the time interval T, the EIF calculates the per bit energy consumption at the network node by using the ratio of the energy consumption difference to the total data volume difference between the two time intervals.

-	Then, the EIF applies the per bit energy consumption as the coefficient to calculate the energy consumption of the required granularity at the node (i.e., UPF and gNB) over the time interval T.

-	The EIF adds the energy consumption of the required granularity at the node (i.e., UPF and gNB) over the multiple time intervals for the required time window, as defined in Release 19.

-	Finally, the EIF obtains the energy consumption of the required granularity by adding the information at UPF(s) and gNB for the required time window, as defined in Release 19.

6.10.1	Description

This solution applies to Key Issue #1.

In the Release 19 features, the calculation formulas for energy consumption in EIF was proposed. The details of the calculation model are captured in Annex T of TS 23.501 [2].

In the Release 19 calculation formulas, it is based on the transmitted data volume information at the related granularity and the coefficients of per bit energy consumption at the node. For example, the energy consumption at UE level at the UPF over a time interval T is calculated as follows:

EUE,UPF=EUPFDVUPFDVUE,UPF	(1)

The main principle of this calculation is that EUPFDVUPF is coefficient that is common across all granularities, within the time interval T, for the network entity UPF and is the value that related to the energy consumption per bit at UPF.

The main issue of the current Data volume based calculation formula (1) is that the coefficient of energy consumption per bit EUPFDVUPF is not accurate and robust.

EUPF, which is obtained from OAM, is the total energy consumed at the UPF within the time interval T, which includes the following two parts:

(a)	EUPF, data transmission : the energy consumed for data processing/transmission related tasks; and

(b)	EUPF,  static part  : the static energy consumption information for maintaining the basic operation at the node (e.g., memory retention or background monitoring).

So, EUPF=EUPF,  data transmission +EUPF,  static part   (1.1)

Regards to energy consumption for part (a) EUPF, data transmission , it is closely related with the data processing/transition at the node. So EUPF, data transmission would be proportional to the processed/transmitted data volume DVUPF at the node.

Regards to energy consumption for part (b) EUPF,  static part  , it is related with the static energy consumption when the node is at idle/inactive state and also related with some functions for basic operation of the node. So EUPF,  static part   cannot be proportional to the processed/transmitted data volume DVUPF at the node.

In this case, the EUPF is not proportional to DVUPF.

So the relationship of the total energy consumption EUPF and total data volume DVUPF at the node UPF is similar to the following curve shown in Figure 6.10.1-1.

Figure 6.10.1-1: Example for the relationship of total energy consumption and total data volume at the node

Considering the fact that the EUPF includes two parts EUPF, data transmission  and EUPF,  static part  , and EUPF cannot be proportional to the processed/transmitted data volume DVUPF at the node UPF, it is not suitable to use the coefficient EUPFDVUPF as the energy consumption per bit at the node UPF. In this case, this coefficient EUPFDVUPF is not robust and accurate enough to reflect the energy consumption per bit information at the node UPF.

In order to minimize the impact of static energy consumption on the calculation for per bit energy consumption, it is proposed to enhance the calculation method  as following.

Let’s take the energy consumption at UPF for example. Currently, the EIF could collect the total energy consumption EUPF and total data volume DVUPF at UPF from OAM for multiple time intervals, e.g. time interval T1, time interval T2, ..., time interval Tn. In order to calculate the per-bit energy consumption over time interval Ta, it is proposed to choose another time interval Tb. Based on Equation (1.1), we have the following expressions:

EUPF,Ta=EUPF, static part, Ta+EUPF, data transmission, Ta    (1.2)

EUPF,Tb=EUPF, static part, Tb+EUPF, data transmission, Tb    (1.3)

If time interval Ta and Tb are close to each other, the static energy consumption during these two time intervals can be assumed to be similar. In this case, we have:

EUPF, static part, Ta=EUPF, static part, Tb                  (1.4)

Therefore, based on Equations (1.2) to (1.4), the difference in energy consumption between time interval Ta and time interval Tb can be expressed as:

EUPF,Ta-EUPF,Tb=EUPF, static part, Ta+EUPF, data transmission, Ta-EUPF, static part, Tb+EUPF, data transmission, Tb=EUPF, data transmission, Ta-EUPF, data transmission, Tb         (1.5)

Based on Equation (1.5), we can obtain that the difference in energy consumption between two time intervals can reflect the difference in energy consumption for data transmission between two time intervals. So the difference in energy consumption between two time intervals is proportional to the difference in data volume between two time intervals.

NOTE 1:	The selection of the time interval Tb could be flexible. However, the energy consumption for basic operation over the two time intervals could be similar, if the two time intervals are close to each other. So, it is suggested that the time interval Tb should be close to time interval Ta, in order to minimize the impact of energy consumption for basic operation.

NOTE 2:	If DVUPF, Tb equals to DVUPF, Ta, it is suggested to select another time interval Tb’ for the calculation.

Normally the change of the total energy consumption is due to the change of the total data volume information. For the case that the total energy consumption of the node may change dramatically not due to the data volume, the selection of time interval Tb could be considered as following:

- For the selection of time interval Tb, the total energy consumption over Tb needs to be considered. Normally, the energy consumption information and data volume information over the two time intervals should have the following relationship:

If DVUPF, Tb >DVUPF, Ta, then EUPF,Tb > EUPF,Ta;

If DVUPF, Tb < DVUPF, Ta, then EUPF,Tb < EUPF,Ta;

If there is energy consumption fluctuation happens as in the EN stated, which means the following special case happens:

If DVUPF, Tb >DVUPF, Ta, then EUPF,Tb >> EUPF,Ta;

The following can be considered:

EIF could select the time interval Tb over which the total energy consumption is not changed dramatically compared with that over Ta.

If the total energy consumption over Tb is not changed dramatically compared with total energy consumption over Ta, it could be selected for calculation; Otherwise, the EIF could select another time interval Tb' over which the total energy consumption is not changed dramatically compared with that over time interval Ta.

-	For the special time interval Ta, if the energy consumption over Ta is dramatically different from all the other time intervals, i.e., there is no other time intervals could help the calculation, the calculation could be based on the formula in Release 19.

NOTE 3:	Normally, the time interval Tb can be any time interval after Ta. Considering the case that energy consumption fluctuation may happens at the time intervals after Ta, it is suggested to choose another time interval Tb’ which may be the previous time interval before Ta.

NOTE 4:	The criterial for how the total energy consumption is considered as the dramatic change is based on EIF implementation.

The EIF consider the following values over the two time intervals Ta and Tb:

For time interval Ta: the total energy consumption EUPF,Ta and total data volume DVUPF, Ta at UPF over Ta.

For time interval Tb: the total energy consumption EUPF,Tb and total data volume DVUPF, Tb at UPF over Tb.

Then, the EIF calculates the energy consumed per bit at UPF for time interval Ta as following:

For time interval Ta:

EUPF, per bit, Ta=EUPF,Ta-EUPF,TbDVUPF,Ta-DVUPF,Tb	(2)

In this formula (2), EUPF,Ta-EUPF,Tb is the difference of the energy consumption at UPF between the two time interval Ta and Tb. Based on formular (1.2) and (1.3), we have the following expression:

EUPF,Ta-EUPF,Tb=EUPF, static part, Ta+EUPF, data transmission, Ta-(EUPF, static part, Tb+EUPF, data transmission, Tb) (2.1)

Considering the time intervals Ta and Tb are close, the static energy consumption over time interval Ta EUPF, static part, Ta EUPF,Taand static energy consumption over time interval Tb EUPF, static part, TbEUPF,Tb are very close and could be considered as the same, so EUPF, static part, Ta=EUPF, static part, Tb. So the formular (2.1) can be further expressed as following:

EUPF,Ta-EUPF,Tb=EUPF, static part, Ta+EUPF, data transmission, Ta-EUPF, static part, Tb+EUPF, data transmission, Tb=EUPF, static part, Ta-EUPF, static part, Tb+EUPF,data transmission, Ta-EUPF, data transmission, Tb=EUPF,data transmission, Ta-EUPF, data transmission, Tb

(2.2)

In this case, the energy consumption difference between the two time interval Ta and Tb, i.e. EUPF,Ta-EUPF,Tb (which equals to EUPF,data transmission, Ta-EUPF, data transmission, Tb), is due to or corresponds to the difference of the total data volume processed and transmitted at the UPF between the time interval Ta and Tb, i.e. (DVUPF,Ta-DVUPF,Tb). So the impact of the static energy consumption is minimized and cancelled out. In this case, the ratio of (EUPF,Ta-EUPF,Tb) to (DVUPF,Ta-DVUPF,Tb) , as expressed in formular (2), could reflect the energy consumption per bit data at UPF at the time interval Ta.

NOTE 5:	The solution aims to minimize the impact of static energy consumption for maintaining basic operation compared with the calculation examples defined in release 19 in Annex T of TS 23.501. The accuracy and robustness issue caused by static energy consumption for maintaining basic operation can be alleviated or improved by this solution. However, this solution doesn’t solve the accuracy and uncertainty issue of node level energy consumption measurement due to the measurement instrument or measurement environment.

NOTE 6:	The solution may cause some delay. However, considering that the energy consumption information calculation for the time window is the sum of energy consumption information over the time intervals T within the time window, so the delay of the solution compared with the current calculation model is mainly at the calculation of energy consumption information over last time interval. For the last time interval, the previous time interval can be selected.

Based on the above principles, the details of the calculation formulas are described in the following clause.

6.10.2	Procedures

The EIF can obtain the total energy consumption information and data volume information at the node (e.g., UPF, gNB) over time interval T from OAM as defined in the procedure in clause 4.29.2 of TS 23.502 [3]. It is proposed to enhance the calculation formulas based on the principles explained in formula (2).

Let's take the energy consumption at UPF as an example. It is proposed that the ratio of (EUPF,Ta-EUPF,Tb) to (DVUPF,Ta-DVUPF,Tb) as defined in formula (2) could be considered as the coefficient of the energy consumption per bit at UPF over the time interval Ta.

So based on the above coefficient defined in formula (2), the energy consumption at UPF for the required granularities (UE, S-NSSAI, PDU Session, Service Data Flow) over time interval Ta could be defined as following:

EUPF, UE, Ta=EUPF,Ta-EUPF,TbDVUPF,Ta-DVUPF,TbDVUPF,UE,Ta	(3)

EUPF,S-NSSAI,Ta=EUPF,Ta-EUPF,TbDVUPF,Ta-DVUPF,TbDVUPF,S-NSSAI,Ta	(4)

E UPF,Session,Ta=EUPF,Ta-EUPF,TbDVUPF,Ta-DVUPF,TbDVUPF,Session,Ta	(5)

EUPF,Flow,Ta=EUPF,Ta-EUPF,TbDVUPF,Ta-DVUPF,TbDVUPF,Flow,Tb	(6)

Similarly, the energy consumption at gNB for the required granularities (UE, S-NSSAI, PDU Session, Service Data Flow) within time interval Ta could be defined as following:

EgNB,UE,Ta=EgNB,Ta-EgNB,TbDVgNB,Ta-DVgNB,TbDVgNB,UE,Ta	(7)

EgNB,S-NSSAI,Ta=EgNB,Ta-EgNB,TbDVgNB,Ta-DVgNB,TbDVgNB,S-NSSAI,Ta	(8)

EgNB,Session, Ta=EgNB,Ta-EgNB,TbDVgNB,Ta-DVgNB,TbDVgNB,Session,Ta	(9)

EgNB,Flow,Ta=EgNB,Ta-EgNB,TbDVgNB,Ta-DVgNB,TbDVgNB,Flow,Ta	(10)

So based on the formulas (3) - (10), it is proposed to derive the energy consumption in the network in the time interval Ta for a UE (EUE), S-NSSAI (ES-NSSAI), PDU Session (ESession), QoS flow (EFlow) by adding the energy consumption for the corresponding granularity at time interval Ta at all nodes for the data transmission and processing, as defined in Annex T of TS 23.501 [2].

For example, the network energy consumption for the UE level at time interval Ta at all the nodes are derived as following:

EUE=gNBi∈gNBUETaEUE, Gnbi+ UPFi∈UPFUETaEUE, UPFi	(11)

where gNBUETa are all gNBs used by the UE in an interval Ta, UPFUETa are all UPFs used by the UE in an interval Ta.

Accordingly, the energy consumption at required granularity, e.g. S-NSSAI (ES-NSSAI), PDU Session (ESession), QoS flow (EFlow) can also be derived by the EIF.

And then, the EIF obtains the energy consumption at the requested granularity over a certain time window by aggregating the energy consumption information calculated at multiple time intervals within this time window. The EIF may expose the energy consumption at required granularity over a certain time window to AF as described in clause 5.51.2.4 of TS 23.501 [2].

6.10.3	Impacts on existing services, entities and interfaces

EIF:

-	Support the new formulas as one of the examples of energy consumption calculation of required granularities. These new formulas are the enhancements on the Release 19 formulas.

End of CHANGES