SA WG2 Meeting #171	S2-2508405

Wuhan, China, 13 – 17 October 2025	(revision of S2-2507075)

Source:	CATT, China Telecom, CENC

Title:	KI #1 New solution: Support QoS differentiation for UP based IMS voice service over NB-IoT NTN

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: Introduce a new solution to key issue#1 in TR 23.700-19.

1. Discussion

TR23.700-19 has proposed the KI#1 that supports IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

Considering SA2 has agreed to study CP CIoT EPS optimization and UP CIoT EPS optimization, this solution shows the support of IMS voice calls using user plane.

2. Proposal

It is proposed to add a new solution for KI#1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

* First change *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[21]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[22]	3GPP TS 23.003: "Numbering, addressing and identification".

[23]	3GPP TR 22.887: "Study on satellite access - Phase 4".

[24]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[a]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS) ".

* Second change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #1 | X | X |  |  |  |
| #2 | X |  |  |  |  |
| #3 | X |  |  |  |  |
| #4 | X | X | X | X |  |
| #5 | X |  |  |  |  |
| #6 | X |  |  |  |  |
| #7 | X | X |  |  |  |
| #8 | X |  |  |  |  |
| #9 | X |  |  |  |  |
| #10 | X |  |  |  |  |
| #11 | X |  |  |  |  |
| #12 |  | X |  |  |  |
| #13 |  | X |  |  |  |
| #14 |  | X |  |  |  |
| #15 |  | X |  |  |  |
| #16 |  | X |  |  |  |
| #17 |  | X |  |  |  |
| #18 |  | X |  |  |  |
| #19 |  | X |  |  |  |
| #20 |  | X |  |  |  |
| #x | X |  |  |  |  |

* Third change *

6.X	Solution #X: Support QoS differentiation for UP based IMS voice service over NB-IoT NTN

6.X.0	High level principles

This solution addresses Key Issue 1 and supports IMS voice call GEO via NB-IoT NTN connected to EPC using UP bearers with the following key principles:

The solution is based on exchanging the IMS signalling and the voice packets over one default EPS bearer or two bearers for the same PDN Connection.

6.X.1	Description

According to clause 6.5.0 of TS 24.301 [a], when the UE accesses the network via NB-IoT, the maximum number of supported DRBs by the UE is determined as follows: If the UE indicates support for multiple DRBs in Attach or TAU request, it may support up to 2 DRBs. Otherwise, the UE supports 1 DRB by default. To support IMS voice service, the IMS signalling and IMS voice media may be transmitted either over 2 DRBs (one for IMS signalling and the other for voice packets) or 1 DRB. Due to the different transmission requirements of IMS signaling and voice packets, it is necessary to distinguish the QoS/priority between IMS signaling and voice data.

6.X.2	Procedures

The high-level procedure for attach and PDN connection establishment for NB-IoT IMS voice call over GEO is depicted in Figure 6.x.3-1:

Figure 6.X.3-1: high-level procedure for connection setup for NB-IoT IMS call over GEO

1. The UE sends an Attach/TAU Request to the MME via the eNB. The request may include the UE capability of supporting IMS voice over GEO via NB-IoT. If the UE supports the multiple DRBs, the steps 7-10 are performed, otherwise, the steps 11-14 are performed. The request may also include the IMS APN.

2a. The MME retrieves the UE's subscription information. The UE's subscription information may include the subscription data for voice calls over GEO satellites such as subscribed QoS profile for GEO satellite voice calls.

2b. The MME determines that the UE is accessing the network via a GEO satellite based on the RAT type, i.e. RAT type being "NB-IoT(GEO)" as defined in clause 4.13.2 of TS 23.401 [5]. Additionally, the MME may initiate a radio capability verification procedure related to GEO satellite voice calls with the eNB.

3. 	The MME sends a Create Session Request message to the S-GW and the subscription data. The Create Session Request message includes the RAT type "NB-IoT(GEO)" and the capability of the UE, which are sent further by the S-GW to the PDN GW and then the PDN GW to the PCRF.

4. 	The MME sends an Attach Accept message to the UE, indicating that the network supports Voice over GEO capability. The default EPS Bearer is established.

5. 	The UE sends a PDN Connectivity Request to MME to indicate the requested PDN type is IP, and the APN is IMS. The PDN Connectivity Request may also include the header compression configuration, which helps reduce the transmission bandwidth and latency for IMS signalling and media related to GEO satellite voice calls.

6. 	The UE initiates the IMS registration procedure, with IMS signalling being transmitted over the default EPS bearer.

7. 	If the UE supports the multiple DRBs, the steps 7-10 are performed, otherwise, steps 11-14 are performed. Based on IMS session establishment-related request messages sent by IMS network functions (e.g., P-CSCF) to the PCRF, the PCRF initiates an IP-CAN Session Modification procedure towards the PDN GW. Then based on the RAT type "NB-IoT(GEO)" and the UE supports multiple DRBs, the PDN GW decides to establish a dedicated EPS bearer for IMS voice media transmission. The PDN GW then sends a Create Bearer Request message to the Serving GW, and the Serving GW sends a Create Bearer Request message to the MME accordingly.

8. 	The MME sends a dedicated EPS bearer context request message based on the message received in step 7 to the UE via the eNB. These messages contain the service flow description information for voice over GEO (voice media) and the corresponding QoS parameters.

9.	The UE stores the parameters received in the dedicated EPS bearer context request message and responds to the network by sending the corresponding accept message.

10. For the activation of a dedicated EPS bearer to carry IMS voice media, IMS signalling continues to be transmitted over the default EPS bearer and IMS voice media is transmitted over the dedicated EPS bearer.

11.	If the UE supports 1 DRB, steps 11-14 are performed. Based on IMS session establishment-related request messages sent by IMS network functions (e.g., P-CSCF) to the PCRF, the PCRF initiates an IP-CAN Session Modification procedure towards the PDN GW. Then based on the RAT type "NB-IoT(GEO)" and the UE supports 1 DRB, the PDN GW decides to modify the existing default EPS bearer for IMS voice media transmission. The PDN GW then sends an Update Bearer Request message to the Serving GW, and the Serving GW sends an Update Bearer Request message to the MME accordingly.

12. The MME sends a modify EPS bearer context request message based on the message received in step 11 to the UE via the eNB. These messages contain the service flow description information for voice over GEO (voice media) and the corresponding QoS parameters.

13. The UE stores the parameters received in the modify EPS bearer context request message and responds to the network by sending the corresponding accept message.

14. For modifying the existing default EPS bearer to carry IMS voice media. Based on the parameters in the modify EPS bearer context request message, the UE uses the default EPS bearer to send/receive IMS voice media and transmit IMS signalling. The UE performs packet marking for IMS signalling and voice packets, or marks the start and the end of packets of IMS signalling and voice media separately, thereby enabling the network to apply differentiated QoS control to IMS signalling and voice media.

6.X.3	Impacts to Services, Entities and Interfaces

UE:

-	Requests for connection setup for IMS voice services when access via NB-IoT (GEO) cell.

-	Indicates its capability of supporting IMS voice call over GEO satellite via NB-IoT in the Attach/TAU Request or PDN connection setup request.

-	Performs packet marking for IMS signalling and voice packets, or marks the start and the end of packets of IMS signalling and voice media separately, when IMS signalling and voice media is transmitted over the same EPS bearer.

-  May indicate its capability of supporting multiple DRBs for IMS signalling and voice, where multiple DRBs refers to no less than two DRBs.

MME:

-	Checks the subscription data for IMS voice over GEO satellite and authorizes the Attach/TAU Request or PDN connection setup request for IMS voice services from the UE.

-	Provides information (e.g. indication of UE supporting multiple DRBs) to the SGW/PGW for creating session for IMS voice over GEO satellite for the UE.

PDN GW:

-	Determines whether to establish a dedicated EPS bearer or modify the existing default EPS bearer for IMS voice media transmission, based on whether the UE supports multiple DRBs and the RAT type "NB-IoT(GEO)".

Editor’s Note: The exact number of multiple DRBs depends on the RAN's conclusion.

* End of changes *