SA WG2 Meeting #171	S2-2508474

13 – 17 Oct, 2025, Wuhan, China	revision of S2-2508130

Source:	[Huawei, HiSilicon, Vivo, China Mobile, Oppo, Boost Mobile Network, ZTE, LG Electronics, Intel, CATT,] China Telecom, [China Unicom, NEC, Lenovo, ETRI, Xiaomi, Futurewei, IIT Bombay, Turk Telekom, KPN, Rakuten Mobile, TNO, Deutsche Telekom, Oracle]

Title:	[WT#3.1] AI for 6G architecture

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document summarises the proposals for WT#3 regarding the use of AI for 6G architecture.

1. Discussion

Based on the discussion in SA2#170 and conference call, partial content in WT scope is stable and acceptable. This paper mainly proposes three contents to be studied based on the version shared by rapporteur in email thread:

- Support of AF requesting AI capabilities.

In 5G AI architecture, some AI capability request is from AF (e.g. VFL request from AF) and AF could also be consumer of NWDAF in several Analytics IDs. In 6G, this is also applied. Besides, SA1 studied use cases in TR 22.870 about the 3rd party requests the AI capability provided by network. Thus, the requests from AF should be considered in the scope as the request from UE.

- Support of performance evaluation to further assure quality of AI technologies.

AI technology inherent uncertainties and model performance variations introduce risks to communication network. For instance, ML hallucination-induced network function level erroneous decisions may trigger systemic failures. Thus, it is proposed to support performance evaluation of introduced AI technologies ensuring operator could monitor and take further actions (e.g. retrain model, determine to fallback non-AI mode).

- Support of Unified Life Cycle Management (LCM) framework of AI/ML model.

For life cycle management in 5G, SA2 specifies training, inference, monitoring, model storage, model delivery, data collection coordination, data delivery, etc. in the core network, while SA5 also covers ML model training, ML model testing, AI/ML inference emulation, ML model deployment, AI/ML inference. These domain-specific definitions show both overlaps and differences. This results in interoperability issues among AI capability and datasets across various domains, significantly reducing the efficiency of model training and influencing the AI performance. Thus, it is necessary to define a unified LCM framework with the coordination with SA5.

As for the Key Issue, it is suggested to use the similar content in WT scope to avoid redundant discussion.

**** First Change ****

Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 4: 		The term AI agent refers to an automated intelligent entity that achieves a specific goal (autonomously or not) on behalf of another entity, by e.g. interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, and executing tasks (independently or in collaboration with other AI Agents). As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.1.1:	Study how to natively integrate and use AI technologies (e.g. AI Agent, NF- embedded Distributed AI, Reinforcement Learning, etc.) that were not introduced in 5G into the 6G system to fulfil 6G services and system performance requirements, support customised services and achieve increased levels of performance, QoS/QoE, efficiency, scalability and resilience, while enabling fully standardized multi-vendor interoperability, including:

-	understanding requests (e.g. intent) from UE  or AF and network events,

-	acting autonomously and controllably to dynamically compose, adapt, and invoke diverse network capabilities via e.g. acquiring contextual information, reasoning, explainability, self-learning, decision-making, generating results, interacting with other network functions, coordinating distributed AI capabilities, enabling closed-loop interactions,

-	accessing  internal network AI capabilities provided by 6G NFs and optionally operator-controlled capabilities provided by AF,

-	determining to use AI-native mode or non-AI mode.

-	performance evaluation of the introduced AI technologies and necessary actions to assure quality,

- Defining a Unified Life Cycle Management (LCM) framework to support model training, inference, performance monitoring, evaluation, testing, etc.

WT#3.1.2:	Study whether and how to:

-	support and enhance AI-related functionalities (e.g. model training, model provisioning, model and analytics performance monitoring, Federated Learning) into 6G CN considering 5G CN AI-related functionalities as a starting point for discussion, which includes fully standardized multi-vendor interoperability, reliable and sustainable AI.

NOTE 1:	 Coordination between 5G CN AI and 6G CN AI, if needed, e.g. for purpose of consistent service across 5G and 6G.

NOTE 2: Integration of AI technologies into 6G CN should be motivated by clear justification.

NOTE 3:	Aspects related to SA3, SA5, SA6, RAN will be coordinated with each group respectively during the study.

NOTE 4: The customized services do not include In Network Computing (WT#6) and ISAC (WT#4) services.

**** Second Change ****

5.X. Key Issue #X: Key issue on AI for 6G architecture

Key Issue #3.1.1:	Study how to natively integrate and use AI technologies (e.g. AI Agent, NF- embedded Distributed AI, Reinforcement Learning, etc.) that were not introduced in 5G into the 6G system to fulfil 6G services and system performance requirements, support customised services and achieve increased levels of performance, QoS/QoE, efficiency, scalability and resilience, while enabling fully standardized multi-vendor interoperability, including:

-	understanding requests (e.g. intent) from UE or AF and network events,

-	acting autonomously and controllably to dynamically compose, adapt, and invoke diverse network capabilities via e.g. acquiring contextual information, reasoning, explainability, self-learning, decision-making, generating results, interacting with other network functions, coordinating distributed AI capabilities, enabling closed-loop interactions,

-	accessing internal network AI capabilities provided by 6G NFs and optionally operator-controlled capabilities provided by AF,

-	determining to use AI-native mode or non-AI mode.

-	performance evaluation of the introduced AI technologies and necessary actions to assure quality,

- Defining a Unified Life Cycle Management (LCM) framework to support model training, inference, performance monitoring, evaluation, testing, etc.

Key Issue #3.1.2:	Study whether and how to:

-	support and enhance AI-related functionalities (e.g. model training, model provisioning, model and analytics performance monitoring, Federated Learning) into 6G CN considering 5G CN AI-related functionalities as a starting point for discussion, which includes fully standardized multi-vendor interoperability, reliable and sustainable AI.

**** End of Changes ****