3GPP TSG-WG SA2 Meeting #171	S2-2508479

Wuhan, China, Oct 13 – 17, 2025	(revision of S2-250xxxx)

Source: 	China Telecom

Title: 	Agreed principle and conclusion update on KI#2

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: It is proposed to update the agreed principles and conclusions for KI#2.

- Discussion

Here are four topics for further consideration:

-	Whether the UPF can be the consumer of the user plane traffic pattern and behaviour analysis or not.

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

For the first topic, this paper proposes the UPF as one of consumers of the user plane traffic analytics. Here are three benefits on UPF as a consumer:

For global abnormal traffic scenarios, such as attacks originating from malicious websites, the UPF should process IP 3-tuple across all UEs. UPF as the consumer to drop packets is more efficient, as N4 rules from SMF are configured per UE. Relying on SMF for this task would reduce efficiency and unnecessarily increase the load on the N4 interface.

In ATSSS scenarios, it is UPF to determine whether specific data packets are delivered to the UE via 3GPP access or non-3GPP access for downlink traffic. If the primarily selected steering method underperforms (e.g., due to signal degradation), repeated packet loss and retransmissions may occur. Consequently, the UPF will select the secondary steering method for packet delivery. If based on analytics output, the UPF can take retransmission patterns into account. For example, the NWDAF outputs show that packets are successfully transferred only after five retransmission attempts, where "five" serves as the threshold for access switching performed by UPF. It means that the primary access may be in poor signal conditions during specific time periods. This enables the UPF to select the alternative access method during this time period in the future, thereby reducing retransmission attempts and enhancing user plane efficiency.

Some abnormal traffic pattern is complex and changeable, for example, sending 1000 data packets within 4 ms, followed by 16 ms of silence, is considered abnormal burst traffic. It is difficult for SMF to instruct UPF to take actions for this specified traffic pattern, due to poor scalability of N4 interface. UPF directly subscribing Analytics from NWDAF via SBI is more direct and automated.

For the second topic, this paper proposes to reuse existing events to collect input data. UPF events (e.g. QoS monitoring, User Data Usage Measures and User Data Usage Trends) are used for UPF input data collection in QoS flow level, PDU session level and node level if the target is “any UE”, which also includes almost all types of input data (user plane related information) collected from UPFs in solutions. Thus, these event IDs could be reused and enhanced for UPF data collection.

For the third topic, this paper proposes to use appropriate Analytics Filter information to reduce the reporting load. In R19 Analytics ID “signalling storm”, the Analytics Filter information is well applied to avoid large signalling load, similar logic may be reused.

For the fourth topic, based on the consumers of analytics service, this paper proposes several typical actions for each consumer as examples, more actions could be defined in normative phase.

2. Proposal

It is proposed to apply following text in TR 23.700-04.

* * * * First Change * * * *

### 7.1.2	Agreed Principles for KI#2

#### 7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information for input data filtering to UPF.

#### 7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics.

-	To derive the output analytics, the following input data may be collected from the UPF, SMF, OAM, AF to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic.

Editor's note:	Detail output and input data will be defined in normative phase.

-  The consumer may take actions to improve the performance of user plane, for example:

- SMF may reselect UPFs, or generate or update N4 rules to instruct the behaviours of UPFs to mitigate the abnormal behaviours.

- PCF may generate or update PCC policy to filter the abnormal traffic.

- UPF may detect malicious IP packets, adjust interface thresholds, select access type.

Editor's note:	Detail actions will be defined in normative phase.

-  Existing event exposure is enhanced for collecting input data.

-  Appropriate Analytics Filter Information is configured to reduce the load on control plane.

#### 7.1.2.3	Agreed Principles for Use Case #2

-	The service consumer of the analytics service is PCF.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics on traffic pattern information.

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow of the collected QoS monitoring information.

-	The consumer actions may take the output Analytics into account, e.g. generate or update PCC rules.

* * * * Second Change * * * *

### 7.2.2	Topics for further consideration for KI#2

The following topics are for further consideration for KI#2:

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

* * * * Third Change * * * *

# 8	Conclusions

### 8.2.	Conclusion for KI#2

The following principles are agreed as conclusion for Key Issue 2:

-  Introduction of new Analytics ID(s) for user plane traffic pattern and behaviour analysis.

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics.

-	To derive the output analytics, the following input data may be collected from the UPF, SMF, OAM, AF to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic.

-	Information identifying the associated application/service data flow of the collected QoS monitoring information.

Editor's note:	Detail output and input data will be defined in normative phase.

-  The consumer may take actions to improve the performance of user plane, for example:

- SMF may reselect UPFs, or generate or update N4 rules to instruct the behaviours of UPFs to mitigate the abnormal behaviours.

- PCF may generate or update PCC policy to filter the abnormal traffic.

- UPF may detect malicious IP packets, adjust interface thresholds, select access type.

Editor's note:	Detail actions will be defined in normative phase.

-  Existing event exposure is enhanced for collecting input data.

-  Appropriate Analytics Filter Information is configured to reduce the load on control plane.

* * * * End of Change * * *