3GPP SA WG2 Meeting #171	S2-2508557

13 - 17 October, 2025, Wuhan, China	(revision of S2-250xxxx)

Source: 	LG Electronics

Title: 	KI#1, Interim Agreements updates

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes to update the interim agreements of KI#1 in TR 23.700-04. Especially it is proposed to focus on CN impacts at this meeting, while RAN related agreements still wait for feedback from RAN WGs.

1	Discussion

Currently, there are some issues as further consideration for KI#1.

While the issues related to UE and RAN are dependent on the feedback from RAN WGs, this paper especially focuses on CN impacts.

-	How the 5GC NF controls UE data transfer.

-	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

-	How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

1.1	5GC NF’s control related UE Data transfer

We can assume the secured user plane connection between UE and DCF, and MNO can explicitly initiate or terminate the data transfer, or provide the validity condition of the data transfer to UE. It implies to support the MNO’s controllability on the data transfer. It can be performed by a standard L3 protocol, in similar to the Location Services User Plane Protocol (LCS-UPP) between UE and LMF.

Proposal 1.

After the secured user plane connection between UE and DCF is established, a standard L3 protocol over the secured user plane connection is used to control UE data transfer.

NOTE:	The specifications on a standard L3 protocol is up to Stage 3 WGs.

1.2	Existing 5GC NF vs. new 5GC NF

In order to ensure uniqueness of feature for data collection and transfer, we propose to specify DCF as new standalone function. However, it is worth noting it can be collocated with LMF depending on network deployment.

Proposal 2.

A new 5GC NF, Data Collection Function, is used.

NOTE:	The DCF can be collocated with other functions (e.g. LMF) depending on network deployment.

1.3	UP connection information (i.e. IP address or FQDN of the 5GC NF)

In principle, the mean for existing LCS secured user plane connection can be adapted.

To trigger the UE to establish a secured user plane connection between the UE and the 5GC NF (i.e. DCF),

- The operator may provide the 5GC NF (i.e. DCF) address information to the UE as part of the URSP rule.

- The 5GC NF (i.e. DCF) shall provide the user plane connection information to the UE, via a DL NAS TRANSPORT message of the AMF.

- The UE can also request the user plane connection information to the 5GC NF (i.e. DCF), in order to obtain it.

Proposal 3.

UP connection information (i.e. IP address or FQDN of the 5GC NF) is sent to UE by DCF via AMF sending DL NAS TRANSPORT message, or by PCF as part of the URSP rule.

NOTE:	It may be configured as UE local configuration by the operator.

1.4	Traffic differentiation

The UE can establish the PDU session with associated DNN and S-NSSAI for the transfer of "UE data collection" traffic. The associated DNN and S-NSSAI can be configured by operator and (if any) SLA.

The QoS flow level granularity requires the further discussion, e.g. the association with specific QoS flow and/or the related configuration in UE and network. Due to lack of time, it is proposed to keep only PDU session level granularity in this release.

Proposal 4.

To differentiate the traffic of collected UE data from the UE regular traffic, only PDU session level is used in this release.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-04.

* * * * Start of 1st Change * * * *

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	A new 5GC NF, Data Collection Function, is used.

NOTE x1:	The DCF can be collocated with other functions (e.g. LMF) depending on network deployment.

-	After the secured user plane connection between UE and DCF is established, a standard L3 protocol over the secured user plane connection is used to control UE data transfer.

NOTE x2:	The specifications on a standard L3 protocol is up to Stage 3 WGs.

-	UP connection information (i.e. IP address or FQDN of the 5GC NF) is sent to UE by DCF via AMF sending DL NAS TRANSPORT message, or by PCF as part of the URSP rule.

NOTE x3:	It may be configured as UE local configuration by the operator.

* * * * Start of 2nd Change * * * *

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether and how the UE controls UE data transfer.

-	How the configuration information for UE data transfer is sent to the UE.

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

-	Whether and how to select UEs for UE data collection.

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

* * * * End of Changes * * * *