SA WG2 Meeting #171	S2-2508604

13 – 17 October, 2025, Wuhan, CN

Source:	Apple

Title:	Interim Agreement Proposals for FS_Sensing_ARC

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes Interim Agreement for FS_Sensing_ARC.

# 1. Discussion

This contribution proposes Interim Agreement for FS_Sensing_ARC.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all new text) * * * *

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

Figure 7.1.1-1: Reference Architecture

The following interim principles apply for the system architecture to support sensing:

-	A new 5GC network function, the Sensing Function (SF), is introduced to provide support for sensing service.

-	The SF may be functionally split into a:

-	Sensing Control Function (SCF), responsible for control plane aspects including authorization, validation, configuration of sensing entities (SE) and selection and configuration of Sensing Processing Functions (SPF).

-	and a Sensing Processing Function (SPF), responsible for data processing and result generation based on data collected from the SE, i.e. gNB.

-	The SF is responsible for key functionalities, including:

-	Receiving and processing sensing service requests

-	Authorizing sensing service requests

-	Discovering and selecting Sensing Entities (SE)

-	Configuring the SEs for sensing operations

-	Generating and exposing the sensing result

-	The Network Exposure Function (NEF) serves as the entry point for sensing service requests from untrusted Application Functions (AFs).

-	In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE 1:	Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 conclusion.

NOTE 2:	No interface between SCF and SPF is specified as part of this study.

### 7.1.2	Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service

The following interim principles apply for the authorization and revocation to support sensing service:

-	Authorization is performed at two levels:

-	The service consumer (i.e. AF) is authorized by the 5GC to request sensing services. If the AF is untrusted, the NEF performs the authorization, and if the AF is trusted, the SCF performs the authorization.

-	The specific sensing service request (e.g. based on sensing type, target area, duration) is authorized by the SCF.

-	Authorization policies are based on operator configuration, service level agreements, and subscriber data, which may be stored in the UDM/UDR

-	The network shall support the revocation of an active sensing service, which can be initiated by either the service consumer (AF) or the network (e.g. SCF or NEF)

### 7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following interim principles apply for the discovery and (re-)selection of sensing entity and sensing function:

-	The discovery and selection of the appropriate SCF to serve a request is the responsibility of

-	the NEF for untrusted AFs, using the NRF if needed.

-	the trusted AF itself, using the NRF if needed

-	The discovery and selection of the appropriate SE(s) (i.e. gNBs) is the responsibility of the SCF. The gNB registers its capabilities in the SCF.

-	SE selection is based on criteria from the service request. The selection criteria must include the sensing area, and optionally sensing type and QoS requirements, matched against the registered capabilities of the SE.

-	The discovery and selection of the appropriate SPF(s) is the responsibility of the SCF, using the NRF if needed.

### 7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following interim principles apply for the collection and transport of the sensing data and associated information collection:

-	Sensing data is transported from the SE (i.e. gNB) to the SF. If the functional split is used, sensing data is transported directly from the SE (i.e. gNB) to the SPF.

-	The transport of sensing data can be achieved via:

-	A direct data connection (e.g. a data tunnel) between the gNB and the SPF for efficient transport of large data volumes.

-	The SCF determines the transport method and provides the necessary connection information to the SE (i.e. gNB).

-	The SCF configures the processing of the sensing data at the relevant SPF(s) in the 5GC, which may exchange intermediate processed data before final result is calculated.

7.1.5	Agreed Principles for KI#5: Sensing Result Exposure

The following interim principles apply for the exposure of sensing result:

-	The SCF is responsible for exposing the final sensing result to the authorized sensing service consumer (AF).

-	If the functional split is used, it is an option, that the SCF configures the SPF to deliver the result to the authorized sensing service consumer (AF) directly.

-	For untrusted AFs, the sensing result is always exposed via the NEF.

-	The result exposure mechanism supports one-time (on-demand), periodic, and event-triggered reporting.

### 7.1.6	Agreed Principles for KI#6: Configuration of Parameters for Sensing Entities

The following interim principles apply for the configuration of parameters for sensing entities:

-	The SCF is responsible for providing the operational configuration parameters to the selected SE (i.e. gNB).

-	Configuration parameters are derived from the AF's service request and the SF's internal logic.

-	The delivery of configuration parameters from the SCF to the SE is performed directly from SCF to SE.

# * * * * End of changes * * * *