3GPP TSG-SA WG2 Meeting #171	S2-2508681

Wuhan, China, 13 – 17 October 2025

Source:	Ericsson

Title:	KI#3, New Solution: SMS over IP delivery to Emergency Response Centre with emergency numbers

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution provides a solution for SMS over NAS delivery to Emergency Response Centre KI#3.

# 1.	 Proposal

It is proposed to update TR 23.700-65 with below changes:

* * * Start of change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| #x |  |  | x |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

* * * Next change  (all new)* * * *

## 6.X	Solution #X: SMS over IP delivery to Emergency Response Centre based on emergency call numbers

### 6.X.0	High level principles

This solution aims to address KI#3 with the following key principles:

-	The UE uses IMS PDU session to support the SMS over IP delivery to Emergency Response Centre (ERC) in case of non-roaming.

-	The IP-SM-GW is responsible to deliver the SMS to the SMS service centre based on the identification of the SM service centre/emergency number.

-	The serving SMS service centre is responsible to forward the SMS to the local Emergency Response Centre.

### 6.X.1	Description

In this proposal, it is assumed that IMS PDU session is used to deliver the SMS over IP to Emergency Response Centre when UE is not roaming. It is also assumed that UE is aware of the supporting capability of the network based on the solutions provided for KI#1 (e.g. UE is informed that SMS over IP delivery to Emergency Response Centre is supported by the network during IMS registration).

The main change is IP-SM-GW deliver the SMS to the SMS service centre based on the RP-DA field.

### 6.X.2	MO SMS over IP delivery procedures

The following procedures are based on TS 23.204 [8].

#### 6.x.2.1	Transport-level interworking: Successful encapsulated Short Message origination procedure with MSISDN

Figure 6.3: Successful encapsulated Short Message origination procedure

1)	The UE registers to S-CSCF according the IMS registration procedure. Note that I-CSCF and P-CSCF are not shown in this figure.

2)	UE submits the encapsulated Short Message (SMS-SUBMIT, SC Address) to the S-CSCF using an appropriate SIP method. When UE builds the SMS message to be sent to Emergency Response Centre, the RP-DA field includes the SMS service centre address/emergency service number when network provided the “+g.3gpp.smsip-delivery to ERC” indication during IMS registration procedure as described in clause 6.x.2.1.

3)	S-CSCF forwards the encapsulated Short Message (SMS- SUBMIT, SC Address) to IP-SM-GW (AS) based on stored iFC.

4)	IP-SM-GW (AS) acknowledges the SIP message.

5)	SIP message acknowledge is forwarded by S-CSCF to UE.

6)	The IP-SM-GW performs service authorization based on the stored subscriber data as described in the clause 6.1. The IP-SM-GW shall check whether the subscriber is authorised to use the short message service (e.g. Operator Determined Barring settings), similar to the authorization performed by MSC/SGSN in case the Short Message is delivered via CS or PS domain. In addition, the IP-SM-GW shall also check whether the user is authorised to use the encapsulated Short Message delivery via IMS. If the result of service authorization is negative, the IP-SM-GW shall not forward the message, and shall return the appropriate error information to the UE in a failure report. Otherwise, the IP-SM-GW (AS) extracts the Short Message (SMS- SUBMIT) and forwards it towards the SMS-SC (SC Address) via the SMS-IWMSC using standard MAP or Diameter based (SGd/Gdd) signalling (as described in TS 23.040 [2]). If the RP-DA includes the serving network SMS service centre address/emergency service number, the IP-SM-GW delivers the SMS to the SMS service centre of the serving PLMN. The SMS service centre delivers the SMS to corresponding Emergency Response Centre based on emergency numbers.

7)	The SMS-IWMSC forwards the Short Message (SMS-SUBMIT) to the SMS-SC (see TS 23.040 [2]).

8)	SMS-SC sends a Submit report (SMS-SUBMIT-REPORT) to SMS-IWMSC (see TS 23.040 [2]).

9)	SMS-IWMSC sends the Submit report to IP-SM-GW (AS) (see TS 23.040 [2]).

10)	IP-SM-GW (AS) sends the Submit report to S-CSCF, encapsulated in an appropriate SIP request.

11)	The S-CSCF sends the Submit report to the UE.

12)	The UE acknowledges the SIP request.

13)	The S-CSCF forwards the acknowledgement of the SIP request to IP-SM-GW (AS).

Editor's note:	roaming case is FFS.

### 6.X.3	Impacts to Services, Entities and Interfaces

UE:

-	Support the handling of SMS over IP delivery to Emergency Response Centre during IMS registration procedure, e.g., support of “+g.3gpp.smsip-delivery to ERC” field header handling.

-	Formulates the RP-DA in the RP-DATA with serving PLMN SMS service centre address/emergency number.

S-CSCF:

-	Support the “+g.3gpp.smsip-delivery to ERC” field header handling during registration procedure.

IP-SM-GW:

-	Support of SMS delivery to SMS service centre of the serving PLMN based on SMS service centre address/emergency number.

-	support the “+g.3gpp.smsip-delivery to ERC” field header handling during IMS registration.

* * * End of change * * * *