3GPP TSG-WG SA2 Meeting #171	S2-2508760

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2507677)

Source:	Huawei, HiSilicon

Title:	WT#3: conclusion and agreement

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: A summary of what this is about. Don’t start it with “This contribution…”

# 1. Introduction/Discussion

In last meeting we discussed KI#3 conclusion, but the document S2-2507677 has been noted. This document indend to proposed conclusions.

There was several proposals to consider the renewable energy as parameter in the NF profile, but it needs to be further clarified the kind of information to be included. there are the following options:

- An indication whether the NF is powered with renewable energy, e.g. when it is present, it indicates that the NF is powered with renewable energy. This information is not time variant.

- An indication of the renewable energy ratio, e.g. when it is present, it indicates that the NF is powered with renewable energy and the amount of energy which is generated with renewable energy. This information is not time variant.

- An indication of the renewable energy ratio with associate scheduling, e.g. when it is present, it indicates that the NF is powered with renewable energy and the amount of energy which is generated with renewable energy with the indication of when a given ratio applies.

All above options require that the SA5 has defined in Rel-20 the support of renewable energy and in which format. The SA5 energy study in Rel-20 in SP-251025 has the scope to improve the KPIs, therefore whether this information and which format is defined needs to be consolidate based on SA5 outcome. It is proposed to consider in the SA2 conclusion conditionally to SA5 outcome and to align during normative phase.

Regarding the states The SA5 energy study in Rel-20 in SP-251025 has the WT-1.3 with the scope of “Study enhancements to 3GPP management system to support different energy states of Network Elements and Network Functions” therefore whether they can be supported or not will depend by SA5. From SA2 point of view we need to define more precisly the definition and the usage.

# * * * * First change * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below. This is a feature requiring the software to be upgraded in NRF and its consumers.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| Energy Priority Information | Value of Energy priority relative to other NFs of the same type considering the energy characteristics of the NF. |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present |
| Renewable Energy support | The value indicates whether the NF is powered with renewable energy and it is configured by OAM. Configure supported/not supported or ratio but not time schedule concept here depedning by SA5. Note 2: |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. NOTE 2: The support of this parameters and the format (e.g. whether it is an indication of support or a renewable energy ratio) needs to be confirmed by SA5 during SA2 normative phase. |  |

- The Energy Priority Information is defined in solution #20 clause 6.20.1.

-	OAM provisions and updates as needed the information within the NRF profile. The above information are not known by the NF itself.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-  The schedule infomation may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection subject to SSC mode of the PDU sessions.

# * * * * End of changes * * * *