3GPP TSG-WG SA2 Meeting #171	S2-2508775

13th – 17th October, 2025, Wuhan, China	(revision of S2-250xxxx)

Source: 	Lenovo

Title: 	KI#3 Interim Agreements

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes interim agreements for KI#3.

# 1	Discussion

The KI#3 is about Sensing Entity and Sensing Function Discovery and (Re-)Selection. There are currently 10 solutions documented in the TR 23.700-14 V1.0.0: #1, #3, #6, #13, #14, #15, #16, #17, #35, #36. We propose to discuss the Sensing Entity aspect and the Sensing Function separately.

Regrading the Sensing Function (SF), it has to be considered that the SF can be deployed in hierarchical way, e.g. having a SF gateway (SF-GW) and distributed SFs. Apparently, the NEF needs to select the SF-GW for a particular incoming sensing service request. The following options exist:

-	the NEF can have a local configuration and select the SF based on target sensing area. This approach requires a local configuration and is not flexible enough.

-	the NEF can use the NRF services. It is required that the SF registers with the NRF and the NF profile should store the target sensing area sensing capabilities (e.g. whether SF-GW functionality is supported).

-	The SF supported sensing area is the sum of the supported sensing area of the SEs which have established association with the SF.

-	In addition, the NF profile may include the supported sensing service type (e.g. motion monitoring, object detection and tracking).

Proposal 1: it is proposed to agree that the SF discovery and selection based on NRF services. The SF NF profile at the NRF can store: target sensing area (e.g. to select distributed SF); NF capability (e.g. SF-GW supports authorization and others like sensing type).

Regrading the RAN Sensing Entity (SE) section, it depends whether direct connectivity architecture between the SE and SF is assumed, or also indirect connectivity architecture via AMF needs to be supported. For any of the connectivity options, the SF can select the RAN SE using one of the following options:

-	The SF can store local configuration with available RAN SEs and their supported sensing area.

-	The RAN SE may establish an association with one or more SF(s) and during the association establishment, the RAN SE can indicate the supported sensing area.

Please note that it is up to RAN3 decision how RAN SE can select an SF to establish an association with.

Proposal 2: it is proposed to agree that the RAN SE establishes an association with one or more SFs. This is applicable for both direct and indirect connectivity. The RAN SE indicates its supported sensing area to the SF.

There may be multiple RAN SEs connected to a SF. The next question is how does the SF select the appropriate SE(s)? We consider that there may be RAN SEs with overlapping areas, e.g. RAN nodes operating in different frequency bands. When the SF receives a sensing service request, the SF may select and send the request all RAN SEs which are within the target sensing area. In order to avoid overlapping use of resources, the SF may perform a discovery procedure to discover the best suitable RAN SE for the specific sensing service request.

Proposal 3: The SF may perform a RAN SE discovery procedure for a specific sensing service request. The SF sends SE discovery request including a sensing service description and the SE replies with the confidence level to perform the sensing service.

Proposal 4: The SF selects one or more RAN SEs for a sensing service. The SF uses the following parameters for the SE selection: mapping of requested sensing area to the SE supported sensing area, confidence level of sensing service accuracy.

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

### 7.1.X	Agreed Principles for KI#3

The following interim agreements on KI#3 “Sensing Entity and Sensing Function Discovery and (Re-)Selection” are concluded:

-	Both direct connectivity between SE-SF and indirect connectivity (i.e. SE-AMF-SF) are supported. In case of indirect connectivity, the AMF transparently forwards the association establishment request from the SE to the SF.

-	For SF discovery and selection, the following applies:

-	The SF registers its NF profile in the NRF that includes supported sensing area and sensing capabilities.

-	The SF supported sensing area is the sum of the supported sensing area of the SEs which have established association with the SF.

-	The SF supported sensing capabilities may include gateway SF functionality or distributed SF functionality.

-	The SF may register gateway SCF functionality and/or distributed SCF functionality.

-	The NEF performs the SF discovery and selection (e.g. gateway SF functionality) based on the target sensing area and sensing service type received sensing service request.

-	The NEF queries the NRF to discover and select an appropriate gateway SF functionality.

-	The gateway SF functionality may select one or more distributed SF functionalities for a specific sensing service.

-	For SE discovery and selection, the following applies:

-	The SE establishes an association with one or more SFs. During the association establishment the SE indicates its SE ID, supported sensing area and supported capabilities.

-	The SF may perform a SE discovery procedure for a specific sensing service request. The SF sends SE discovery request including a sensing service description and the SE replies with a pre-assessed confidence level to perform the sensing service.

-	The SF selects one or more SEs for a sensing service. The SF uses the following parameters for the SE selection: mapping of requested sensing area to the SE supported sensing area, pre-assessed confidence level of SE for sensing service accuracy.

# * * * * End of change* * * *