3GPP TSG-WG2 Meeting #171	S2-2508853

October 13 – 17, 2025, Wuhan, China	(revision of S2-250xxxx)

Source:	Ericsson

Title:	K#5, Sol#28 update: Describing the Target Sensing Area concept

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution:

This contribution updates Solution#28 to address the EN on a description of the Target Sensing Area information that are used in the context of sensing service requests. This update is required also in order to further initiate the coordination with RAN on defining a sensing area of a particular NG-RAN node. The contribution also proposed to include the respective definitions among the Terms.

# 1. Discussion

This contribution updates Solution#28 to address the EN on a description of the Target Sensing Area information that are used in the context of sensing service requests. This update is required also in order to further initiate the coordination with RAN on defining a sensing area of a particular NG-RAN node. The contribution also proposed to include the respective definitions (i.e. External Target Sensing Area, Target Sensing Area, SnCF Serving Sensing Area, NG-RAN Serving Sensing Area) among the Terms.

# 2. Text Proposal

It is proposed to agree and include the following solution into 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Sensing Service Consumer: The entity that may consume the Sensing Result. The Sensing Service Consumer may also request the Sensing Result.

Sensing Entity: The Sensing Entity referring to a Sensing Transmitter and/or to a Sensing Receiver.

NOTE 1:	In this release, the Sensing Entity is only referring to gNB.

Sensing Function: Indicating the logical function which is involved to support Sensing Service.

NOTE 2:	The Sensing Function cannot be a Sensing Entity.

Sensing Service: Capability to collect and provide information about object and/or characteristics of the environment using radio signals.

gNB-based Sensing: it indicates gNB acting as Sensing Entity. This doesn't imply any assumptions for the location of the sensing result generation.

External Target Sensing Area: An area used between an NEF and a third-party AF in sensing service operations; the area is identified by a geographic location using, e.g., GAD shapes such as polygon, see TS 23.032 [11].

Target Sensing Area: An internal area of a CSP within which a sensing task is requested.

NG-RAN Serving Sensing Area: An area where NG-RAN can perform sensing measurements as defined in TS 38.XXX [TBD].

Editor's note:	The definition of NG-RAN Serving Sensing Areas is FFS and reference to RAN specification needs to be updated based on RAN inputs.

SnCF Serving Sensing Area: A geographical area, within which a specific Sensing Core Function instance (SnCF) can perform sensing tasks.

The terms defined in TS 22.137 [2] apply to this study:

-	Sensing assistance information.

-	Sensing contextual information.

-	Sensing result.

-	Target sensing service area.

-	Sensing receiver.

-	Sensing transmitter.

Editor's note:	The terminology will be further revised during the study work, if needed.

>>>> Next Change <<<<

### 6.28.4	Procedures

Procedure for sensing result exposure in cases when a sensing service consumer (e.g. AF) invokes the sensing service towards a core network is shown in Figure 6.28.4-1.

Figure 6.28.4-1: Procedure of sensing result exposure to an AF

1.	The Sensing service consumer (e.g. AF) invokes a sensing service request (e.g. Nnef_SensingEvent_Subscribe) to the NEF. Inside the request the AF includes the following mandatory and optional parameters:

-	Mandatory: AF Identifier, External Target Sensing Area information (e.g. geographical area information and, optionally, altitude information), Type of sensing to perform (e.g. SensingEventType = "Object Detection", "Object Tracking"), sensing result detail level (e.g. "notify result only", "notify result with associated information").

-	Optional: event reporting mode (e.g. one-time, event-triggered or periodic reporting), sensing result exposure parameters (e.g. time and/or spatial resolution of the determined sensing result).

NOTE 1:	Other parameters that AF can include in a sensing service request can be added during the later phases.

NOTE 2:	The exact names of service operations and semantics can be determined in the later phase.

2.	The NEF performs the authorization of the AF for the sensing service.

NOTE 3:	Data used for the AF-requested sensing service authorization and other details on how the NEF authorizes the AF requesting the sensing service is outcome of KI#2.

Editor's note:	Whether the NEF or other core network NF (e.g. SnCF) needs to determine that the AF is allowed to perform the sensing in the requested Target Sensing Area is FFS and needs coordination with SA WG3.

If the AF request for sensing service is authorized, the NEF maps the External Target Sensing Area information in the AF request to a Target Sensing Area information and then discovers and selects an SnCF to handle the sensing service request.

The handling of area information for sensing services is described in clause 6.28.5.

NOTE 4:	How the NEF selects an SnCF is outcome of KI#3.

3.	The NEF invokes the corresponding sensing service request (e.g. Nsncf_SensingEvent_Subscribe) to the selected SnCF and provides the relevant sensing service parameters from the AF request in step 1.

4.	The SnCF selects NG-RAN node(s) for the relevant Target Sensing Area of the authorized sensing service.

NOTE 5:	How the SnCF selects NG-RAN node(s) to handle the AF-requested sensing service is covered by solutions for KI#3 of this study.

NOTE 6:	How the SnCF authorizes the sensing service operation, if applicable, is the outcome of KI#2 of this study.

5.	The SnCF sends the sensing service request to the selected NG-RAN node(s). Inside the request, the SnCF includes, for instance, Target Sensing Area information, type of sensing to perform, relevant parameters from the AF request in step 1, and other parameters derived by the SnCF for the authorized sensing service.

6.	Based on the received sensing service request from the SnCF, the NG-RAN node(s) starts obtaining sensing data.

NOTE 7:	How sensing measurements are performed based on the SnCF-provided sensing request parameters and how the NG-RAN node(s) obtain the sensing data are RAN WGs decisions. For instance, if the SnCF includes in the request the following parameters: SensingEventType = "Object Detection" and Event reporting mode = "periodic reporting", the NG-RAN starts performing sensing measurements with determined periodicity (continuously) and required processing in order to provide (periodically or continuously) the obtained sensing data to the SnCF for a sensing result determination and exposure towards the sensing service consumer. However, if SensingEventType = "Object Detection" and Event reporting mode = "one-time reporting", the NG-RAN makes the required sensing measurement and then provides (once) the obtained and processed sensing data to the SnCF.

7.	The NG-RAN node(s) apply processing to the obtained sensing measurements before providing the sensing data to the SnCF,

NOTE 8:	The form of sensing data provided by NG-RAN depends on level processing the NG-RAN applies to the sensing measurements and, thus, may include the determined result or pre-processed sensing data for subsequent processing at the SnCF for instance.

Editor's note:	Whether NG-RAN nodes can perform a level of processing required to determine a sensing result (e.g. to detect an object) is RAN WGs decision and requires further coordination.

8.	The NG-RAN node(s) provides the sensing data (i.e. sensing measurements processed by the NG-RAN) to the SnCF.

NOTE 9:	How the NG-RAN node(s) provides the obtained sensing data to the SnCF is covered by KI#4 of this study.

9.	The SnCF may determine a sensing result by processing, if required, to the sensing data received from the NG-RAN node(s) and considering the parameters included in the AF request in step 1.

NOTE 10:	How the SnCF uses the sensing data provided by the NG-RAN to determine a sensing result is up to implementation; the SnCF can cache the sensing data. However, sensing data processing/handling is limited to information provided by RAN.

10.	The SnCF provides the determined sensing result to the NEF using e.g. the Nsncf_SensingEvent_Notify service operation.

11.	The NEF exposes the sensing result to the AF via e.g. Nnef_SensingEvent_Notify service operation, considering the sensing result exposure parameters from the AF request in step 1.

>>>> Next Change <<<<

### 6.28.5	Handling areas for sensing

The following areas are used within the sensing context:

-	External Target Sensing Area: An area used between an NEF and a third-party AF in sensing service operations; the area is identified by a geographic location using, e.g., GAD shapes such as polygon, see TS 23.032 [11].

-	Target Sensing Area: An internal area of a CSP within which a sensing task is requested.

-	NG-RAN Serving Sensing Area: An area where NG-RAN can perform sensing measurements as defined in TS 38.XXX [TBD]

Editor's note:	The definition of NG-RAN Serving Sensing Areas is FFS and reference to RAN specification needs to be updated based on RAN inputs.

-	SnCF Serving Sensing Area: A geographical area, within which a specific Sensing Core Function instance (SnCF) can perform sensing task.

The use of the aforementioned areas in the context of the sensing services is shown in Figure 6.28.5-1.

NEF

SnCF

Sensing service consumer (e.g. AF)

NG-RAN

1. External Target Sensing Area

2. Mapping the External Target Sensing Area to a Target Sensing Area

3. Discovering the relevant SnCF via NRF based on SnCF Serving Sensing Areas and the Target Sensing Area

4. Sensing service request containing the Target Sensing Area

5. Selecting the relevant NG-RAN based on NG-RAN Serving Sensing Areas for NG-RAN nodes associated with the SnCF

6. Sensing service request containing the Target Sensing Area

7. Performing the sensing measurements relevant for the provided Target Sensing Area

Figure 6.28.5-1: Area handling for sensing services.

Procedure in Figure 6.28.5-1 includes the following handling of area information for sensing services:

1.	A third-party AF provides an External Target Sensing Area to the NEF inside the sensing service request.

2-3.	The NEF maps the External Target Sensing Area to a Target Sensing Area, which is then used to discover an SnCF for handling the sensing service request. For the SnCF discovery, the NEF can use the NRF (based on the information provided during the SnCF registration with NRF).

4.	The NEF sends the corresponding sensing service request to the selected SnCF including the Target Sensing Area.

5.	The SnCF selects the NG-RAN node based on the received Target Sensing Area and NG-RAN Serving Sensing Area information.

6.	The SnCF sends the corresponding sensing service request to the selected NG-RAN node including the Target Sensing Area.

7.	Based on the received sensing service request including the Target Sensing Area, the NG-RAN performs the relevant sensing measurements.

>>>> End of Changes <<<<