3GPP TSG-WG SA2 Meeting #171	S2-2508880

13-17 October, 2025, Wuhan , China

Source:	China Mobile

Title:	[KI#2, New Solution] DO-A Capable AIoT Device Registration Procedure

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Solution for Registration Procedure for the DO-A Capable AIoT device.

1. Introduction/Discussion

In the WT in Study on Architecture support of FS_AmbientIoT_Ph2, the following is the description for the registration procedure for a DO-A capable AIoT device:

WT#2: Study the support of DO-A Capable AIoT Devices, including: - Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure). - Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF. - Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF. NOTE 4: topology 2 aspect of WT#2 has dependency on WT#1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new text) * * * *

6.X	Solution #X: DO-A capable AIoT Device Registration Procedure

6.X.0	High-level solution Principles

DO-A capable AIoT devices possess the capability to transmit MO signalling and data. Furthermore, the AIoT device actively registers with the core network to notify it of its presence autonomously.

6.X.1	Description

This solution addresses KI#2, specifically concerning the method by which the DO-A capable AIoT device can autonomously notify the network of its presence for Topology 1.

In this solution, it is assumed:

The AIoT device actively initiates the registration process with the network. The registration types include:

- initial registration.

- mobility registration update.

- Periodic registration.

ADM stores the DO-A capable AIoT device profile information.

AIOTF handles NAS termination and device authentication for DO-A capable AIoT devices and manages registration and mobility for AIoT Devices.

Taking the deployment into consideration, the connection between the NG RAN and the AIOTF is via the AMF for outdoor scenarios.

6.X.2	Procedures

Figure 6.X.2.1: DO-A capable AIoT Device Registration Procedure

- The DO-A capable AIoT Device’s subscription data is provisioned in the ADM

Editor’s Note: The subscription data of a DO-A capable AIoT device is FFS.

- The DO-A capable AIoT Device meets the following trigger conditions, then it will send a NAS Registration Request to the core network.

The AIoT Device performs an Initial Registration if it is not registered with the network.

The AIoT Device performs Mobility Registration if it has moved outside of its AIoT Registration Area.

The AIoT Device performs periodic registration if it has registered on the network and remains in its AIoT Registration Area beyond the designated time period.

Editor’s Note: The details of the AIoT Registration Area need to align with the RAN WG

2.	AIoT Device sends a message to NG-RAN, including a NAS Registration Request (Registration Type, Temporary ID, device information, security parameters). Registration can be initial registration, mobility registration, or periodic registration. Then, NG-RAN sends the Registration Request to the selected AMF.

3.  The AMF can discover the serving AIOTF based on the following methods.

Method 1: AMF queries the local configuration to obtain the serving AIOTF.

Method 2: AMF queries NRF to obtain the serving AIOTF.

Method 3: AIOTF uses a subscribe/notification mechanism to the AMF for registration-related activities of DO-A capable AIOTF devices and provides the serving AIOTF information to the AMF.

4.  The AMF forwards the registration request to the serving AIOTF.

5.	[Conditional] The serving AIOTF determines the old AIOTF using the temporary ID and retrieves the AIoT Device context from the old AIOTF.

6.	The AIOTF performs the Authentication/Security procedure and stores the security context.

Editor’s Note: Details of authentication/security procedures will be determined by the SA3 WG.

7.	[Conditional] The serving AIOTF may perform the black/white check with 5G-EIR based on TID

8.	The AIOTF sends the Registration Accept to the AIoT device via AMF, including a new temporary ID assigned by the serving AIOTF, AIOT Registration Area, and periodical registration timer.

9.	The AMF forwards the Registration Accept to the AIoT device via NG RAN.

10. The AIoT device responds to the AIOTF with Registration Complete via NG RAN and AMF.

11. The AMF forwards the Registration complete to the AIOTF.

12. The AIOTF notifies the ADM of the registration complete status about the AIoT device.

13. The ADM stores the serving AIOTF for the AIoT device.

14. [Conditional] If the ADM discovers that the serving AIOTF has changed, then it needs to initiate the context release procedure to the old AIOTF to delete the context in it.

6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Support Registration and Mobility Management.

AMF:

-	Supports AIOTF selection and transferring NAS messages between the AIOTF and NG RAN.

NG-RAN:

-	Supports AMF selection and transferring NAS messages to and from the AIoT Device.

AIoT Device:

-	Support performing the registration procedure.

ADM:

-	Support new interface with AIOTF.

# * * * * End of changes * * * *