3GPP TSG-SA2 Meeting #171	S2-2508881

# Wuhan, China, October 13th – 17th, 2025		(revision of S2-2506338)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1655 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Cancel procedure for subscriber specific IMS Events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | After successful creation of a subscription for a subscriber specific IMS Event Exposure, the HSS may need to cancel the subscription due to the following reasons: - UE subscription (e.g. IMS Public Identity) associated to the created Event Exposure Subscription is withdrawn by the operator. - AF Authorization for the particular UE Subscription (IMPU) is revoked. This situations in HSS, require that HSS informs the NEF that the subscription for the subscriber specific IMS Event is cancelled. This is not yet covered in TS 23.228. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Network-initiated explicit event notification subscription cancel procedure for subscriber specific IMS events |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF will not be informed about the subscription was cancelled consuming resources waiting to be notified about events which will never occur for a particular UE subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.1.1, AA.2.1.5.X (new), AD.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Appropriate CRs have been submitted to stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF, IMS AS |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |
|  | Notify |  |  |

# *** Next Change ***

#### AA.2.1.5.X	Nhss_ImsEE_Notify service operation

Service operation name: Nhss_ImsEE_Notify

Description: The HSS notifies the NF consumer that a subscription for a specific subscriber related IMS event is removed (e.g. due to withdrawal of the IMS Public Subscriber identity).

Inputs, Required: Event ID(s), Notification Correlation Information.

Inputs, Optional:  Event Removal Indication.

Outputs, Required: Operation execution result indication.

# *** Next Change ***

## AD.3.X	HSS-initiated notification subscription cancel procedure for subscriber specific IMS events

The procedure is used by the HSS to delete a subscription to a subscriber specific IMS event.

Figure AD.3.X-1: HSS-initiated notification subscription cancel procedure for subscriber specific IMS event

0.	An IMS Event Exposure Subscription procedure according to clause AD.3.2 has already executed successfully.

1.	The UE/IMPU subscription is withdrawn or AF authorisation for the IMPU is revoked in HSS.

2.	If the AF authozisation for the IMPU is revoked, the HSS unsubscribes to the subscriber specific IMS event in the IMS AS by sending a Nimsas_ImsEE_Unsubscribe service operation.

NOTE: If the UE/IMPU subscription is withdrawn, the IMS user is deregistered from the IMS and the IMS AS is assumed to remove the subscription to the IMS event

3.	The HSS notifies the NEF that the subscription to the subscriber specific IMS event for the IMPU has been removed or the AF authorisation has been revoked by sending a Nhss_ImsEventExposure_Notify service operation.

4.  The NEF notifies the NEF that the subscription to the subscriber specific IMS event for the IMPU has been removed by sending a Nnef_ImsEventExposure_Notify service operation.

If the NEF determines that the reporting associated to the AF subscription is complete, the NEF deletes the associated event notification subscription.

# *** End of Changes ***