3GPP TSG-SA2 Meeting #171	S2-2508888

Wuhan, China, 13 – 17 October 2025	(revision of S2-2506408)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6353 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Energy Consumption Information collection from SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Data volume collection from UPF using N4 session update is not working in all cases. The starting time of the measurements is not aligned with the rest of the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SMF subscribes to Nupf_EventExposure for Data volume information instead of sending and N4 Session Update which is complex, incompatible with optimizations like predefined rules, and not guaranteed to deliver what EIF needs (see DP S2-2508887). The Energy Consumption information reported to the EIF shall be time-aligned. So, not only the period duration but the period start times are relevant. The periodic Volume measurements and reporting can be requested to UPF with network-wide configured starting time and interval T for calculation of energy consumption (see TS 23.501 clause 5.51). UPF can be requested the measurement at data path change. For MA PDU Sessions, UPF can be requested the measurement per access. For Service Data Flow granularity, Application Id or Flow Info (packet filters) can be received in SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Energy Consumption Information collection from SMF not working in all cases as described in DP S2-2508887 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5523 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | reference clause number 4.15.4.5.x to be updated from CR 5523 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Enhnaced with Report Request at data path change and per access reports, |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.51.2.2.2	Energy Consumption information collection from SMF

The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and optional S-NSSAI or (S-NSSAI, DNN). Also, the EIF subscribes to the UDM to be notified with the applicable SMFs.

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Flow Info (i.e. Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1. The SMF receives the data volume of the required granularities from the PSA UPF. The SMF derives the gNB ID(s) from the AMF provided ULI as described in clause 4.3.2. If the I-SMF is involved in the PDU session, the I-SMF reports to the SMF of PDU session via Nsmf_PDUSession_Update the current I-UPF ID and subsequent change of the I-UPF ID(s). If the I-SMF is involved in the PDU session, the SMF of PDU session requests the I-SMF via Nsmf_PDUSession_Update to report the current ULI and subsequent change of ULI only when the gNB ID changes.

The SMF then sends the collected data volume, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. The information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2. Upon release of a PDU session, the SMF terminates subscriptions related to the PDU session. The SMF periodically notifies EIF with the information at the end of each time interval T until the EIF unsubscribes as defined in TS 23.502 [3]. The SMF shall collect data volume only for the PDU session carried over the 3GPP Access in 5GS. When the PDU session is over Non-3GPP Access type, or it is a PDN connection over EPS handled at a SMF+PGW-C, the SMF reports UL/DL Data volume in the measurement period equal to 0 (zero) and the gNB ID is not provided.

NOTE 1:	The SMF can derive gNB ID(s) according to the AMF provided ULI as described in clause 4.3.2 of TS 23.502 [3].

In the case of MA PDU session, the SMF shall not include in the report the UL/DL Data volume related to the traffic over the Non-3GPP access leg and the serving gNB ID refers to the traffic over the 3GPP leg.

When the SMF receives the Nsmf_EventExposure_Subscribe for energy consumption, the SMF determines the PDU Session, and it sends Nupf_EventExposure_Subscribe to PSA UPF for User Data Usage Measures event to request volume measurements (see clause 5.2.26.2 of TS 23.502 [3]). The subscription includes applications Id or Flow Info if received. In the case of MA PDU session, the SMF shall request the measurement per access. The subscription indicates periodic reporting with measurements taken at the network-wide configured starting time and interval T. At the end of each measurement time interval, UPF generates event reports that it sends to SMF in Nupf_EventExposure_Notify.

If there is a change in the user data path (e.g. handover between gNBs or insertion/removal of I-UPF) during the measurement time interval, at that moment SMF requests to UPF a report for the subscribed event (see clause 4.15.4.5.x in TS 23.502 [3]).

The SMF shall store the gNB ID and I-UPF ID that belong to the user plane path before the change happened.

Table 5.51.2.2.2-1: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Application Id or Flow Info | Information shall be reported with granularity of service data flow (traffic matching this filter). |

NOTE 2:	The user-plane energy consumption information reporting interval from the SMFs is the network-wide configurable starting time and interval T.

Table 5.51.2.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in clause 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > UL/DL Data Volume | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > gNB ID | Identifier of the gNB serving the UE. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g. time stamps). |

NOTE 3:	Each entry of List of Data Volume information represents the serving gNB and UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the gNB or UPF(s) is changed.

# * * * * End of changes * * * *