3GPP TSG-SA WG2 Meeting #171	S2-2508990

13 – 17 Oct, 2025, Wuhan, China										                    (was S2-2506464)

Source:	MediaTek Inc.

Title:	KI#3: New Sol: Support for eCall over NB-IoT NTN via GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: Solution to address the support for eCall over NB-IoT NTN..

1.	Background & Discussion

Per TR 23.700-19, the KI#3 studies how to support IMS emergency call defined in TS 22.101:

5.3	Key Issue #3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC

This key issue studies:

-	How to support IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC to comply with the relevant requirements defined in TS 22.261 [8] and TS 22.101 [9].

In TS 22.101, eCall is specified to be supported, see:

3.1	Definitions: …

eCall:	A manually or automatically initiated emergency call (TS12 or IMS emergency call), from a vehicle, supplemented with a minimum set of emergency related data (MSD)…

MSD: The Minimum Set of Data [46] forming the data component of an eCall sent from a vehicle to a Public Safety Answering Point or other designated emergency call centre. The MSD has a maximum size of 140 bytes and includes, for example, vehicle identity, location information and time-stamp.

10	Emergency Calls, 10.1	General requirements:…It shall be possible to initiate emergency calls to different emergency call centres, depending on the type of emergency. The following types of emergency calls shall be possible:

-	Police

-	Ambulance

-	Fire Brigade

-	Marine Guard

-	Mountain Rescue

-	Manually Initiated eCall (MIeC)

-	Automatically Initiated eCall (AIeC)

-	Spare

22.4	IMS Services: The set of IMS services supported by ICS shall include at least the following, subject to the constraints of the UE and access networks:

- IMS Multimedia Telephony services, including:

- speech,

- video,

- fax,

- data (CS),

- Supplementary services defined within [40]; and

- Multimedia priority service.

- IMS emergency calls, including eCall.

This solution proposes changes in the procedures to support IMS eCall over NB-IoT NTN connected to EPC.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * * Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  |  | X |  |

* * * * Change * * * *

6.X	Solution #X: Support for eCall over NB-IoT NTN

6.X.1	High level principles

This solution is to address Key Issue #3.

This solution has the following high level principles:

To support eCall over IMS support (ECL) over NB-IOT NTN, Emergency Services support is a prerequisite:

-	to support Emergency Services, following changes are needed:

a)	MME needs include an indication for Emergency Services Support within the NAS Accept to the UE also for UE accessing over NB-IoT NTN;

b)	eNB needs to support related broadcast indicator that the cell supports:

1)	Emergency Services over NB-IOT NTN in order to support UEs in limited service state; and

-	to further support eCall over IMS, following change is needed:

2)	eCall over IMS support (ECL) over NB-IOT NTN in order to supports eCall over IMS.

6.X.2	Description

To support eCall over IMS support (ECL) over NB-IOT NTN, Emergency Services support is a pre-condition:

-	to support Emergency Services, following aspects are needed:

a)	MME needs include an indication for Emergency Services Support within the NAS Accept to the UE also for UE accessing over NB-IoT NTN;

b)	eNB needs to support related broadcast indicator that the cell supports:

1)	Emergency Services over NB-IOT NTN in order to support UEs in limited service state; and

-	to further support eCall over IMS, following aspects are needed:

2)	eCall over IMS support (ECL) over NB-IOT NTN in order to supports eCall over IMS.

Since it is likely that for a NTN cell it supports PLMNs from different countries and different countries have different requirements, the support indicators (bullets b)1)) and b)2)) can be per PLMN granularity.

The functionality that is already described in TS 23.401 [5] clause 4.3.12 for WB-EUTRAN access, need to apply NB-IoT and UE operating supporting emergency services over NB-IoT NTN. The following text in TS 23.401 [5] clause 4.3.12 restricts the NB-IoT from providing emergency services, thus needs to be removed:

“ Support for emergency bearer services is not available when the UE is using NB-IoT, i.e. the MME shall not indicate support for emergency bearer services using the Emergency Service Support indicator in the Attach and TAU procedures to a UE that accesses the network using a RAT Type equal to NB-IoT, and an NB-IoT cell shall not indicate support for emergency services in any broadcast information in AS.”

Regarding the domain priority and selection for a UE that attempts to make an eCall over IMS session using E-UTRAN or NG-RAN radio access networks, the same logic like in current TS 23.167 [7] is used without change, with the only difference is that now the NB-IOT using E-UTRAN can support ECL (i..e., belongs to the "Y" in Table H.2 in Annex H.6 in TS 23.167 [7])

6.X.3	Procedures

Existing procedures are reused.

6.X.4	Impacts to Services, Entities and Interfaces

UE

-	Support procedures described in TS 23.401 [5] for NB-IOT RAT to support eCall services over NB-IOT RAT.

MME

-	Include an indication for Emergency Services Support within the Attach and TAU Accept to the UE.

eNB of NB-IOT NTN

-	Include related per cell or per PLMN broadcast indicators, including:

(1)	the cell/PLMN support status of Emergency Services over NB-IOT NTN and

(2)	the cell/PLMN support status of eCall over IMS support (ECL) over NB-IOT NTN.