SA WG2 Meeting #171	S2-2509287

Wuhan, China, 13 – 17 October 2025	(revision of S2-2509602)

Source:	China Mobile

Title:	KI#4: New Sol: Location Verification of UE via NB-IoT GEO Access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#4.

# 1	Discussion

Emergency calls and some regulatory services rely on the UE's location information, with different services requiring varying levels of location accuracy. In scenarios where the UE accesses via NB-IoT NTN, some traditional positioning methods (e.g., OTDOA, AoA) will become ineffective. Additionally, while the GNSS location information reported by the UE may be precise but untrusted.

Therefore, this solution proposes a method to obtain precise and trusted UE location information.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.2.0.

* * * First Change * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x |  |  |  | x |

* * * Next Change * * *

6.X	Solution #X: <Support of Location Verification of UE via NB-IoT GEO Access>

6.X.1	High level principles

This solution resolves Key Issue #4 Location service for IMS emergency call and regulatory services over NB-IoT NTN,   the following high level principles are included in this solution:

-	The MME can obtain and store coarse location information by SMC procedure.

-	The UE reports geographical location information during emergency calls.

-	The reported geographical information from the UE (if untrusted) is verified based on pre-stored coarse location.

6.X.2	Description

As specidied in TS 23.167. The Location information is needed for 2 main reasons in emergency services. The initial purpose of the location information is to enable the IMS network to determine which PSAP serves the area where the UE is currently located, so that the IMS network can route the emergency session to the correct PSAP. The second purpose is for the PSAP to get more accurate or updated location information for the terminal during or after the emergency session where required by local regulation.

In the case of satellite access for NB-IoT, the MME can only get the coarse location from UE as specified in TS 23.401[5]:

4.13.10	UE Coarse Location information for NB-IoT satellite access

As described in clause 4.13.4, in the case of NB-IoT, the MME may request the UE to report its Coarse Location Information for location verification purposes. In addition, as described in TS 36.300 [5] and TS 36.413 [36], the eNB may request, in a S1-AP Initial UE message, the MME to provide the Coarse Location Information to eNB. In that case, the MME provides, if available, the Coarse Location Information to the eNB as described in TS 36.413 [36].

As Specified in TS 36.413, the accuracy of coarse location is approximately 2 km, which can satisfy the purpose of PSAP selection. However, this does not meet certain regulatory service requirements (e.g., the accurate location information mandated by Public Safety Answering Points, PSAPs).

UEs equipped with GNSS modules can report GNSS information to the IMS network, but this location data is considered untrusted by the IMS.

To solve this, this solution proposes a method of verify GNSS information against coarse location , to obtain  accurate and trusted location information.

6.X.3	Procedures

6.X.3.1		Obtaining location information for emergency calls

Figure 6.x.3.1-1: Location retrieval for IMS emergency call over NB-IoT via GEO

- The UE initiate emergency IP-CAN bearer establishment over NB-IoT NTN access, the UE reports its Coarse location information to the MME during the NAS Security Mode Command procedure.

- The MME sends the Subscriber LCS Report to LRF/GMLC, the message may include UE identity and Coarse location.

- The LRF/GMLC store UE identity and corresponding Coarse location, and sends Subscriber LCS Report ACK to the MME.

- The UE initiates an emergency call session request over the emergency call PDN via NB-IoT NTN access. The INVITE message carries UE’s geographical location information (e.g. GNSS)

- If the location information provided in step 4 is trusted and sufficient to determine the correct PSAP, the procedure continues from step 11 onwards. If the location information is untrusted or insufficient. The IMS core sends the Retrieve Location-routing information request to the LRF, the request shall include UE identity and any geographical location information provided by the UE in step 4.

- The LRF obtained the corresponding location information based on the UE identity. If the obtained Coarse location meets the accuracy requirements of the Requested QoS, it directly proceeds Step 10 and returns the coarse location to the IMS core. If the coarse location associated with the UE does not meet the accuracy of Requested QoS, The LRF sends Provide Subscriber Location request to the MME. This message carries the UE identity, Coarese location, UE’s geographical location information (e.g. GNSS), location verification indication.

- The MME sends the location request to E-SMLC, the request shall include Coarse location, UE’s geographical location information (e.g. GNSS), location verification indication.

- The E-SMLC performs location verification based on the location verification indication to validate whether the UE's geographical location information is within the coarse location range. If the verification is successful, the E-SMLC returns the successfully verified location information to the MME, along with a verification success indicator. If the verification fails, it returns a verification failure indication.

- The MME sends the Provide Subscriber Location ACK to LRF/GMLC, the response shall include verified UE location and verification result.

- The LRF/GMLC sends the Return Location-routing information to IMS core, if the verification in step 8 is successful, the response shall include the verified location information. If the verification in step 8 fails, the response shall include the coarse location and Indication that the requested QoS was not met.

- The IMS core selects the PSAP based on the location information obtained from the LRF.

- The IMS Core forwards the INVITE message to the PSAP. The message carries the location information obtained from the LRF.

6.1.3	Impacts to Services, Entities and Interfaces

MME:

-	Supports obtaining and storing UE's coarse location via SMC procedure.

LRF:

-	Supports determining whether the coarse location meets the requested QoS.

SMLC:

-	Supports verifying the geographical location based on the Coarse Location.

* * * End of change * * *