SA WG2 Meeting #171	S2-2509375

13 – 17 October 2025, Wuhan, China	(revision of S2-2508739)

Source:	LG Electronics, Nokia, Lenovo, NEC, Toyota

Title:	KI#1: Interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1.

Discussion

There are 8 solutions for KI#1 "Network energy related information exposure" TR 23.700-67v0.2.0.

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution # & title | 1 | 2 | 3 |
| 1: Enhancement of Exposure API to include UE categorization data | X |  |  |
| 2: Policy Control Considering Slice or 5GC Energy Consumption | X | X |  |
| 8: Estimation of Renewable Energy consumption | X |  |  |
| 9: Consumption energy source exposure | X |  |  |
| 10: Energy consumption calculation enhancements | X |  |  |
| 11: Energy Consumption estimation considering idle energy consumption | X |  |  |
| 12: Enhancement of energy related information exposure | X |  |  |
| 22: Support of UP path adjustments to maximize the usage of renewable energy | X |  | X |

Proposal#1: It is proposed to discuss Sol#2 when handling interim agreements/conclusions for KI#2 because Sol#2 is considered more related to KI#2 "Support of service adjustments for the UE".

Proposal#2: It is proposed to discuss Sol#22 when handling interim agreements/conclusions for KI#3 because Sol#22 is considered more related to KI#3 "NF (re)selection and/or UP path adjustment based on energy related information".

Proposal#3: It is proposed to discuss other solutions than Sol#2 and Sol#22 related to KI#1 interim agreements.

***********************************************************************************************

LGE as a pen holder provides the following summary:

IN	In case that tdoc captures Sol#x under Agreed Principles.

IN*	In case that tdoc captures Sol#x under Topics for further consideration.

OUT	In case that tdoc mentions not support/recommend Sol#x in Discussion part or Proposal part.

|  |  | Exposing additional Information |  | Renewable energy |  | New formulas |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| tdoc# | source companies | Sol#1 | Sol#12 | Sol#8 | Sol#9 | Sol#10 | Sol#11 |
| S2-2508739 | LG Electronics, Nokia, Lenovo | IN | IN | IN* | IN* | IN* | IN* |
| S2-2508493 | Vivo | IN | IN | IN | IN | IN | IN |
| S2-2508758 | Huawei, HiSilicon |  |  | IN | IN | OUT | OUT |
| S2-2508764 | NTT DOCOMO | IN | IN | IN* | IN* | IN | IN* |
| S2-2508822 | ZTE | IN* | IN* | IN | IN | OUT | OUT |
| S2-2508841 | NEC | IN | IN | IN | IN | OUT | OUT |
| S2-2509262 | Samsung | IN* | IN* | IN | IN |  | IN |
|  | Way Forward ? | IN? | IN? | IN? | IN? |  |  |

***********************************************************************************************

Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1.Y	Agreed Principles for KI#1

The following principles are agreed for KI#1.

1)	The following additional energy related information is exposed by EIF:

-	The exposure of additional information in association to the exposure of energy consumption information at a desired granularity, proposed by Solution#1 described in clause 6.1 is proposed for normative work, specifically,

a)	energy consumption per bit

b)	“Energy consumption category” that is a scalar with multiple values that are worked out by the EIF using formulas that are not in scope of 3GPP. This category is applicable to the same scope as the scope that the EIF provides for the energy consumption calculations.

c)	Categorization data for the Energy Consumption information for the UEs and the granularities are based on currently monitored by an EIF. The percentile does not represent the whole UE population with Energy Saving subscription. In case of multiple EIFs, the percentile represents only those currently monitored by the EIF. It is expected that the AF will derive the overall categorization based on mechanism outside the 3GPP scope if required.

-	Per UE application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session as proposed by Solution#12 described in clause 6.12 with the following restriction:

a)	The application ranking related information exposed to the AF is only about the applications that the AF has permission/authorization with the applications and for which has requested the ranking. For example, vehicle UEs can be the target of Solution#12 as vehicle UEs can operate various applications related to an AF managed or owned by car OEM such as road safety related applications, applications for In-Vehicle Infotainment (e.g. entertainment, navigation, connectivity with smartphones), AI/ML related applications (e.g. Federated Learning).

b)	The EIF performs the ranking only of the applications for which has been requested by AF.

c)	The EIF is not requested to perform the ranking considering all applications currently active in the PLMN.

NOTE 1:	As described in clause 6.12.1, Solution#12 exposes the application ranking related information only to AF(s) related to those applications. For example, vehicle UEs can be the target of Solution#12 as vehicle UEs can operate various applications related to an AF managed or owned by car OEM such as road safety related applications, applications for In-Vehicle Infotainment (e.g. entertainment, navigation, connectivity with smartphones), AI/ML related applications (e.g. Federated Learning).

NOTE 2:	For Solution#12, it is assumed that a single EIF is involved for all the applications that can be exposed together as the application ranking related information.

2)	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF, as proposed by Solution#8 and Solution#9 described in clause 6.8 and clause 6.9, respectively.

NOTE 3:	The normative work for renewable energy will be aligned to SA5 conclusion on whether and how the renewable energy can be provided by OAM.

* * * * End of Changes * * * *