SA WG2 Meeting #171	S2-2509574

13 - 17 Oct, 2025, Wuhan, China                            revision of S2-2509296

Source:	China Mobile, ZTE, SK Telecom, Ericsson, Samsung, China Telecom, vivo, ETRI, Nokia, NTT DOCOMO

Title:	Update for KI#2 Interim Agreements and Conclusions

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements for AIML_CN KI#2.

- Discussion

In TR 23.700-04 V0.3.1 (2025-09), some principles are agreed for KI#2. However, there are still several topics for further consideration depicted as 7.2.2 in TR 23.700-04.

The following topics are for further consideration for KI#2:

- Whether the UPF can be the consumer of the user plane traffic pattern and behaviour analysis or not.

- Whether new UPF or SMF events are needed or not for NWDAF data collection.

- How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

- Consumer actions are FFS.

This proposal discusses these topics and concludes the aspects for KI #2.

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** First change ***

7	Interim agreements

7.1	Agreed Principles

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288) for input data filtering to UPF.

7.1.2.2	Agreed Principles for Use Case #1

- The service consumer of the analytics service may be SMF, PCF, OAM, UPF and AF. The consumer may take the output Analytics into account.

- To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns , unexpected traffic volume or burst.

-	volume, rate or burst size of abnormal traffic,

-

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s),

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

- To derive the output analytics, the following input data may be collected from the UPF or SMF,( using existing UPF event exposure services) and AF, to support the NWDAF-based analytics:

- From UPF/SMF

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters

-	identification of corresponding UEs

-

Information of traffic characteristics.

- The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

-	Configuring UPF to block, redirect or shape abnormal traffic, to enforce bandwidth limitations

- UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations)

-	adjust packet processing resources,

- The PCF as consumer of NWDAF analytics may for instance take the following actions:

-	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

- 	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE Y: This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

*** Second change ***

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

- For UC1, the input from AF is FFS

*** End of the changes ***