SA WG2 Meeting #171	S2-2509777

Wuhan, P.R. China; 13th – 17th October 2025

Source:	NTT DOCOMO (pen holder)

Title:	[General aspects] Architectural Assumptions merge

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Architectural Assumptions

# Proposal

List of input papers that are covered in this merge proposal:

| 20.6.0 | S2-2508297 | 23.801-01: [General aspects] Architecture Assumptions for 6G | OPPO |
| --- | --- | --- | --- |
| 20.6.0 | S2-2508320 | 23.801-01: [General aspects] Architectural Assumptions | CATT |
| 20.6.0 | S2-2508704 | 23.801-01: [General aspects] Architecture Assumptions for 6G | Samsung |
| 20.6.0 | S2-2508790 | 23.801-01: [General aspects] Summary of Architectural Assumptions. | NTT DOCOMO |
| 20.6.0 | S2-2508976 | 23.801-01: Architecture assumptions | Nokia, T-Mobile USA, AT&T |
| 20.6.0 | S2-2508984 | 23.801-01: [General aspects] Architecture Assumptions for 6G | CSCN |

Proposals on top of the postponed baseline text at SA2#170 (in S2-2508790) are described below:

8790, 8320, 8704, 8976:

6G CN: In this study, it refers to the Core Network for the 6G System.

NOTE:	Whether 6G CN can be considered as evolution of 5GC or as a new Core Network will be determined during the study.

Pen-holder's proposal: Add the term as above

8790, 8704, 8976:

2. 	This study assumes that control and user plane of the 6G RAN connects to a single core network. It is assumed that the 6G RAN is a single technology framework based on a stand-alone architecture.

Pen-holder's proposal: Add bullet 2 as above

Baseline:

NOTE: approaches beyond stand-alone architecture will be considered in coordination with TSG RAN [ref: RP-251881].

8704: removes the note

8790:

NOTE: Additional migration options beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR will be studied (if needed) in alignment with TSG RAN timeline [ref: RP-252912].

Vs.

8976:

NOTE: Additional migration options beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR will be studied (if needed) in alignment with TSG RAN timeline (i.e. after September 2026) [ref: RP-252912].

Pen-holder's proposal: Add the NOTE as in 8790; listing exact dates in TR needs further discussion.

8704:

NOTE: SA2 System Architecture decision on RAN-CN Interface (e.g., whether P2P or SBI) will be determined before RAN check point (i.e., TSG#112 June 2026).

Pen-holder's proposal: Needs further discussion, the question is whether SA2 can make the decision before RAN3?

8704:

5. 	It is assumed the NFs in 6G CN may be newly defined or re-designed or extend 5GC NFs. In this case, the stand-alone architecture of NG-RAN as defined in TS 23.501 [x] can be connected to the 6G CN.

NOTE x: What parts of 5GC can be re-used or require re-design is to be determined as part of this study.

NOTE y:  SA2 System architecture decision on whether NG-RAN can be connected to the 6G CN will be determined before RAN check point (i.e. TSG#112 June 2026).

Vs.

8320:

5.   6G CN shall support NFs that use 5GC NFs as basis and any enhancements, that need further study and may be redesigned, or that to be added for supporting new features when identified during the study.

Pen-holder's proposal: Add the bullet 5 and NOTE x as in 8704, except correct the wording regarding the reference to TS 23.501.

NOTE y Needs further discussion: bullet 5 implies that 6G CN may support 5G N2 reference point, in this case there is no impact to NG-RAN and no need to align with RAN WGs.

8320:

x.	Support of orthogonal functionality split between 6G CN NFs.

Pen-holder's proposal: Needs further discussion.

8984:

x. 	The 6G system architecture, encompassing core network (CN) functions, is assumed to natively support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

Pen-holder's proposal: add the bullet as above.

8297 (not using the baseline document S2-2508790):

It’s assumed that:

x. 	The interface between 6G RAN and 6G CN only supports new RAT (i.e., 6G RAT).

y.	The Policy Framework defined in TS 23.503 is used as baseline.

z.	When the UE is in idle state, the UE location is known at the TA list granularity as defined in EPS and 5GS.

Pen-holder's proposal: Bullet x: Needs further discussion, overlap with Bullet 2. 
Bullet y: to be covered under WT#1.2 Policy Framework. 
Bullet z: add the bullet.

**** First Change (all text new) ****

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

6G Core Network (6G CN): In this study, it refers to the Core Network for the 6G System.

NOTE: To what degree 6G CN can be considered as evolution of 5GC or as a new Core Network will be determined during the study.

6G RAN: In this study, it refers to the radio access network for 6G radio access technology (6GR) being studied by 3GPP RAN WGs [ref: RP-252912].

6G System (6GS): 3GPP System consisting of 6G RAN, , 6G CN and UE and supports also connectivity of UEs via specific non-3GPP access(es).

NOTE: Which specific non-3GPP access(es) are supported is studied under WT#1.3.

**** Next Change (all text new) ****

## 4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

2. 	This study assumes that control and user plane of the 6G RAN connects to a single core network type, i.e. 6G CN. It is assumed that the 6G RAN is a single technology framework based on a stand-alone architecture.

NOTE 1: This does not exclude network sharing scenarios such as Multi-Operator Core Network.

NOTE 2: Additional migration options, if included by TSG RAN as per [ref: RP-252912], beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied (if needed) in alignment with TSG RAN timeline.

3.	The 6G system is assumed to maintain the RAN and CN functionality split as in 5GS.

3b.	The study aims to avoid duplication of functionality between 6G RAN and 6G CN.

4.	Voice and video services will be provided by IMS.

6. 	The 6G system is assumed to natively support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

**** End of Changes ****