3GPP TSG-WG SA2 Meeting #172	S2-2509990

Dallas, US, 17th – 21st Nov, 2025

Source:	CEWiT

Title:	Conclusion proposal for KI#2

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide conclusions for KI#2 to be incorporated into Clause 8.

# 1.	Discussion

This contribution proposes the conclusions for KI#2. The conclusions are drafted based on the principles agreed in the interim agreements for KI#2 in Clause 7 of 3GPP TR 23.700-14 V1.1.0.

The following conclusions are arrived at based on the interim agreements with the following updates,

- Based on conclusions of KI#1, the sensing gateway entity is not involved in the architecture. As SeMF manages and controls the sensing procedure, it will be appropriate for it to perform authorization of sensing service request. Removed the corresponding NOTE in the interim agreements of KI#2.

- There can be multiple triggers from various entities based on which SeMF can initiate revocation process. The clarity on this and multiple possible triggers are added.

- The conditions where SeMF may decide to skip the authorization, such as regulatory sensing services, is added.

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14 in Clause 8.

# * * * * First change * * * *

# 8	Conclusions

## 8.b	Conclusion for KI#2: Authorization and Revocation to Support Sensing Service

For KI#2 Authorization and Revocation to Support Sensing Service, the following principles are concluded and is recommended for normative work:

a)	The Sensing Service authorization consists of two steps:

1)	Authorization of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorization of the AF's Sensing Service Request is performed by the SeMF;

b)	The SeMF authorizes the Sensing Service Request from the AF, considering the example of Sensing authorization information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in the SeMF for authorization of the sensing service request.

d)	The SeMF can revoke the Sensing Service Request based on the triggers from AF/NEF, Sensing entity(ies) and internal logic. The examples of triggers can be authorization criteria are no longer met, load conditions, movement to a sensing non-allowed area, regulatory services etc.

e)	The AF can cancel the on-going Sensing Service Request when required.

f)	There is no need for authorization for the gNB as the Sensing Entity, as the gNB is deployed by operator.

g)   Depending on the type of sensing service, such as regulatory services, the SeMF may decide to skip the   authorization of sensing service request.

# * * * * End of changes * * * *