3GPP TSG-SA2 Meeting #172	S2-2510163

Dallas, TX, USA, 17-212025	(revision of S2-2509424)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6438 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling of application ID/predefined PDRs in reporting of usage data volume to EIF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current specification for Energy reporting does not fully cover all the different configurations for a PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating SMF behaviour for different PDU session configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of SDF/Application ID level reporting in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.51.2.2.2	Energy Consumption information collection from SMF

The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and optional S-NSSAI or (S-NSSAI, DNN). Also, the EIF subscribes to the UDM to be notified with the applicable SMFs.

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (i.e. Application Identifier, or Packet Filters) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1.The SMF receives the data volume of the required granularities from the PSA UPF. The SMF derives the gNB ID(s) from the AMF provided ULI as described in clause 4.3.2. If the I-SMF is involved in the PDU session, the I-SMF reports to the SMF of PDU session via Nsmf_PDUSession_Update the current I-UPF ID and subsequent change of the I-UPF ID(s). If the I-SMF is involved in the PDU session, the SMF of PDU session requests the I-SMF via Nsmf_PDUSession_Update to report the current ULI and subsequent change of ULI only when the gNB ID changes.

The SMF then sends the collected data volume, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. The information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2. Upon release of a PDU session, the SMF terminates subscriptions related to the PDU session. The SMF periodically notifies EIF with the information at the end of each time interval T until the EIF unsubscribes as defined in TS 23.502 [3]. The SMF shall collect data volume only for the PDU session carried over the 3GPP Access in 5GS. When the PDU session is over Non-3GPP Access type, or it is a PDN connection over EPS handled at a SMF+PGW-C, the SMF reports UL/DL Data volume in the measurement period equal to 0 (zero) and the gNB ID is not provided.

NOTE 1:	The SMF can derive gNB ID(s) according to the AMF provided ULI as described in clause 4.3.2 of TS 23.502 [3].

In the case of MA PDU session, the SMF shall not include in the report the UL/DL Data volume related to the traffic over the Non-3GPP access leg and the serving gNB ID refers to the traffic over the 3GPP leg.

When the SMF receives the Nsmf_EventExposure_Subscribe with application information (i.e. Application Identifier or Packet Filters) from the EIF for energy consumption, the SMF generates a URR for the Data Volume counting with a Periodic measurement threshold set according to the Timer interval T and associates the subscription from EIF to the generated URR. The SMF is then associating the generated URR to an appropriate PDR. SMF logic ensures that the PDRs and their precedence values are configured appropriately in the UPF so that both, the URRs for Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rules can co-exist and the traffic  is treated in the same way as before. For example, when the SMF receives the Nsmf_EventExposure_Subscribe with application information, the SMF may behave as follows:

-	If there is an existing PCC rule for exactly the same Service Data Flow as the application information in the energy consumption request (i.e. with same Application Identifier or same Packet Filters), the SMF associates the generated URR with the PDR of that PCC rule.

-	If there is no PCC rule matching with the application information in the energy consumption request (i.e. the Application Identifiers are different or no packet filter overlap exists) but a match-all PDR has been provisioned for the PDU Session, the SMF generates a PDR for the application information and associates the generated URR with that PDR. All other instructions of the PDR related to the match-all PCC rule are copied (so that the traffic is treated in the same way as before).

NOTE x1:	The SMF can only generate a PDR due to the energy consumption request and request the UPF to forward the matching traffic if this is allowed by the PCF or the local policy. If there is no match-all PDR provisioned for the PDU Session, any application traffic not authorized by the PCF or the local policy has to be discarded.

-	If the application information in the energy consumption request is partially overlapping with the Service Data Flow(s) of (one or more) existing PCC rule(s) other than the match-all PCC rule (which can only occur when the subscription contains Packet Filters), the SMF may need to generate two or more PDR(s). For the PDR(s) with overlapping packet filters, the SMF associates the generated URR with the(se) PDR(s) and copies all other instructions of the PDR(s) related to the existing PCC rule(s) with overlap. SMF also associates the generated URR to the PDR with the non overlapping packet filters (if any), and it copies all other instructions of the PDR related to the match-all PCC rule. This ensures that both, the Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rule(s) and the match-all PCC rule can be realized.

NOTE x: One or more of the above can apply to an energy consumption request.

NOTE w: For a MA PDU Session, the generated URRs are only included for the 3GPP access in the Multi-Access Rule (see clause 5.8.5.8).

If the PCC rules are changed, the SMF shall again perform the above checks and actions.

The same type of application information (Application Identifier or Packet Filters) should be used by the EIF in the subscription request as it is used in the corresponding  PCC rule(s) for service data flow detection.

NOTE y:	SMF may be unable to determine the matching or overlapping PCC rule(s) and PDR(s) if the PCC rule includes an Application Identifier and EIF provides Packet Filters and the other way around.

If predefined PCC rules (see clause 5.8.2.8.1) associated to packet detection rules locally defined in UPFs (see TS 29.244 [65]) are used for the PDU Session, the procedure described above requires that the SMF also needs to be aware of the semantics of such rules in order to behave as described above and avoid conflicts if they arise. If the SMF is not aware of the semantics, then the SMF should reject requests that could conflict with packet detection rules locally defined in UPFs.

When the SMF receives the Nsmf_EventExposure_Subscribe for the PDU Session from the EIF (i.e. no application information is provided by the EIF), the SMF associates the subscription from EIF to the PDU Session, generates a URR for the Data Volume counting (with a Periodic measurement threshold set according to the Timer interval T) and associates it with all PDRs of the PDU Session.

In both cases, the UPF will send the collected data volume information of this URR at the end of each Time interval to the SMF in a Usage Report (see clause 4.4.2.2 of TS 23.502 [3]). If there is a change in the user plane path (e.g. handover between gNBs or insertion/removal of I-UPF) during the Time interval, the SMF shall request the collected data volume information of this URR from the UPF. If this happens, the UPF will immediately send the collected data volume information to the SMF in a Usage Report (see clause 4.4.2.2 of TS 23.502 [3]) and the SMF shall then store this information together with the gNB ID and I-UPF ID that belong to the user plane path before the change happened.

Table 5.51.2.2.2-1: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU Session. |
| Application Identifier or Packet Filters | Information on targeted reporting granularity |

NOTE 2:	The user-plane energy consumption information reporting interval from the SMFs is the network-wide configurable starting time and interval T.

Table 5.51.2.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in clause 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > UL/DL Data Volume | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data Flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data Flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > gNB ID | Identifier of the gNB serving the UE. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g. time stamps). |

NOTE 3:	Each entry of List of Data Volume information represents the serving gNB and UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the gNB or UPF(s) is changed.

End of changes