3GPP TSG SA WG2 Meeting #83	S2-110983

21 - 25 February 2011, Salt Lake City, Utah, USA	(Revision of S2-110808)

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.203 | CR | 0505 | rev | 2 | Current version: | 11.0.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean-up of TS 23.203 for TDF related functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Allot Communications, Ericsson, Telcordia, AT&T, Huawei, Tekelec, GENBAND |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG 2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SAPP |  |  |  |  |  |  | Date: |  | 16/02/2011 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | To make some expressions and pictures, related to TDF functionality, more clear and accurate. To define funcional requirements in an accurate way to avoid references to any particular implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Application terminology instead of service terminology for TDF, remove reference to implementation and some minor text corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing text covering the TDF in the relevant sections is inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5, 7.4.2, 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 4.5	Application Detection and Control

The application detection and control feature comprise the request to detect the specified application traffic, report to the PCRF on the start/stop of application traffic and to apply the specified enforcement actions.

Two models may be applied, depending on operator requirements: solicited and unsolicited application reporting.

Solicited application reporting: The TDF is instructed by PCRF on which applications to detect, report to the PCRF and the actions to be enforced for the detected application traffic. The detection is applied only if user profile configuration allows this.

Unsolicited application reporting: The TDF is pre-configured on which applications to detect and report. The enforcement is done in the PCEF. It is assumed that user profile configuration indicating whether application detection and control should be enabled is not required.

For the solicited application reporting, the report shall be done by the TDF only if PCRF has explicitly requested to do so.

The report to the PCRF shall include the same information for solicited and unsolicited application reporting that is whether the report is for start and stop, the detected Application Identifier and, if available, the service data flow descriptions for the application user plane traffic.

# *** 2nd Change ***

### 7.4.2	IP-CAN Session Modification; PCRF initiated

This clause describes the signalling flow for the IP-CAN Session modification initiated by the PCRF. The AF or TDF may be involved. An example of the scenario is initiation and authorization of a session-based service for which an IP-CAN Session is modified. IP-CAN Session handling and handling of PCC rules for non-session based services, and also general handling of PCC rules that are not subject to AF-interaction or TDF-interaction is also applicable here.

# Figure 7.5: IP-CAN Session Modification; PCRF initiated

This procedure concerns both roaming and non-roaming scenarios. In the roaming case when home routed access applies (figure 5.1-3) or if case 2a applies (as defined in clause 7.1) for Local Breakout (figure 5.1-4), when a Gateway Control Session is used, the V-PCRF shall proxy Gateway Control and QoS Rules Request between the BBERF in the VPLMN and the H-PCRF over S9. For this case the H-PCRF may also initiate a Gateway Control and QoS Rules Provisioning procedure towards the BBERF in the VPLMN and proxy the information via the V-PCRF over S9.

For case 2b in the Local Breakout scenario (figure 5.1-4) and if the Gateway Control Session is terminated locally at the V-PCRF, the V-PCRF shall reply to/initiate Gateway Control Session and QoS Rules Request/Provisioning procedures locally without notifying the H-PCRF. For this case the V-PCRF shall proxy the Policy and Charging Rules Provisioning and Acknowledge over S9 between the PCEF in the VPLMN and the H-PCRF. If the AF is located in the VPLMN for this scenario, the V-PCRF shall proxy AF session signalling over S9 between the AF and the H-PCRF.

NOTE 1:	The case when the AF resides in the VPLMN is not showed in the figure.

In the non-roaming case (figure 5.1-1) the V-PCRF is not involved at all.

1a.	Optionally, the AF provides/revokes service information to the PCRF due to AF session signalling. The AF may subscribe at this point to notification of bearer level events related to the service information.

NOTE 2:	For the PCRF to generate the applicable events, the PCRF instructs the PCEF to report events related to the corresponding PCC rules. Such events are not shown in this sequence diagram.

1b.	Alternatively, optionally, in case of standalone TDF, the TDF detects the start/stop of a application traffic that matches with one of the activated ADC Rules. Then, in case of solicited application reporting, for the start of traffic detection, in case the enforcement actions were provided as a part of ADC rules, the TDF shall enforce those actions.

For the solicited application reporting, if the start/stop of application traffic detection event trigger was received from PCRF, the TDF shall provide application information to the PCRF, including the ADC Rule Identifier, start or stop of application traffic detection event trigger and the service flow descriptions, if available.

For the unsolicited application reporting, the TDF shall provide application information to the PCRF, including the ADC Rule Identifier and the service data flow descriptions, if available.

2.	The PCRF stores the service information if available and responds with the Acknowledgement to the AF. This is applicable to 1a case.

NOTE 3:	Without AF interaction, a trigger event in the PCRF may cause the PCRF to determine that the PCC rules require updating at the PCEF, e.g. change to configured policy.

NOTE 4:	This procedure could also be triggered by the Gateway Control and QoS Rules Request procedure as described in clause 7.7.3.

3.	The PCRF makes the authorization and policy decision.

4.	For the case 1b, the PCRF stores the application information, if available, and responds with the Acknowledgement to the TDF. If the TDF is standalone, for the solicited application reporting, the PCRF may provide a new ADC decisions to the TDF within this acknowledge. This may include ADC Rules activation, deactivation and modification. In case of local breakout, the V-PCRF shall provide functions to extract ADC rules from PCC rules provided by the H-PCRF over S9.

# *** 3rd Change ***

### 5.2.2	Gx reference point

The Gx reference point resides between the PCEF and the PCRF.

The Gx reference point enables a PCRF to have dynamic control over the PCC behaviour at a PCEF.

The Gx reference point enables the signalling of PCC decision, which governs the PCC behaviour, and it supports the following functions:

-	Request for PCC decision from PCEF to PCRF;

-	Provision of IP flow mobility routing information from PCEF to PCRF; this applies only when IP flow mobility as defined in TS 23.261 [23] is supported;

-	Provision of PCC decision from PCRF to PCEF;

-	Reporting of the start and the stop of a detected applications and transfer of service data flow descriptions for detected applications, if available, from the PCEF to the PCRF,

-	Delivery of IP-CAN-specific parameters from PCRF to PCEF or from PCEF to PCRF; this applies only when Gxx is deployed.

-	Negotiation of IP-CAN bearer establishment mode (UE-only or UE/NW);

-	Termination of Gx session (corresponding to an IP-CAN session) by PCEF or PCRF.

NOTE:	The PCRF decision to termination an Gx session is based on operator policies. It should only occur in rare situations (e.g. the removal of a UE subscription) to avoid service interruption due to the termination of the IP-CAN session.

A PCC decision consists of zero or more PCC rule(s) and IP-CAN attributes. The information contained in a PCC rule is defined in clause 6.3.

# *** End of Changes ***