# 3GPP TSG-SA2 Meeting # 83	         S2-111074
SALT LAKE CITY, USA, 21 – 25 Feb 2011

Title:	[DRAFT] Reply LS on QoS-Negotiation and QoS-Upgrade on the Gx for GnGp SGSN

Response to:

Release:	Rel-8/Rel-9/Rel-10

Work Item:	SAES

Source:	SA2

To:	CT3

Cc:	CT4

Contact Person:

Name:                   Tony Xia

Tel. Number:        +86 10 82882308

E-mail Address:	xiaxu@huawei.com

Attachments:

1. Overall Description:

SA2 thanks CT3 for the LS QoS-Negotiation and QoS-Upgrade on the Gx for GnGp.

SA2 has discussed this issue and have the following analysis:

- It was agreed by SA2 that in case of Gn/Gp SGSN accesses to the PGW, the IP-CAN type is set to 3GPP-EPS, the PDN GW shall mediate Gn/Gp procedures, this has the advantage that the PCRF experiences no difference compared to S5/S8 procedures;

- The PCEF performs the bearer binding for 3GPP-EPS access, the PCRF has nothing to do with the bearer level parameters of QoS-Negotiation AVP and QoS-Upgrade AVP;

- Usually, the PCRF is unlikely to authorize QoS higher than requested QoS.

So, it’s no need for the PDN GW sending the QoS-Negotiation and QoS-Upgrade to the PCRF.

- Question 1: For the case when No QoS negotiation is required or QoS upgrade is not allowed, if the PCRF modifies the QoS in a way not acceptable by the Gn/Gp SGSN, should the PGW terminate the related PDP Context?

[SA2 Answer] No.

- Question 2: How to support this functionality over Gx considering the current SA2 statement that “the PDN GW shall mediate Gn/Gp procedures so that PCRF experiences no difference compared to S5/S8 procedures”.

[SA2 Answer] CT3 shall align with SA2 decision and does not send the QoS-Negotiation and QoS-Upgrade to the PCRF. If the PCRF modifies the QoS when P-GW received the No QoS negotiation and No QoS upgrade from the Gn/Gp SGSN, the P-GW shall notify the PCRF that the requested QoS has failed. If this being a frequent error case, the No QoS Upgrade AVP over Gx can be introduced over Gx.

- Question 3: What kind of QoS information (i.e. QCI, ARP, APN-AMBR, MBR) can be applied for the QoS Upgrade and QoS negotiation and what the related impacts are (e.g. in bearer binding when changing QCI and/or ARP) in the network behaviour.

- [SA2 Answer] The QoS upgrade in this case is related to GPRS QoS defined in 23.107 as this is for the Gn/Gp interface. The parameters related to Gn/Gp interface MBR, GBR, traffic class, traffic priority are part of the definition. Thus the context in the Gx interface (i.e. QCI, ARP, APN-AMBR, MBR) can be applied for the QoS upgrade in this case.,

2. Actions:

To CT3 group:

ACTION: 	SA2 asks CT3 to take above information into account.

3. Date of Next SA2 Meetings:

SA2#84		11 - 15 Apr 2011  	Bratislava   	SK