SA WG2 Meeting #92	S2-122746

Barcelona, Spain - 09 - 13 July 2012	(revision of S2-12xxxx)

Source:	HTC

Title:	Discussion on issues for group based messaging

Document for:	Approval / Discussion

Agenda Item:	8.2.4

Work Item / Release:	MTCe_GROUP/Rel12

Abstract of the contribution: this paper proposes to include architectural requirements for group based messaging by referring to authorization functionality and report message in TS 23.682.

Discussion:

In SA2#91 meeting, a key issue of group based messaging is included in building block of Group based feature under MTCe WID. To efficiently distribute the same message, one possible usage of the group-based message is to distribute trigger request to UEs who are members of a MTC group.

In TS 23.682, the 3GPP system has included architecture and network functionalities to support device trigger delivery. For the sake of simplicity and unity, the reuse of an existing interface and functionalities are needed for authorization functionalities and reporting function:

- Authorization function:

It is required to consider the authorization functionalities in the system before determining how to distribute a group-based message. According to TS 23.682, the following architectural requirements for authorization functionalities in the 3GPP network shall also be applied for device trigger via group message:

1. The functionality of the MTC-IWF includes the ability to authorize control plane requests from an SCS.

2. The S6m Reference Point shall support interrogation of HSS/HLR to:

-	determine if a SCS is allowed to send a device trigger to a particular UE.

- Reporting function:

In response to the result of authorization, it is also required to support the same report messages in the Tsp reference point as well as an interface beteen MTC-IWF and a CN node for distributing group message:

3. The Tsp reference point shall supports the following device trigger functionality:

-	report to the SCS the acceptance or non-acceptance of the device trigger request;

-	report to the SCS the success or failure of a device trigger delivery;

4. The T4 reference point shall supports the following device trigger functionality:

-	report to MTC-IWF the submission outcome of a device trigger and the success or failure of delivering the device trigger to the UE.

Proposal:

It is proposed to agree that the system shall support the authorization functionalities as indicated in 23.682 for device trigger delivery via group-based message. The corresponding changes are made for architectural requirements of group based messaging.

****** First Change ******

## 8.1	Key Issue - Group based Messaging

### 8.1.1	Description

MTC applications generally involve a group of devices. Typically applications today involve more than 1000 subscriptions for a single customer. From both customer and operator points of view, there is benefit in optimised handling of groups of MTC devices.

Group based messaging can be used to efficiently distribute the same message (e.g. a trigger request) to those members of an MTC group that are located in a particular geographical area on request of the SCS.

### 8.1.2	Architectural Requirements

The following group messaging related requirements shall be met:

-	The network shall provide a mechanism to distribute a group message from an SCS to those members of an MTC group located in a particular geographic area.

NOTE 1: The geographic area for the distribution may be a cell sector, a cell or a group of cells. Any members of the group who are outside the geographic area won't receive the message (e.g. the device is roaming on a different network or just in a different cell in the same network). This does not affect how those inside the designated geographic area respond to the message, nor is there an expectation that the 3GPP network do anything to try to send the message to those members of the MTC group who happen to be outside the designated geographic area.

-	The group based messaging feature shall not require additional new functionality for UEs that do not use this feature.

-	The system shall support a mechanism where a UE that uses the group based messaging feature can efficiently recognize distributed group messages addressed to the UE.

NOTE 2: The 3GPP system does not need to provide an acknowledgment of receipt of the group message by an MTC device.

-	The system shall provide an interface for the SCS to send a group message. This interface shall be able to carry the following information:

-	the application layer content of the group  message,

-	the group identification for which the group message is intended, and

-	the geographical area and RAT(s) in which the group message shall be distributed.

NOTE 3: The reuse or extension of an existing interface is preferred.

- The system shall have the ability to authorize control plane requests of a group message from an SCS.

- The system shall provide an interface to support the functionalities for reporting to the SCS including the acceptance/non-acceptance of a group message and the success/failure of distributing a group message.

- The system shall provide an interface to support interrogation of HSS/HLR to:

- 	determine if a SCS is allowed to send a group message to UEs who are members of a MTC group;

Editor’s Note: It is FFS if there is a need to conduct authorization for other attributes in a group message, e.g. geographical area and RAT(s).

- The system shall provide an interface for the MTC-IWF to distribute a group message and to receive the submission outcome and delivery report about the success or failure of distributing a group message.

-	The system shall be protected against overload resulting from devices responding to the distributed group message.

-	Group based messaging shall be supported in GERAN, UTRAN, and E-UTRAN access.

Editor’s Note: Support for non 3GPP access such as Wi-Fi is FFS

Editor’s Note: Charging requirements for the group message feature are FFS.

Editor's Note:	 Whether 3GPP system support is needed for security or whether an application layer security solution suffices with group based messaging is FFS.

****** End Changes ******