SA WG2 Meeting #93	S2-123703

Sofia, Bulgaria - 08 - 12 Oct. 2012	(revision of S2-12xxxx)

Source:	HTC

Title:	Requirement for Monitoring Service Provisioning

Document for:	Approval

Agenda Item:	8.2.2

Work Item / Release:	MTCe-MONTE/Rel-12

Abstract of the contribution: It is proposed to add requirements for MTCe monitoring key issue in TR 23.887.

# Discussion

In architecture requirements in TR 23.887 clause 6.1.2, the UEs and Network needs to be able to configure, detection, reporting for monitoring events. These requirements are for provisioning monitoring service to subscribed UEs. Therefore a new requirement for managing subscription information about monitoring service is needed, e.g. the UE’s subscription of the monitoring events indicates the activation or deactivation status of the monitoring event.

Further, it is not clear what the relation between configuration and the subscription is. There are two possible cases:

- The configuration of the monitoring events is based on the service subscription of the UE. In this case, the configuration of the monitoring service for a UE is fixed, i.e. the monitoring service is provisioned to a UE according to the service contract/agreement between the network provider and the service providers.

- The configuration of the monitoring events is based on both of the service subscription of the UE as well as the service request from the SCS. That is, the SCS can manage the activation or deactivation of the monitoring events that are subscribed by the UE. In this case, the configuration of the monitoring service is dynamic, i.e. the monitoring service is triggered by the SCS for the UEs subscribed to the monitoring service.

Therefore, clarification of architecture requirements is needed before the consideration of candidate solutions.

# Proposal

It is proposed to include the proposed requirements in TR 23.887.

* * * First Change * * * *

## 6.1	Key Issue - Monitoring

### 6.1.1	Description

Monitoring is intended for monitoring of configured events. This comprises of means that allow for activating monitoring of specific events, the event detection and the reporting to authorised users, e.g. for use by applications or logging. For MTC Devices, it is desirable that the network detects and reports events (including location) caused by those devices that may result, for example, from vandalism or theft of the communication module. If such an event is detected, the network might be configured to perform special actions, e.g. limit the access or reduce the allocated resource.

This key issue aim at studying and providing solutions for enhancements to 3GPP procedures for configuration of monitoring events, monitoring event detection and reporting.

### 6.1.2	Architectural Requirements

The following architectural requirements shall be met –

Provision of Monitoring Service:

- It shall be possible for the network to manage subscription information of the UEs for monitoring services.

- The network shall provide a mechanism to provision monitoring service for the subscribed UEs per SCS request indicating feature parameters of the monitoring service, e.g. monitoring event to be activated/deactivated.

Configuration of Monitoring Events and actions:

-	It shall be possible to configure monitoring events in the 3GPP system (UE or Network Nodes), for example - monitoring the association of the MTC Device and UICC, change in the point of attachment, loss of connectivity, alignment of subscribed feature etc.

-	It shall be possible to configure the action to be executed by the network when configured monitoring event is detected, for example - reduce services provided to the UE, restrict access of the UE, detach the UE etc.

-	It shall be possible to configure the maximum time between the actual loss of connectivity and its detection.

Detection of Monitoring Events:

-	It shall be possible for the network to detect monitoring events. On detection of the monitoring event UE or Network may execute a configured action.

Reporting of Monitoring Events:

-	It shall be possible for the UE or network to report the detected events to the Services Capability Server or Application server. Application layer reporting of monitoring events is outside the scope.

* * * End of Change * * * *