3GPP TSG SA WG 1 Meeting #101 	S1-230115

Athens, Greece, 20 – 24 February 2023	(revision of S1-22xxxx)

Source:	Qualcomm, Charter Communications

pCR Title:	Update to Use case on Sensing Assisted Automotive Maneuvering and Navigation

Draft Spec:	3GPP TR 22.837

Agenda item:	7.1

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: We propose updates to the Use Case on Automotive navigation and Maneuvering in Clause 5.8

1. Introduction

The updates in this document provide more clarification on the use of non-3GPP sensors in the sensing study

2. Reason for Change

- Adding defintions for non-3GPP based sensing data and updating  3GPP sesning measurement data and sensing results.

- Updating requirements

- Removing the EN Clause 5.8.5

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020, https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29] 	https://www.ieee802.org/11/Reports/tgbf_update.htm

[30]	https://mentor.ieee.org/802.11/dcn/20/11-20-1712-02-00bf-wifi-sensing-use-cases.xlsx

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[39] 	H. V. Habi and H. Messer, “Recurrent Neural Network for Rain Estimation Using Commercial Microwave Links,” in IEEE Transactions on Geoscience and Remote Sensing, vol. 59, no. 5, pp. 3672-3681, May 2021, doi: 10.1109/TGRS.2020.3010305.[40]                    Roberto Opromolla, etc., “Perspectives and Sensing Concepts for Small UAS Sense and Avoid”, 2018 IEEE/AIAA 37th Digital Avionics Systems Conference (DASC)

[41]	https://www.rwjbh.org/trinitas-regional-medical-center/treatment-care/sleep-disorders/sleep-apnea/

[42]	https://my.clevelandclinic.org/health/articles/10881-vital-signs

[43]	3GPP TR 22.855, "Study on Ranging-based Services".

[44]	Guoxuan Chi, et. al., "Wi-Drone: Wi-Fi-based 6-DoF Tracking for Indoor Drone Flight Control", MobiSys 22, Association for Computing Machinery, 2022.

[45]	Report on Automated Valet Parking: technology assessment and use case implementation description – 5G Automotive Association (5gaa.org). https://5gaa.org/news/report-on-automated-valet-parking-technology-assessment-and-use-case-implementation-description/

[46]	Nie Y B , Zhang L . Main amendments to Working Safety Regulation of State Grid Company(Dynamical Part for Hydrodynamic Power Plant)[J]. East China Electric Power, 2008.

[47] 	Giuseppe Fragapane, René de Koster, Fabio Sgarbossa, Jan Ola Strandhagen, Planning and control of autonomous mobile robots for intralogistics: Literature review and research agenda, European Journal of Operational Research, Volume 294, Issue 2,2021, Pages 405-426

[48]	Li S, Li X, Lv Q, et al. WiFit: Ubiquitous bodyweight exercise monitoring with commodity wi-fi devices, 2018 IEEE SmartWorld, Ubiquitous Intelligence & Computing, Advanced & Trusted Computing, Scalable Computing & Communications, Cloud & Big Data Computing, Internet of People and Smart City Innovation, IEEE, 2018: 530-537.

[49]	https://www.synopsys.com/automotive/autonomous-driving-levels.html

[y]	TS 22.261 https://www.3gpp.org/ftp/Specs/archive/22_series/22.261

* * * Next Change * * * *

## 5.8	Use Case on Sensing Assisted Automotive Maneuvering and Navigation

### 5.8.1	Description

To support smart transportation and autonomous driving, more vehicle and devices are equipped with sensing technologies. For example, cameras, Radar, and Lidar systems are the most used sensors by the automotive industry to maintain the perception for autonomous vehicles at various levels of autonomy. Accurate sensing information is crucial to enable the safe and reliable control of the vehicles.

Due to the mounting position of the sensors (e.g., 3GPP based sensors) information collected from a single vehicle's sensors may not be sufficient or accurate enough to satisfy the advanced automotive use cases, e.g., autonomous driving, coordinated manoeuvre, etc. Therefore, the 5G system could coordinate sensing to enable the vehicles derive 3GPP sensing results which could be consumed at the vehicle and used for the vehicular control and driver assistance, e.g., feed into the Automated Driving System (ADS) in the car [21]. Alternatively, the 3GPP sensing data collected by the UE can be sent alongside relevant sensing information to other sensing entities (including other vehicles, roadside units, and network) for further processing (if required) before sharing with a 3rd party application as shown in Figure 5.8.1-1.

The network facilitated NR based sensing described above could significantly improve the sensing reliability and quality, enabling new and advanced automotive use cases.

Figure 5.8.1-1: 5G System Assisted Automotive maneuvering and navigation

### 5.8.2	Pre-conditions

In this use case, Joe and Bob’s vehicles are equipped with 3GPP  sensing technology. Non-3GPP sensors like radar, camera and Lidar sensors could also be available in the vehicles. Additionally, the vehicles are capable of 5G communications, including direct communication with other vehicles, communication with 5G system via RAN entities.

### 5.8.3	Service Flows

5G system assisted coordination of sensing service

Step 1 (Network provides configurations and policies): When Bob’s car registers for 3GPP sensing service, the network provides policies and configurations to enable UEs take appropriate actions during sensing e.g., obtaining sensing data from another UEs/RAN entities.  For example, the policies provided by network could provide guidance for the discovery UEs/RAN entities with appropriate NR RF sensing capabilities, when to trigger requests, when to stop sending requests, messaging formats, the communication configurations (such as which 5G communication mode to use and under which conditions), etc. These polices and configurations could be updated frequently as determined by the network.

Step 2 (Bob determines his sensors are blocked): Bob's sensor(s) is(are) blocked by Joe's vehicle, and cannot adequately detect its surroundings (e.g., detect if there is another vehicle in front). This could result in the vehicle miscalculating the needed distance to stop before a traffic light. In other cases, Joe's vehicle could also reduce the valid sensing region and result in misdetection of incoming vehicles size or shape, especially near intersections. The sensing results cannot fully satisfy the autonomous driving needs and requirement.

Step 3 (Bob recognizes need for sensing inputs): Due to unsatisfactory autonomous driving needs and requirements, the UE in Bob's vehicle is notified that its sensors are blocked and needs 5G System assistance for coordination of the sensing service.

Step 4 (Bob’s vehicle discovers Joe’s vehicle): With the policies and configurations provided by the 5G system, Bob’s vehicle can search for neighbouring UEs/RAN entities or ask the network to provide recommendations for UEs/RAN entities and their 3GPP NR RF sensing capabilities (e.g., if UE/RAN entity supports sensing service). This information would be used to discover other vehicles and RAN entities with 3GPP NR RF sensors that can support sensing in the area. In this example, Bob's vehicle discovered Joe's vehicle could be useful in providing sensing inputs.

Step 5 (Bob’s vehicle connects to Joe’s vehicle): Bob's vehicle then establishes 5G communication connection with Joe's vehicle and/or RAN entities as shown in Figure 5.8.3-1. The most suitable 5G communication mode (e.g., broadcast, unicast, etc) is determined by the Bob’s vehicle based on 5G system configuration and policies.

Step 6 (Bob's vehicle requests sensing info from Joe’s vehicle). The request could indicate the information needed to perform sensing, e.g., the additional region to be covered, additional sensing target, synchronization info, etc.

Step 7 (Joe sends sensing results/sensing measurement data to Bob’s vehicle) Based on the information provided by Bob’s vehicle; Joe sends Bob 3GPP sensing data identifying objects in its surroundings. It is important to note that when 3GPP sensing data is shared between Joe and Bob, it is expected to be performed in compliance with operator policy on the use of the operator resources (e.g., licensed/unlicensed spectrum).

Step 8a (Bob processes sensing data locally) Based on the fact that Bob’s has non-3GPP  sensors (e.g., camera, Lidar), Bob’s car combines the 3GPP sensing data from Joe’s vehicle with other sensors and displays the sensing results on the application in its autonomous driving application in his car.

Step 8b (Bob sends NR and non-NR sensing results to 3rd party application) Alternatively, Bob sends  sensing results (without non-3GPP sensing data) and non-3GPPsensing data from the camera and Lidar to a 3rd party application server (through the network) for fusion by the 3rd party.  It should be noted that in case assistance information is required with the sensing result that is shared with the 3rd party application provider, this information should be shared with the appropriate consent and permissions.

Figure 5.8.3-1:  5G system assisted automotive maneuvering and navigation

With the sensing information provided by Joe’s vehicle and/or the network, Bob’s vehicle obtains a full map of the region. The autonomous driving algorithm can make corresponding decisions reliably.

### 5.8.4	Post-conditions

Using 5G system assistance, Bob’s vehicle would be able to achieve highly reliable and accurate sensing results, by coordinating the operation with other vehicles to collaborate with other sensing devices to improve quality. With this high-quality sensing results, advanced smart transportation use cases and autonomous driving could be achieved.

### 5.8.5	Existing features partly or fully covering the use case functionality

V2X communication supports the information exchange among the vehicles, between vehicle and infrastructure or network.

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-1] The 5G system shall be able to support mechanisms to control UEs and RAN entities for a sensing service.

NOTE1:	In the requirement above, control may include configuration such as sensing specific policies and settings (e.g., conditions for triggering sensing requests, location,  etc.) coordinated amongst UE and RAN entities.

[PR 5.8.6-2] For a sensing service, the 5G system shall be able to support mechanisms for the UEs and RAN entities to provide 3GPP sensing data.

NOTE2:	This requirement can cover scenarios making use of information already available in the EPC and E-UTRA (assuming no new functionalities are required in the EPC and E-UTRA).

[PR 5.8.6-3] The 5G system shall be able to support an authorized UE in the discovery of UEs and selection of RAN entities with the required 3GPP NR RF sensing capabilities for the sensing service.

[PR 5.8.6-4] Subject to user consent and national or regional regulations, based on operator policy, the 5G system shall be able to provide means to authorize and configure a UE for sensing operation (e.g., based on location, time, etc) and for establishing the communication connection needed to assist the sensing service.

NOTE3: 	The above requirement assumes that the communication connection used for assisting sensing service (e.g. for transferring 3GPP sensing data or  sensing results) can include existing communication connection modes such as direct network communication, direct device connection under network coverage and indirect network connection [y].

[PR 5.8.6-5] Subject to user consent and national or regional regulations, based on operator policy, the 5G system shall be able to support exposure of UEs' sensing results and assistance information (e.g. UE location), to a trusted 3rd-party application, e.g., to facilitate 3rd-party control and coordination of sensing inputs from one or more sensing UEs outside the 5GS.

* * * End of Change * * * *