3GPP TSG SA WG 1 Meeting #102		 S1-231132

Berlin, Germany,  22 - 26 May 2023

Title:	[DRAFT] Reply LS on access to stand-alone non-public network services via PLMN and vice versa

Response to:	LS C1-232971 on access to stand-alone non-public network services via PLMN and vice versa from CT1

Release:	-

Work Item:	-

Source:	SA1

To:	CT1

Cc:	SA2

Contact person:	Xiaonan Shi

shixiaonan@chinamobile.com

+86 18810577369

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA1 thanks CT1 for this LS on access to stand-alone non-public network services via PLMN and vice versa. In the incoming LS, CT1 express they cannot reach consensus about the following information:

1. Whether a UE needs to take into account the non-public network services the UE is currently using when the UE selects a PLMN to access these non-public network services, to ensure service continuity (and vice versa)?

2. Whether a UE needs to take into account which non-public network services the UE intends to access when the UE selects a PLMN to access the non-public network services, to ensure the selected PLMN has agreement/SLA with the non-public network (and vice versa)?

3. Can the UE use any RAT of a PLMN to access non-public network services via the PLMN or take into account RAT for service experience consistency of the non-public network services?

In SA2, TS 23.501 has followed the requirements from SA1 and further specified PDU session continuity, reference architectures for underlay/overlay network, support QoS related to 5G system feature in clause 5.30.2.7/5.30.2.8 and annex D.

For question 1, SA1 answer is yes.

1. SA1 think a UE should take into account the non-public network services the UE is currently using when the UE selects a PLMN to access. As in TS 22.261,

Subject to an agreement between the operators and service providers, operator policies and the regional or national regulatory requirements, the 5G system shall support for non-public network subscribers:

- access to subscribed PLMN services via the non-public network;

- seamless service continuity for subscribed PLMN services between a non-public network and a PLMN;

- access to selected non-public network services via a PLMN;

- seamless service continuity for non-public network services between a non-public network and a PLMN.

SA1 think it is assumed that in the agreement between the operators and service providers, it should be clarified in which condition (such as which service UE is currently using) service continuity should be supported in order to access to selected non-public network services via a PLMN.

2. For question 2, SA1 answer is yes.

The reason is similar to the answer of question 1. The agreement between the operators and service providers ensure that which NPN service can be access via which PLMN (and vice versa). In addition, many NPNs have very critical KPIs about service SLA when they provide service to users. If the NPN that UE intend to access do not have agreement with the PLMN the UE is going to use, the critical KPI requirement will not be satisfied and the access will even fail. So a UE needs to take into account which NPN services the UE intends to access when the UE selects a PLMN to access the NPN services, to ensure the selected PLMN has agreement/SLA with the NPN (and vice versa), so that the KPI requirement of this service can be satified.

3. For question 3, SA1 answer is no, UE cannot use any RAT of a PLMN to access non-public network services via the PLMN, and UE should take into account RAT for service experience consistency of the non-public network services

In TS 22.261, as the scope is within 5G system including a UE, NG-RAN, and 5G Core network, the condition is under 5G network. So SA1 don’t think a UE can use any RAT of a PLMN to access non-public network services via the PLMN.

2	Actions

To CT1

ACTION:

SA1 asks CT1 to take the above answers into account.

3	Dates of next TSG SA WG 1 meetings

SA1#103	21-25 Aug 2023			Gothenburg, Sweden

SA1#104	13 – 17 Nov. 2023		Chicago, USA