3GPP TSG-SA WG1 Meeting #102 															S1-231251

(Revision of S1-230623)

22 – 26 May, 2023, Berlin, Germany

Source:	OPPO, CMCC, Futurewei, vivo, Huawei？

pCR Title:	Consolidation on KPI for Ambient IoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	xuweijie@oppo.com,xuyang@oppo.com, amanda.xiang@futurewei.com

Abstract: This document provides a Text Proposal for the consolidation on KPI for Ambient IoT.

Discussion

30 use cases and 3 traffic scenarios have been agreed and captured in the latest version of TR 22.840 v1.0.0.

KPI consolidation methodology:

In order to do KPI consolidation for ambient IoT use cases. The following methodology and steps have been proposed and performed:

- Step 1: Grouping use cases into several categorizations based on the functionality that ambient IoT can provide;

- Step 2: For each categorization, the use cases are further grouped based on their deployment scenarios, i.e. indoor, outdoor.

- Step 3: For each sub-group use cases under each categorization, merge the KPI values and generate the consolidated KPI values.

Use case categorization and grouping:

Step 1 and step 2 are for use cases categorizations and grouping. The following motivation is identified for step 1 and step 2:

Motivation of use cases categorizations:

- There are different requirements (payload size, device density, transfer intveral, need of positioning etc.) for different categorizations and different deployment

- There may be similar requirements for use case under each combination of a categorization and a deployment scenario (e.g. indoor +inventory, outdoor + sensor, indoor + actuator).

  - For some combination, the KPI may be still diverse for different. In this case, the use cases can be further grouped into different sub-types.

- KPI consolidation is performed for each combination or for each sub-type (i.e. depending on the KPI diversity). The consolidated KPI will facilitate the specification implementation for the downlink group by take the specific characteristcis of each combination or sub-types.

For step 1, based on the functionality that ambient IoT provides, the use cases and traffic scenarios are put into the following 4 categorizations as in the following:

- Use case categorization 1: Inventory

- Use case categorization 2: Sensors

- Use case categorization 3: Positioning

- Use case categorization 4: Actuator (e.g., switch)

In this step, it is noted that some use cases invole more than one function, in order to avoid KPI dupulication it is proposed one use case is only grouped within one   categorization based on the main function that ambient IoT that can provide for them. For example, use case 5.1 Automated warehousing is only put into categorization 1: Inventory, use case Ambient IoT devices for smart laundry is only put into categorization 2: Sensors. Traffic scenario 6.3 electronic shelf label is put into categorization 4: Actuator.

For step 2, the above 4 categorizations are further grouped based on their deployment scenarios, i.e. indoor, outdoor.

Similarly, some use cases, it related to both indoor and outdoor. For this kind of use cases, it is grouped into only indoor or outdoor based on the most possible deployment scenario. For example, use case 5.7 Airport terminal / shipping port is mainly an indoor use cases althougu it also related to outdoor in some cases.

Using above methodology, all the use cases and traffic scenarios with KPI values are catorgorized and grouped into Table 1:

Table 1: Ambient IoT use case categorization and grouping

| Use cases Categories | Use cases agreed in TR 22.840 v0.3.0 |
| --- | --- |
| Inventory | 1; 2; 5; 7; 16; traffic scenario 1 |
| Sensors | 3; 6; 13; 15; 18; 19; 20; 22; 23; 24; 25; traffic scenario 2; |
| Positioning | 8; 9; 10; 12; 14; 21; |
| Actuator | 11;26;30; traffic scenario 3 |

KPI consolidation:

After use case categorization and grouping, the KPI consolidation is performed. The specific consolidation method is:

Firslt, the KPI values from use case under each combination of a categorization and a deployment scenario (e.g. indoor +inventory, outdoor + sensor, indoor + actuator) will be checked:

  - If all the values are within a similar range, the KPIs from all the use case under this combinarion will merged (consolided) into one row (as highlighted in green).

    - Example include indoor inventory etc.

  - Otherwise, the KPI may be still diverse for different use cases. In this case, the use cases within each combination will be further grouped into different sub-types. And the KPIs from all the use case under each sub-type will merged (consolided) into one row (as highlighted in green).

    - Example includes indoor sensor (includes 2 sub-type: Room envirionment monitoring and Indoor agriculture and husbandry), outdoor sensor (include smart grid, Outdoor husbandry and logistics and smart city) ect as in the following.

Table 2 KPI consolidation for inventory

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Automated warehousing | 1s | 99% | NA | <100/128bits/s | 96/128 bits | [NA] | 30m indoors | NA | 5~10km/h | NA | NA | NA | 2~3 m | UC#5.1 |
|  | 2 | Medical instrument inventory management and positioning | hundreds ms level | 99% | NA | <2Kbps | 176bits | ≥1000/km2 | 50m Indoor 200m Outdoor | NA | Static or walking speed <6km/h | NA | NA | NA | NA | UC5.2 |
|  | 3 | Automatic Intralogistics in automobile manufacturing | >100 ms | 99% | NA | <1 kbit/s | 96 bits | <1,5 Million/km2 | <30 meters Indoors | 600 000 m2 | NA | NA | NA | NA | [3] m | UC#5.5 |
|  | 4 | Airport terminal / shipping port | >1sec | 99% | NA | NA | 256 bits (UL) | >100 devices/1km2 | 50m | 1-10km2 | NA | NA | NA | NA | NA | UC#5.7 |
|  | 5 | Ambient IoT devices for automated supply distribution | >10 s | 99% | NA | <100 bit/s | Typically, <100 bytes | <1,5 Million/km2 | NA | 600 000 m2 | NA | NA | NA | NA | [3] m (Indoor, 90% confidence level and in horizontal) | UC#5.16 |
|  | 6 | Container logistics in a flower auction | [<10 s] | [99,99%] | NA | [5] kbit/s | [96] bits | [< 1,3] Million/km2 | [35] m Indoors | [1 700 000] m2 | NA | NA | NA | NA | NA | TS#6.1 |
|  |  | Indoor inventory or asset management | Typically, second level | 99% | NA | <1 Kbps | 96/256 bits | <1,5 Million/km2 | 30~50m indoo 200 outdoor | 1-10km2 | 5~10km/h | NA | NA | NA | 3 m | UC#5.1, 5.2, 5.5, 5.7, 5.16 TS#6.1 |
| Outdoor | 7 | Medical instrument inventory management and positioning | hundreds ms level | 99% | NA | <2Kbps | 176bits | ≥1000/km2 | 200m Outdoor | NA | Static or walking speed <6km/h | NA | NA | NA | NA | UC5.2 |
|  |  | outdoor inventory or asset management | Typically, second level | 99% | NA | <1 Kbps | 96/256 bits | <1,5 Million/km2 | 200m | 1-10km2 | 5~10km/h | NA | NA | NA | Cell-level |  |

Table 3 KPI consolidation for sensors

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Ambient IoT devices in smart home | 20 s | 99.9 % | NA | NA | 8~96bits | NA | [10-30]m Indoors | NA | Stationary | NA | NA | NA | NA | UC#5.6 |
|  | 2 | BSMR environmental supervision | 1s | 99% | 99.9% | <1kbit/s | NA | NA | 30m indoors | NA | NA | NA | NA | NA | NA | UC#5.13 |
|  | 3 | Smart Agriculture | >1 s | 99.9% | NA | <1 kbit/s | <1000 bits | 1 per m2 | 30-100m | 500-70000 m2 per greenhouse | static | 1 hour | NA | NA | NA | UC#5.20 |
|  | 4 | Smart livestock farming (pig barns) | >10 s | NA | NA | <500 bit/s | Typically, [< 100 bytes] | 850 000 devices / km2 | 250 m Indoor | 6000 m2 | NA | 15 minutes to half an hour | NA | NA | NA | UC#5.23 |
|  | 5 | Cows in dairy stable | 1 s | [99,9%] | NA | [< 0.5] kbit/s | [500] bits | [< 1] /km2 | [35] m Indoors | [1 800] m2 | NA | NA | NA | NA | NA | TS#6.2 |
|  |  | Room envirionment monitoring | 1-20 s | 99 % | TBD | <several 100 bit/s | < 100 bits | NA | [10-30]m | NA | Stationary | NA | NA | NA | NA | UC#5.6, 5.13 |
|  |  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, [< 100 bytes] | 1 per m2 | 30 – [100] m | 6000 m2 | NA | 15 minutes to half an hour | NA | NA | NA | UC#5.20, 5.23, TS#6.2 |
| Outdoor | 6 | Remote monitoring of transmission and distribution networks in smart grids | 1 s | 99% | NA | < 1kbit/s | Typically, < 100 bytes | < 10,000 /km2 | Outdoor: typically 50-200 meters | [several km2 up to 100 000 km2] | Stationary | 5-15 min | NA | NA | several 10 m | UC#5.3 |
|  | 7 | Forest Fire Monitor | > 10sec | 99.9% | NA | NA | NA | 100 per km2 | [15-200] meters | [10000 –400,000] km2 | static | 1hour | NA | NA | NA | UC#5.19 |
|  | 8 | Smart dairy farm | >1 s | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km2 | [300 m - 500 m] Outdoors | 430000 m2 | NA | 15 min | NA | NA | NA | UC#5.22 |
|  | 9 | Smart manhole cover remote monitoring | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km2 | [300 m - 700 m] Outdoors | City wide including rural areas | Stationary | 15 min | NA | NA | NA | UC#5.24 |
|  | 10 | Smart bridge health monitoring | 10 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km2 | [300 m - 500 m] Outdoors | Along the bridge | NA | 15 min | NA | NA | NA | UC#5.25 |
|  | 11 | Smart laundry | >10 s | NA | NA | <100 bit/s | Typically, <100 bytes | 20 / 100m2 | NA | several m2 up to 1000 m2 | NA | NA | NA | NA | NA | UC#5.15 |
|  | 12 | Ambient IoT devices for food supply chain | >1 minute | NA | NA | <0.12 bit/s | Typically, < 100 bits | 1.5 Million devices/ km2 | NA | 30,000m2 | NA | NA | NA | NA | NA | UC#5.18 |
|  |  | Smart grid | 1 s | 99% | NA | < 1kbit/s | Typically, < 100 bytes | < 10,000 /km2 | Typically 50-200 meters | [several km2 up to 100 000 km2] | Stationary | 5-15 min | NA | NA | several 10 m | UC#5.3 |
|  |  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km2 | [300 m - 500 m] | 430000 m2 | NA | 15 min | NA | NA | NA | UC#5.22, 5.18, 5.19 |
|  |  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km2 | [300 m - 700 m] | City wide including rural areas | Stationary | 15 min | NA | NA | NA | UC#5.24, 5.25 |

Table 4 KPI consolidation for positioning

| Deployment |  |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 |  | Remote lost item finding (Indoor) | >5s | 99% | NA | NA | 256 bits | <750 devices/100m2 | 10m | NA | NA | NA | NA | NA | NA | UC#5.8 |
|  | 2 |  | Finding Items in a home | NA | NA | NA | NA | NA | 20 Ambient IoT devices/(100m2) | 10m IC/PC/OOC | NA | Static/ Moving(<1m/s) | 500ms | NA | 95 % | 100 cm, 10 degree | UC#5.10 |
|  | 3 |  | Personal belongings finding (indoor) | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <5 per 100 m2 | 10 m | <200 m2 | Static | 1 per hour | 1s | 99% | [1-3] m, 90% availability | UC#5.12 |
|  | 4 |  | Indoor positioning | 0.5 s | 99.9% | NA | <1 kbit/s | 96 bits | 2500/ 10000m2 | 10 m | NA | NA | NA | 0.5 s | 90% | 3 m | UC#5.14 |
|  | 5 |  | Museum guide (indoor) | [2] s | 99.9% | NA | [< 1 kbit/s] UL | [96] bits | [<10,000 /km²] | [30] m | [20,000] m² | [3] km/h | NA | NA | 90% | [3] m | UC#5.21 |
|  |  |  | Indoor positioning | 1s | 99.9% | NA | <1 Kbps | <1k bits | 25/100 m2 | 10m indoors | 200 m2 | Static/NA | 1 per hour | 1s | 95% | 1-3 m, 90% availability | UC#5.12, 5.14 |
| outdoor | 4 | Absolute positioning |  | NA | NA | NA | NA | NA | NA | NA | NA | Indoor - up to [5 km/h] Outdoor - up to [20 km/h] | NA | [10 s] | [95 %] | [10 m] Horizontal Accuracy [3 m] Vertical Accuracy | UC#5.9 |
|  | 5 | Personal belongings finding (outdoor) |  | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m2 | 100 m | Up to the whole PLMN | Static | 1 per hour | 1 s | 99% | FFS | UC#5.12 |

Table 5 KPI for actuators using ambient IoT

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Medical instrument inventory management and positioning | hundreds ms level | 99% | NA | <2Kbps | 176bits | ≥1000/km2 | 50m Indoor | NA | Static or walking speed <6km/h | NA | NA | NA | NA | UC#5.11 |
|  | 2 | Indoors elderly health care scenario | 1 s | NA | NA | <1 kbit/s | <100 bits | <20 per 100 m2 | 20 m | <250 m2 | Static | NA | NA | NA | NA | UC#5.26 |
|  | 3 | Electronic Shelf Label | 1s | 99% | NA | 0.8kbps DL | 100 Bytes | FFS | 50m indoors | 15,800 square meters | stationary | FFS | NA | NA | NA | TS#6.3 |
|  |  | Indoor actuators | 1s | 99% | NA | 1kbps | 100 Bytes | FFS | 50m indoors | 15,800 square meters | stationary | FFS | NA | NA | NA | UC# 5.26,TS#6.3 |
| Outdoor | 4 | Medical instrument inventory management and positioning | hundreds ms level | 99% | NA | <2Kbps | 176bits | ≥1000/km2 | 200m Outdoor | NA | Static or walking speed <6km/h | NA | NA | NA | NA | UC#5.11 |
|  | 5 | Outdoors elderly health care scenario | 1 s | NA | NA | <1 kbit/s | <100 bits | <200 per 100 m2 | 200 m | City wide including rural areas | Static | NA | NA | NA | NA | UC#5.26 |
|  | 6 | Ambient IoT controller in smart agriculture | hundreds ms level | 99% | N/A | NA | 128bit (DL) | NA | [500]m outdoors | NA | Static | NA | NA | NA | NA | UC#5.30 |
|  |  | Outdoor actuators | 1 s | 99% | N/A | <1kbps | 128bit (DL) | NA | [500]m outdoors | NA | Static | NA | NA | NA | NA | UC#5.26, 5.30 |

****************Start of Change *************

7.2	Consolidated potential KPIs

7.2.1 KPIs for inventory

Table 7.2.1-1 KPIs for inventory

|  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Indoor inventory or asset management | Typically, second level | 99% | NA | <1 Kbps | 96/256 bits | <1,5 Million/km2 | 30~50m indoo 200 outdoor | 1-10km2 | 5~10km/h | NA | NA | NA | 3 m | UC#5.1, 5.2, 5.5, 5.7, 5.16 TS#6.1 |

7.2.2 KPI for sensor

Table 7.2.2-1 KPIs for sensors

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor |  | Room envirionment monitoring | 1-20 s | 99 % | TBD | <several 100 bit/s | < 100 bits | NA | [10-30]m | NA | Stationary | NA | NA | NA | NA | UC#5.6, 5.13 |
|  |  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, [< 100 bytes] | 1 per m2 | 30 - 200 m | 6000 m2 | NA | 15 minutes to half an hour | NA | NA | NA | UC#5.20, 5.23, TS#6.2 |
| outdoor |  | Smart grid | 1 s | 99% | NA | < 1kbit/s | Typically, < 100 bytes | < 10,000 /km2 | Typically 50-200 meters | [several km2 up to 100 000 km2] | Stationary | 5-15 min | NA | NA | several 10 m | UC#5.3 |
|  |  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km2 | [300 m - 500 m] | 430000 m2 | NA | 15 min | NA | NA | NA | UC#5.22, 5.18, 5.19 |
|  |  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km2 | [300 m - 700 m] | City wide including rural areas | Stationary | 15 min | NA | NA | NA | UC#5.24, 5.25 |

7.2.3 KPI for positioning

Table 7.2.3-1 KPIs for positioning

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor |  | Indoor positioning | 1s | 99.9% | NA | <1 Kbps | <1k bits | 25/100 m2 | 10m indoors | 200 m2 | Static/NA | 1 per hour | 1s | 95% | 1-3 m, 90% availability | UC#5.12, 5.14 |

7.2.4 KPI for actuator

Table 7.2.4 KPIs for actuators

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor |  | Indoor actuators | 1s | 99% | NA | 1kbps | 100 Bytes | FFS | 50m indoors | 15,800 square meters | stationary | FFS | NA | NA | NA | UC# 5.26,TS#6.3 |
| Outdoor |  | Outdoor actuators | 1 s | 99% | N/A | <1kbps | 128bit (DL) | NA | [500]m outdoors | NA | Static | NA | NA | NA | NA | UC#5.26, 5.30 |

*******************End of Change*****************