3GPP TSG SA WG 1 Meeting #102	S1-231277

Berlin, Germany, 22 - 26 May 2023	(revision of S1-23xxxx)

Source:	Huawei

pCR Title:	Pseudo-CR on quality improvements to FS_EnergyServ TR 22.882

Draft Spec:	3GPP TR 22.882 v1.0.0

Agenda item:	7.10

Document for:	Approval

Contact:	Edward Hall

edward.hall@huawei.com

Abstract: This is a quality improvement CR for TR 22.882 v1.0.0 which implements the 3GPP specification drafting rules and improves the use of language.

1. Introduction

Before sending TR 22.882 for approval, it is important to fully implement the 3GPP specification drafting rules found in TR 21.801 and to ensure good quality language. The present pCR provides quality improvements to TR 22.882.

2. Reason for Change

The present pCR provides quality improvements to TR 22.882.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.882 v1.0.0.

* * * First Change * * * *

# 1	Scope

The present document provides stage 1 use cases and potential 5G requirements on the following aspects regarding enhancements to Energy Efficiency of 5G network:

- Defining and supporting energy efficiency criteria as part of communication service to user and application services;

- Supporting information exposure of systematic energy consumption or level of energy efficiency to vertical customers;

- Gap analysis between the identified potential requirements and existing 5GS requirements or functionalities;

- Potential requirements on security, charging and privacy aspects.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI ES 201 554: "Environmental Engineering (EE); Measurement method for Energy efficiency of Mobile Core network and Radio Access Control equipment".

[3] 	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[4] 	GSMA Intelligence: "Going green: benchmarking the energy efficiency of mobile", June 2021.

[5] 	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[6] 	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[7] 	3GPP TR 28.813: "Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G".

[8] 	3GPP TR 38.864: "Study on network energy savings for NR".

[9]	ETSI ES 202 336-1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 1: Generic Interface".

[10]	ETSI ES 202 336-12: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[13]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	Void

[15]	3GPP TS 22.261: "Service requirements for the 5G system".

[16]	3GPP TS 22.115: "Service aspects; Charging and billing".

[17]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[18]	3GPP TS 32.299:  Telecommunication management; Charging management; Diameter charging applications".

[19]	NGMN: "NGMN Energy Efficiency White Paper, Phase 2", Dec 2022.

[20]	GSMA: "5G energy efficiencies: green is the new black", Nov 2020 .

[21]	Renewable Energy Certificates (RECs): https://www.epa.gov/green-power-markets/renewable-energy-certificates-recs

[22]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[23]                      ETSI GS OEU 020 (v1.1.1): "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

* * * Next Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

energy state: state of a cell, a network element and/or a network function with respect to energy, e.g. (not) energy saving states, which are defined in TS 28.310 [6].

energy charging rate: a means of determining the energy usage consequence (use of energy credit) associated with charging events.

NOTE 1:	The rate listed above is a charging rate, completely distinct and unrelated to the 'Maximum Energy Utilization Rate' as discussed in 5.1.

energy credit: a quantity of credit associated with the subscriber that can be used for credit control by the 5G system.

maximum energy credit limit: a policy establishing an upper bound on the quantity of energy used by the 5G system to provide services provided to a specific subscriber.

carbon emissions: kilograms of equivalent carbon dioxide emitted (kg of CO2 equivalent)

carbon intensity: quantity of CO2 equivalent emission per unit of final energy consumption for an operational period of use [23]

renewable energy: energy from renewable sources as energy from renewable non-fossil sources, namely wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 2: This definition was taken from [22].

* * * Next Change * * * *

# 4	Overview

Climate change and global energy shortage are issues that requires international cooperation and coordinated solutions at all levels, many regions and countries have published related policies and requirements to control carbon release and promote energy efficiency. These policies have made energy efficiency a strategic priority for many telecom operators around the world. Energy efficiency has been considered in many standard groups and specifications.

The existing studies concentrate more on how to satisfy user experience and try to achieve energy efficiency at the same time and achieve energy efficiency within the network, so the requirements, use cases and solutions are basically within the network itself. Verticals and customers have no approach for energy efficiency related information from the network.

Introducing energy efficiency as a service will allow users to have the choice to select proper energy efficiency criteria as well as other network performance parameters when they need them, which may include:

-	Define and support energy efficiency criteria as part of communication service to user and application services;

-	Provide information exposure on systematic energy consumption or level of energy efficiency to vertical customers.

Such as in satellite and terrestrial convenience scenario, for some regions where both satellite and terrestrial coverage exist, energy saving could be taken as a dimension while providing the communication service, users or operators could have the choice to find out a best way in satisfying both user experience and energy efficiency. From another perspective, the network could also react to different energy consumption modes of applications or adjust network resources.

Both aspects above need more interactions between applications and networks on energy consumption status. It is worth considering how to deliver services with energy efficiency as service criteria, associated with verticals’ preferences, and how to support the policy of handling energy as part of a subscription.

* * * Next Change * * * *

## 5.1	Energy Utilization as a performance criteria for best effort communication

### 5.1.1	Description

Currently energy utilization and efficiency can be monitored and considered through O&M and network operation, but not as a service performance criterion, as for example bit rate, latency or availability. Guidance from SA to all working groups states:

"The EE-specific efforts so far undertaken e.g., in SA5 have aimed mostly at improving the energy efficiency by impacting the operations of the system. As we now are starting to specify the 5G-Advanced features, TSG SA kindly requests the recipient WGs and TSGs to consider EE even more as a guiding principle when developing new solutions and evolving the 3GPP systems specification, in addition to the other established principles of 3GPP system design.

TSG SA clarifies that in addition to EE, other system level criteria shall continue to be met (i.e.  the energy efficiency aspects of a solution defined in 3GPP is not to be interpreted to take priority or to be alternative to security, privacy, complexity etc. and to meeting the requirements and performance targets of the specific feature(s) the solution addresses)."

There is an important type of traffic where energy efficiency policy, for example a maximum amount of energy to be utilized could be applied without conflict with this guidance. Best effort traffic is a type of traffic that is provided as a service to customers everything else being equal. Of course, security, privacy and complexity principles will not be sacrificed, but there is no conflict between a service policy that constrains performance (e.g. latency, throughput, even availability) on the basis of energy utilization and a best effort service, since there are no guarantees in the case of best effort traffic. We can say that best effort traffic is not associated with QoS policy service performance level criteria.

Today the 5G system works to support services efficiently, though does not take into account energy utilization at the service level. The use case explores a particular opportunity to identify this information and use it to make more efficient use of all network resources without sacrificing service quality. In particular, information gathered through O&M, and in the future possibly from the network (see 5.1.5 which identifies a gap and opportunity), can be leveraged to make it possible to employ energy utilization information as part of service delivery.

In the following use case, the possibility of using energy utilization as a new service criterion for this less constrained type of mobile telecommunication service is explored.

A large-scale logistics company L has deployed a large number of communicating components. These are integrated into vehicles, palettes, facilities, etc. Essentially, IoT terminals enable remote tracking and monitoring functions. The information gathered is relevant, but not constrained with respect to latency. In fact, eventual delivery (e.g. after hours or even a full day) of communication is entirely acceptable for L. The MNO M offers a 'green service' which limits the rate of energy utilized for communication over a particular time interval (e.g. per day) and this service is appropriate for L, whose overall corporate goals are also served by 'green service,' as they strive to operate with energy efficiency.

* * * Next Change * * * *

### 5.1.3	Service flows

1.	The fleet of trucks belonging to L leave the logistic center located in the middle of the uninhabited region hundreds of kilometers northeast of the major city Erehwon. There are many devices located in this fleet. The trucks and their contents comprise a physically dense group of UEs, all communicating periodically with the network. This 'massive IoT' group leaves the coverage of the logistics center. The network coverage over the road through the uninhabited region is very sparse.

2.	As the trucks proceed into extreme low coverage, the energy utilized to communicate with the IoT devices increases. This energy utilization increase is monitored by the 5G network and can be aggregated, e.g. at the slice level.

3.	The 'green service' policy for the service provided to L includes a maximum energy utilization rate. At a certain point the IoT communication of the fleet exceeds this maximum energy utilization rate.

4.	The policy indicates that latency can be traded off with energy utilization for service to L; the communication service is delay tolerant in this condition. As the energy utilization rate has exceeded the maximum, the latency is increased to enforce this policy. In effect, L's fleet receives very limited service, with high latency, even for a limited period of time, no service at all.

NOTE:	This use case does not describe how latency is increased, but does assume that this increase will result in a reduction of energy utilization. It is possible to reduce energy utilization by offering less service.

The use case description does not define how operator M offers the 'green service'. One possibility is that the maximum energy utilization policy applies to all services for the subscription of a device deployed by L with operator M. This simple policy may not be appropriate if the UEs deployed by L use different kinds of services at different times. In this case, the policy would apply to specific services (service flows, etc.) A requirement at the service flow level is not pursued in this use case.

A further option is that specific network slices apply a 'green service' policy to all services communicating by means of that slice.

The use case does not describe how energy utilization is determined. There is related work in SA5 and RAN3 to determine energy utilization. If energy utilization cannot be determined at the granularity, e.g. of a specific service or network slice or even the aggregate energy utilization of a subscriber, it is still possible to identify the total energy utilization of different elements in the 5G network. It is therefore possible, at least in principle, to divide the total energy by the number of served sessions, subscribers, etc. 'Average utilization' of a node or cell or slice, etc. is a course unit of measurement, and does not reflect the true energy utilization at the finer granularity, though it still can be a useful metric.

Though an averaging approach could be useful to count the total amount of energy used to attribute to each subscriber, this approach is not enough to measure the rate of energy utilization as described in this use case. For this, there would have to be finer granularity energy reporting than 'per node' or 'per cell.' Though this is not yet supported in the 5G network.

* * * Next Change * * * *

### 5.1.4	Post-conditions

The IoT devices in the fleet belonging to L are able to communicate with varying latency, depending on the energy utilization required to serve the devices. When the UEs are in poor coverage, they communicate seldom, when under good coverage, they can communicate more frequently.

The total energy utilization of M's network has reduced while still providing adequate service to customer L.

It is important to emphasize that there has been no trade-off between 'energy efficiency' and 'service quality.' Customer L received what was necessary while using less energy precisely because the energy utilization was taken into account in the service delivery.

* * * Next Change * * * *

### 5.1.6	Potential new requirements needed to support the use case

[PR.5.1.6-1]	Subject to operator’s policy, the 5G network shall support subscription policies that define a maximum energy utilization rate for services without QoS criteria (also termed "best effort" services.)

NOTE 1:	The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. This requirement's applicability is limited to UEs that only support services without QoS criteria.

[PR.5.1.6-2]	Subject to operator’s policy, the 5G network shall support enforcement of subscription policies that define a maximum energy utilization rate for services without associated QoS criteria (also termed "best effort" services.)

[PR.5.1.6-3] The 5G network shall support a means to define maximum energy utilization rate with specific granularities:

a)	subscriber granularity (considering all services of the 5G network for the subscriber);

b)	network slice granularity.

NOTE 2:	The energy utilization of the UE is out of scope of this requirement.

[PR.5.1.6-4]	Subject to operator's policy, the 5G network shall support energy utilization monitoring at per network slice and per subscriber granularity.

NOTE 3:	Energy utilization monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required.

* * * Next Change * * * *

### 5.2.3	Service flows

1.  Operator T provides a dedicate set of UPF and MEC platform for factory F. Factory F is an environmental conscious enterprise that cares about energy saving (and efficiency) along its whole industrial chain.

2. When the manufacturing load of Factory F reaches a certain threshold (lower or higher), which is evaluated by Factory F, a notification will be sent to Operator T.

3.  Operator T will change the energy state of the dedicated network functions accordingly to energy saving, based on the pre-agreed policy with Factory F.

4. After one year of this kind of usage, the charging information of the communication service will consider the actual usage time of the different energy states.

* * * Next Change * * * *

### 5.2.6	Potential new requirements needed to support the use case

[PR.5.2.6-1] The 5G system shall support different energy states of network elements and network functions.

[PR.5.2.6-2] The 5G system shall support dynamic changes of energy states of network elements and network functions, based on pre-configured policy with authorised 3rd party.

NOTE: Pre-configured policy may include: the time of changing energy states, which energy state map to which level of load, etc.

[PR.5.2.6-3] The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions.

* * * Next Change * * * *

### 5.3.3	Service flows

1. The 5G network “N” of operator A acquires the energy usage information of related 5G network functions serving customers C and D.

2. Customer C asks the “Green Energy Moni” of Operator A to provide the network energy usage information associated with the 5G network functions serving it via dual-connectivity deployment.

3. Operator A provides the network energy usage information to customer C.

4. Customer D asks the “Green Energy Moni” of Operator A to provide the network energy usage information associated with the 5G network functions serving it via DU deployment.

5. Operator A provides the network energy usage information to customer D.

* * * Next Change * * * *

### 5.3.6	Potential new requirements needed to support the use case

[PR.5.3.6-1] Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios, and expose this information to the customer and authorized third parties.

* * * Next Change * * * *

### 5.4.3	Service flows

1. The 5G network of operator A acquires the energy efficiency information of the NPN ”N1” and PLMN “R”.

2. Customer C asks the “Green Energy Moni” of Operator A to provide the energy efficiency information of its network “N1”.

3. The operator A acquires and provides energy efficiency information of the network “N1” to customer C.

* * * Next Change * * * *

### 5.4.6	Potential new requirements needed to support the use case

[PR.5.4.6-1] Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy efficiency information of the NPN, including the shared network function(s) which is (are) serving the NPN, and expose this information to the NPN customer and authorized third parties.

* * * Next Change * * * *

## 5.5	Use Case on service energy monitoring by an application server

### 5.5.1	Description

In this scenario, a service provider monitors events resulting from energy utilization policy triggers in the 5G system. These triggers correspond to monitoring policy in the 5G system as well as energy enforcement policies.

Figure 5.5.1: Monitoring of Energy Events by the 5G network for an AS

In Figure 5.5.1, the application server AS obtains information corresponding to the energy consequences of a UE 'A' served by the 5G network.

This use case will provide a description of a scenario in which the service provider (who operates an application server) cares about energy utilization in the 5G system as a result of the service to UE A. This could be for 3 reasons:

- the service provider needs to show they are saving energy;

- the service has an associated energy cost, and the service provider wants to reduce it. This is analogous to the use of industrial or consumer electronics when energy rates are lower, and also as an incentive to operate more efficiently;

- the service provider recognizes that there are policies that limit energy use (such as aggregate energy use of a slice) and controls the overall use of the service to operate within those constraints.

The use case introduces five new concepts related to new energy events and energy event monitoring:

a)	the ability for the network operator to create a 'maximum energy credit' policy, after which services are gated;

b)	the ability for the network operator to inform an AS of the 'maximum energy credit expired' event;

c)	the ability for the 5G system to calculate 'energy credit' use;

d)	the ability to monitor and provide to the AS the use of 'energy credits' (or other energy 'quantum');

e)	the support of a new policy that establishes the energy consequence for charging control - either charging for use of energy or establishing an 'energy credit limit' for enforcement by the 5G system.

* * * Next Change * * * *

### 5.5.3	Service flows

1.	The application service provider of AS has an energy policy related to the service for the subscription related to UE "A". As a result, AS requests to monitor 'Energy Use', which is a kind of usage monitoring supported by the 5G system. The monitoring policy has an established 'threshold' for the 5G system to notify the AS.

In addition, the AS requests to monitor 'Out of Energy Credit' events.

2.	The 5G system provides service to UE A according to a 'best effort communication subject to energy constraints' policy, where the policy charges for energy use and also imposes an 'energy credit' limit, after which the UE A subscription is 'gated' (receives no further services from the 5G system until more 'energy credit is available).

3.	UE A proceeds to use services of the 5G system, especially data communication. As it does so, the charging system is triggered and generates records. The 3GPP charging system uses a means to identify how much credit is used and whether a credit limit is exceeded. The 3GPP charging system in this use case also uses a means of calculating energy credits on the basis of charging events. That is, there is a 'rating policy' used to multiply a 'charging event' by an 'energy utilization' unit.

NOTE 1:	The actual amount of energy corresponding to an 'energy unit' used in energy credit control is out of scope of this use case. A mobile network operator can develop a model by which they analyze the total energy needed to provide services and assign fractions of these to each event triggered in the charging system.

4.	When the total 'energy units' exceed the reporting threshold according to the energy monitoring policy, the 5G system exposes this energy usage information to AS.

NOTE 2:	Monitoring of energy usage could be done by other means than 'energy units' corresponding to the same units as the credit limit. This could be useful for the third party. However, only by exposing units that result in charging or gating enforcement by the network operator can the third party determine the consequences of their use of services and potentially change their use of those services, e.g. to communicate sparingly, to communicate more efficiently (e.g. at times in less energy use is reported per 'byte', etc. of communication, as calculated by the third party based on their own measurements and the monitoring reports of the 5G system.)

5.	When the total 'energy units' exceed the energy credit limit, this results in the 5G system exposing this event to the AS. The AS could take some action to restore energy credit, but this is out of scope of the use case.

* * * Next Change * * * *

### 5.5.4	Post-conditions

The UE A's use of energy can be monitored by AS. The AS can alter their activity (e.g. communicate less intensely or less frequently) to remain within their expectation - be it to keep the charging per energy utilization to their expectation, or to avoid exhausting A's energy credit limit.

The MNO is able to create and enforce policies that attach consequences to use of energy. This can lead to energy efficient behaviour on the part of service providers which is both in their interest and the interest of the MNO.

* * * Next Change * * * *

### 5.5.5	Existing feature partly or fully covering use case functionality

The 5G system provides support for credit limits [16, clause 8.2] and for performance monitoring [15]. There are a number of other events that are exposed by the 5G system to third parties by the Policy and charging control framework by the 5G System [17]. These events and their triggers, which are not detailed in stage 1, allow for usage monitoring to be exposed to a third party in specific circumstances, e.g. sponsored connectivity. The scenario in this use case is similar to sponsored connectivity, as the application service provider is a directly concerned party that seeks to operate successfully in an efficient manner, as there are charging and even gating consequences as the UE communicates with AS.

Note that the existing usage monitoring and reporting for sponsored connectivity is not sufficient to support this use case because these do not in any way take into account the energy consequence of the service. Only traffic volume and time-based monitoring are supported today. Other chargeable events (and therefore significant from an energy perspective) are not captured by usage monitoring as supported in the 5G system.

* * * Next Change * * * *

### 5.5.6	Potential new requirements needed to support the use case

[PR.5.5.6-1]	Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services.

[PR.5.5.6-2]	Subject to operator’s policy, the 5G system shall support subscription policies that support a means to associate energy utilization units with charging records.

[PR.5.5.6-3]	Subject to operator’s policy, the 5G system shall support a means to expose energy utilization to authorized third parties for services, such that the energy utilization information clearly identifies the 'approaching' enforcement of an energy credit limit.

[PR.5.5.6-4]	Subject to operator’s policy, the 5G system shall support a mechanism to perform energy utilization credit limit control for services.

NOTE 1:  The result of the credit control is not specified by this requirement. Examples include gating, increased charging rates, etc.

NOTE 2:	Credit control [18] compares against a credit control limit. In this use case, charging events are assigned a corresponding energy utilization and this is compared against a policy of energy credit limit. The use case assumes it is possible that there is a new policy to limit energy utilization allowed.

* * * Next Change * * * *

## 5.6	Use case on supporting service-level energy efficiency analysis for verticals

### 5.6.1	Description

Company A is located in an industrial campus. There are three internal applications used by employees for daily work which are based on two network slices. App A is for internal communication. App B is for production control. App C is for office automation. A and C are running on one slice, while B running on a separate slice. The data of these three applications are all dealt with a locally deployed UPF in this campus. The operator provides the additional service of exposing energy consumption of locally deployed UPF. Company A find that energy consumption of the UPF become higher recently, but cannot find out the cause, hope that 5G system can help to analysis which users or application are abnormal.

5G system analysis the data volume on this UPF and energy consumption of each app periodically.

Figure 5.6.1: Supporting service-level energy efficiency analysis for verticals

* * * Next Change * * * *

### 5.6.3	Service flows

1. Company A finds abnormal energy consumption on the local network entity and request 5G system to report data usage of app A, B and C in past 3 days.

2. 5G system analyses data volume and energy consumption of each app in every 2 hours.

3. 5G system report shows that app B has a large data usage during 3am-5am every day.

4. Company A finds that app B has an abnormal setting which lead to system update repeatedly and large energy consumption.

* * * Next Change * * * *

### 5.6.6	Potential new requirements needed to support the use case

[PR.5.6.6-1] The 5G system shall support service level energy consumption measurement of network functions and exposure to authorised 3rd party.

NOTE: The granularity of service level energy consumption measurement could be vary according to different situations, for example, sub-slice level while several services share a same network slice, or user level based on average calculation, etc.

Editor's Note: This potential requirement is FFS.

* * * Next Change * * * *

### 5.7.3	Service flows

1. Customer M asks the “GreenPark” Operator to provide the “GreenPark” energy usage information and its communication services QoS status information (e.g. the data rate and packet loss) per hour during the working time (e.g. from 9am to 5pm) in the industrial park.

2. Operator acquires the energy usage information of the 5G network functions in the industrial park dedicated serving for customer M per hour.

3. At the same time, operator collects the QoS status information (e.g. data rate and packet loss) per hour, from the 5G network functions which are serving customer M.

4. Operator provides the energy usage information of the network functions serving for the customer M and the QoS status information e.g. the average data rate and average packet loss during the working time in the industrial park.

* * * Next Change * * * *

### 5.7.4	Post-conditions

Customers M can get the not only the energy usage information but also the average data rate and average packet loss of the network functions during the working time. With these information, Customers M can choose more suitable communication service, e.g. it can compare this information among different working days to find the lowest energy consumption with related QoS status information which can support the industry normal work, or try to degrade its QoS demand to reduce energy consumption which maybe in some degree to impact the industry work.

* * * Next Change * * * *

### 5.7.6	Potential new requirements needed to support the use case

[PR.5.7.6-1] Subject to Operator policy and consent by the customer, the 5G system shall be able to, via same update rate e.g. per hour or per day, to expose together the energy usage information and QoS status information of the network to the authorized third party.

NOTE: Which QoS status information need to be exposed together with the energy usage information is depend on implementation which is not in SA1 scope.

[PR.5.7.6-2] Subject to Operator policy, the 5G system shall provide means for the trusted 3rd party to ask which QoS status information is needed together exposed with the network energy usage information.

Editor’s Note: Both requirements are FFS.

* * * Next Change * * * *

### 5.8.3	Service flows

1. Service provider X asks the “App EnergyEff Moni” of Operator A to provide the predicted application service energy efficiency information for App Service #1 and one or more service modes for a given service area and time of the day.

2. The 5G system of operator A acquires the energy usage information of related 5G system functions serving the App Service #1 of service provider X. Then, the 5G system of operator A predicts the AEE for the application service #1 and optionally the service mode X, based on the acquired energy usage information.

3. Operator A exposes the predicted AEE for the application service #1 (and optionally the service mode X) to the Service provider X.

4. Service Provider X configures or adapts the application service parameters based on the Operator A feedback. Such adaptation of the application service parameters can be for instance the application server re-location to an edge data network to enhance the energy efficiency for the application.

* * * Next Change * * * *

### 5.8.6	Potential new requirements needed to support the use case

[PR.5.8.6-1] Based on operator policy, the 5G system shall be able to provide means to allow an application service provider to monitor the energy efficiency of one or more application services (or service levels).

[PR.5.8.6-2] Based on operator policy, 5G system shall be able to provide means to predict the energy efficiency per application service, and expose the predicted notifications to the application service provider.

Editor’s Note: Both potential requirements above are FFS.

* * * Next Change * * * *

## 5.9 	Use case on renewable energy usage information exposure

* * * Next Change * * * *

### 5.9.3	Service flows

1. Customer X subscribes to the ‘Green Energy’ service for its warehouse, provided by operator R

2. Operator R provides a dedicated network slice that utilizes a minimum 80% renewable energy for customer X’s NPN at their warehouse.

3. Customer X receives periodic report every month regarding the ratio of renewable energy utilization from Operator R.

4. Operator R supplies customer X with the requested information.

* * * Next Change * * * *

### 5.9.6	Potential new requirements needed to support the use case

[PR.5.9.6-1] Subject to operator’s policy, the 5G system shall be able to provide to a 3rd party a dedicated NPN or a network slice that operates above a minimum ratio of renewable energy.

NOTE 1: This requirement does not imply that the 5G system will actively monitor the dedicated resources.

Editor's Note: This requirement is FFS.

[PR.5.9.6-2] Subject to operator’s policy, the 5G system shall be able to provide to a 3rd party the reporting of the ratio of renewable energy used to provide dedicated communication service to the 3rd party on periodic basis.

NOTE 2: The reporting period could be set, e.g., on monthly or yearly basis.

* * * Next Change * * * *

### 5.10.3	Service flows

- Eva subscribes the communication service provided by operator A.

- During the commute between the home and the workplace, Eva wears her XR device and enjoys the immersive entertainment via 5G system operated by operator A.

- During the service time, the 5G system incurs carbon emissions due to the energy consumption.

- The operator A collects the carbon intensity information of energy usage from an authorized third party.

- By "carbon-aware communication service”, the operator A calculates the estimated carbon emissions for the service and exposes the estimated carbon emissions result to the service provider B.

- From the service provider B, Eva can know the estimation of carbon emissions for her requested service from the operator A.

* * * Next Change * * * *

### 5.10.6	Potential new requirements needed to support the use case

[PR.5.10.6-1] Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide a mechanism to expose to the authorized third parties the energy efficiency information (e.g., including the estimated carbon emissions) related to a subscriber based on the subscriber’s data volume over a specific period of time, the operator’s energy consumption, and the carbon intensity of operator’s network.

NOTE 1:	The carbon intensity of operator’s network can be provided by an authorized third party and can vary based on locations.

NOTE 2:	The granularity of reporting (e.g., per month) is not discussed in this study.

* * * Next Change * * * *

Annex A (informative):
Existing energy efficiency standardisation

# A.1 Overview of existing energy efficiency standardisation

In ETSI, GSMA and 3GPP, there were many reports, studies, specifications related to energy efficiency. And now there are also ongoing 3GPP R18 studies on energy efficiency in both SA5 and RAN.

In ETSI, existing specifications cover several aspects of energy efficiency, which include energy efficiency metrics and measurement methods for mobile core equipment, metrics and methods to measure energy performance of Mobile Radio Access Networks, measurement and monitoring of power, energy and environmental parameters for ICT equipment in telecommunications. [2] [3]

GSMA has done lots of work in assessing energy consumption in different fields within a communication system. In "Going green: benchmarking the energy efficiency of mobile", GSMA states that 73% of the energy of the participating operators is consumed in the radio access network (RAN). The network core (13%), owned data centres (9%) and other operations (5%) account for the rest. [4] The statistics show that energy efficiency is an end-to-end issue.

In 3GPP, energy efficiency has been studied in SA, SA5 and RAN. SA have studied system requirements and principles and provided an Energy Efficiency Control Framework. [5] SA5 has specified concepts, use cases, requirements and solutions for energy efficiency assessment and optimization for energy saving, as well as Energy Efficiency (EE) KPIs. [6] [7] RAN EE study has concentrated on the definition of network energy consumption models, evaluation methodology and KPIs, also studied and identified techniques on the gNB and UE sides to improve network energy savings in terms of both BS transmission and reception. [8]

* * * Next Change * * * *

# A.2 Energy efficiency KPIs

3GPP Energy Efficiency KPI definitions are under SA5 (Telecom Management) responsibility. They are based on measurements collected on RAN or CN network elements / network functions via OA&M. The KPI calculation is a generalisation of the work in ETSI TC EE. Figure A.2-1 below shows the KPI derivation with notes to the source specifications.

Performance Measurements

(TS 28.552, 3GPP SA5)

Energy Consumption Measurements a.k.a. PEE parameters (Power, Energy and Environmental)

(TS 28.552, TS 28.554, 3GPP SA5, based on input from ETSI ES 203 336-12)

3GPP generic EE KPI = Performance Energy Consumption

is a generalisation of

O&M Collection Method

(TS28.550, TS 28.532, 3GPP SA5)

Defined in ETSI TC EE 203 228

Figure A.2-1: KPI derivation and sources

* * * Next Change * * * *

# A.3 Summary of existing energy efficiency standards

Table A.2-1 below shows the standards relevant to the present document with a synopsis taken from the Scope clause of the standard.

Table A.3-2: List of EE specifications

| SDO | Group | Standard | Summary |
| --- | --- | --- | --- |
| 3GPP | SA | TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards" [5] | Identifies and studies the key issues and the potential solutions in defining Energy Efficiency Key Performance Indicators and the Energy Efficiency optimization operations in existing and future 3GPP networks. |
| 3GPP | SA5 | TS28.310: "Management and orchestration; Energy efficiency of 5G" [6] | Specifies concepts, use cases, requirements and solutions for the energy efficiency assessment and optimization for energy saving of 5G networks. |
| 3GPP | SA5 | TS28.552: "Management and orchestration; 5G performance measurements" [11] | Specifies the performance measurements for 5G networks including network slicing. Performance measurements for NG-RAN are defined in this document, and some L2 measurement definitions are inherited from TS 38.314. The performance measurements for 5GC are all defined in this document. Related KPIs associated with those measurements are defined in TS 28.554 [12]. |
| 3GPP | SA5 | TS28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI) " [12] | Specifies end-to-end Key Performance Indicators (KPIs) for the 5G network and network slicing |
| 3GPP | SA5 | TS28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) " [13] | Specifies the Generic network resource information that can be communicated for telecommunication network management purposes, including management data about energy efficiency |
| 3GPP | SA5 | TR28.813: "Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G" [7] | Investigates the opportunities for defining new Energy Efficiency (EE) KPIs and new Energy Saving (ES) solutions. |
| 3GPP | RAN1 | TR 38.864: "Study on network energy savings for NR" [8] | Investigates network energy consumption modelling, techniques for network energy saving, evaluation of gains and impact. |
| ETSI | TC EE | ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency" [3] | Defines the topology and level of analysis to assess the energy efficiency of mobile networks (excluding terminal) |
| ETSI | TC EE | ETSI ES 202 336-1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface" [9] | Defines monitoring and control of Infrastructure Environment i.e. power, cooling and building environment systems for telecommunication centres and access network locations. |
| ETSI | TC EE | ETSI ES 202 336-12: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model" [10] | Defines measurement and monitoring of power, energy and environmental parameters for ICT equipment in telecommunications or datacenter or customer premises |

Table A.3-2: List of EE specifications