3GPP TSG SA WG 1 Meeting #102 	S1-231313

Berlin, Germany,  22 - 26 May 2023	(revision of S1-23xxxx)

Source:	Deutsche Telekom AG

pCR Title:	Pseudo-CR on introducing overview section of FS_Sensing

Draft Spec:	3GPP TR 22.837 v.1.0.0

Agenda item:	7.1

Document for:	Approval

Contact:	Vasil Aleksiev, vasil dot aleksiev at magenta dot at

Abstract: This pCR introduces overview section of TS22.837 v.1.0.0

1. Introduction

pCR propose to capture overview of the TR 22.837 v.1.0.0.

2. Reason for Change

Currently overview section is empty and there is a need to describe what aspects have been captured during the study phase of Integrated Sensing and Communication.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v.1.0.0.

* * * First Change * * * *

# 4	Overview

Wireless sensing is a technology enabler to acquire information about characteristics of the environment and/or objects within the environment, that uses radio waves to determine the distance (range), angle, or instantaneous linear velocity of objects, etc.

This technical report investigates the potential of integrated sensing and communication technology for enabling new services and use cases for varios industries. 5G wireless sensing service, as part of a cellular network provides new posibilities for enhanced usage of the telecommunication infrastructure in areas of object detection and tracking, environment monitoring and human motion monitoring. It provides input to various verticals - UAVs, smart home, V2X, factories.

The use cases examined in the report cover a wide range of applications, including:

- Intruder detection for smart home, on a highway, for railways, for factory, for predefined secure areas around critical infrastructure

- Collision avoidance and trajectory tracking of UAVs, vehicles, AGVs

- Automotive maneuvering and navigation

- Public safety search and rescue

- Rainfall monitoring and flooding

- Health and sports monitoring

5G wireless sensing service also brings challenges related to confidentiality and privacy. There is a need to protect the data from unauthorized access, interception and eavesdropping, but also to make sure there is compliance with regulation and user awareness.

In summary, it is considered beneficial for 3GPP specifications to address 5G system support of different use cases and service requirements for Integrated Sensing and Communication.

* * * End of Changes * * * *